3GPP TSG-RAN WG3 Meeting #R3-126       	                                                               R3-247626

Orlando, USA, Nov 18-22, 2024

Agenda Item:	17.3

Source: 	Rakuten

Title:  	Discussion of new procedures to support OD-SIB1

Document for:	Discussion and decision

# Introduction

At the RAN3#125 meeting, the following was agreed [3]:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

FFS if Cell A broadcasts it right away or not.

From RAN3#125-bis, we have the following agreements for the topic of On Demand SIB1.

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

# UL WUS Configuration from NES Cell to Cell A

It has been agreed in the previous meetings that the UL WUS Configuration is decided by the NES Cell gNB-DU, and it sends the UL WUS Configuration to the NES Cell gNB-CU and that is forwarded to Cell A gNB-CU. It has also been agreed that the UL WUS Configuration will be transmitted over Xn using a new class 1 procedure. We propose the new class 1 procedure to follow the design principles of the existing basic procedures defined in 38.423.

## UL WUS Config Transmission

When a source NG RAN node decides to transition to a NES mode, it needs the support of the neighboring NG RAN nodes. The source NG RAN node sends its UL WUS Configuration to the target NG RAN node to broadcast to UEs in IDLE/INACTIVE state in the target NG RAN node. This makes the NES NG RAN node discoverable by the UEs in the target NG RAN node.

The procedure uses non-UE-associated signalling.

Successful Operation:

UL WUS Config Transmission Request procedure is used by source NG RAN node (NES Cell) to determine whether the target NG RAN node (Cell A) is willing to broadcast UL WUS Configuration to the UEs camped on the target NG RAN node (Cell A gNB). If the target NG RAN node is willing to the transmit the UL WUS Configuration of the source NG RAN node, it responds with UL WUS Config Transmission Acknowledgement.

Unsuccessful Operation:

If the target NG RAN node (Cell A) is unable or unwilling to broadcast UL WUS Configuration to the UEs camped on the target NG RAN node, the target NG-RAN node shall send the UL WUS CONFIGURATION TRANSMISSION FAILURE message to the source NG-RAN node. In subsequent meetings, we could discuss whether this message should contain a Cause IE.

RAN3 to agree on the new class 1 procedure initiated by the NES Cell towards Cell A to request the broadcast of its UL WUS Configuration.

# Activation/Deactivation of NES mode

In RAN3#125-bis, it was agreed that Cell A gNB can decide and signal to NES Cell gNB that it wants to stop the UL WUS configuration broadcast in its SIB.

Based on the agreement, we have the following understanding:

- NES Cell can decide when to start/stop NES mode.

- Cell A can decide when to start/stop broadcasting UL WUS Configuration of the NES Cell

- Observation 1: The trigger to activate NES mode of a NES cell comes from NES Cell only. The trigger to deactivate NES mode of a NES cell can come from either NES Cell or Cell A.

This leads us to the following possible scenarios:

Scenario 1a: Activation of NES mode triggered by NES Cell and supported by Cell A

Scenario 1b: Deactivation of NES mode triggered by NES Cell

In this case, the NES Cell sends a UL WUS Config Transmission Stop procedure to the target NG RAN node. On receiving this, the target NG RAN node may stop broadcasting the UL WUS configuration of the source NG RAN node to the UEs camped on target NG RAN node.

RAN3 to agree on the new class 2 procedure initiated by the NES Cell towards Cell A to stop the broadcast of its UL WUS Configuration.

Scenario 2a: Activation of NES mode triggered by NES Cell and supported by Cell A

Scenario 2b: Deactivation of NES mode triggered by Cell A: Cell A stops broadcasting UL WUS Configuration; therefore, the NES Cell is forced to transition out of NES mode

In this case, the source NG RAN node decides to stop the transmission of the UL WUS Configuration of the target NG RAN node. The source NG RAN node sends a UL WUS Config Transmission Cancel procedure to the target NG RAN node. On receiving this procedure, the target NG RAN node may choose to transition out of the NES mode.

It is possible that one Cell A has stopped broadcasting the UL WUS configuration of the NES Cell, but another Cell A continues to broadcast the UL WUS configuration of the NES. It is up to the implementation at the NES Cell to decide when and under what conditions it should deactivate the NES mode after receiving the UL WUS Configuration Transmission Cancel from a Cell A.

RAN3 to agree on the new class 1 procedure initiated by Cell A towards NES Cell to indicate its cancelation of the broadcast of the UL WUS Configuration of the NES Cell.

# Conclusion

We have the following observations and proposals:

Proposal 1: RAN3 to agree on the new class 1 procedure initiated by the NES Cell towards Cell A to request the broadcast of its UL WUS Configuration.

Observation 1: The trigger to activate NES mode of a NES cell comes from NES Cell only. The trigger to deactivate NES mode of a NES cell can come from either NES Cell or Cell A.

Proposal 2: RAN3 to agree on the new class 2 procedure initiated by the NES Cell towards Cell A to stop the broadcast of its UL WUS Configuration.

Proposal 3: RAN3 to agree on the new class 1 procedure initiated by Cell A towards NES Cell to indicate its cancelation of the broadcast of the UL WUS Configuration of the NES Cell.

# Reference

Chairman’s Notes RAN3#125

Chairman’s Notes RAN3#125-bis