3GPP TSG-RAN WG3 #126								R3-247627

Orlando, USA, 18th – 22th Nov 2024

Agenda Item:	12.2

Source:	China Telecom

Title: 	Discussion on RAN2 Impact and Functional Aspects of WAB

Document for:	Discussion

Introduction

The new WI on additional topological enhancements for NR was approved in RAN#105 meeting [1]. And the following functional aspects of WAB-nodes need to be discussed in WI Phase:

- NG interface aspects.

- WAB authorization.

- Mobility handling.

- Xn interface aspects.

In addition, per the Note in WID, RAN2 impact should be identified as early as possible, and should be minimal.  In SI phase, there was rarely discussion on RAN2 impact as no impact on Uu interface was assumed in WAB. Since the WAB entity also comprises of two entities, namely WAB-MT and WAB-gNB, like IAB and NCR, it is need to specify the functionalities of WAB-MT and the behaviour of WAB-gNB during mobility procedure at least in stage 2. In this contribution, we mainly discuss RAN2 impact and functional aspects of WAB.

Discussion

RAN2 impact

RRC States

According to agreement in RAN3#125bis meeting, WAB-MT supports at least a subset of functionalities:

The WAB-MT supports at least a subset of UE functionalities.

Therefore, it is need to discuss whether all RRC states are supported in WAB-MT. The behaviour and functionalities of NCR-MT and IAB-MT are captured in existing stage 2 and stage 3 specifications. As is quoted in TS38.300, the NCR-MT supports all RRC states and the behaviour of NCR-Fwd during state transition.

When the NCR-MT is in RRC_CONNECTED state, the NCR-Fwd may amplify-and-forward RF signals based on the side control information received from the gNB. The NCR-MT does not support RRM measurements in RRC_CONNECTED.

When the NCR-MT transitions from RRC_CONNECTED state to RRC_INACTIVE state, the NCR-Fwd may continue to amplify-and-forward RF signals in accordance with the last side control information received from the gNB. When the NCR-MT is in RRC_INACTIVE state, the NCR-Fwd ceases to amplify-and-forward RF signals if no suitable cell is detected, or if the NCR-MT selects a different cell than the last serving cell on which side control configuration was received.

When an NCR-MT in RRC_INACTIVE state determines degradation of the NCR-Fwd backhaul link beam, then the NCR-Fwd should cease amplifying-and-forwarding RF signals, and the NCR-MT should attempt to resume its RRC connection (with cause value mo-Signalling). The criteria to evaluate backhaul beam degradation are left to the NCR-node implementation.

When the NCR-MT transitions from RRC_CONNECTED state to RRC_IDLE, the NCR-Fwd ceases any amplifying-and-forwarding of RF signals. How an NCR-MT transitions back from RRC_IDLE state to RRC_CONNECTED state is left to NCR-node or network implementation.

The WAB-MT also needs to support all the existing RRC states, namely Connected, Idle, and Inactive. In addition, WAB-MT also needs to support all SRBs as it may be configured with single carrier or MR-DC by BH gNB(s). Based on the above analysis, it is concluded that WAB-MT need to support all RRC States and all types of SRBs.

Proposal 1: WAB-MT supports all RRC States and all types of SRBs.

In Connected state, the WAB-gNB could send/receive signal to/from UE and establish Xn and NG interface with BH gNB/5GC. Regarding the behaviour of WAB-gNB for Idle and Inactive state, there was not discussed in the past RAN3 and RAN2 meetings. Upon the WAB-MT transit from Connected state to Idle state, the WAB-gNB ceases to send/receive signal to/from UE and remove Xn/NG interface, and redirection all UEs to other NG-RAN nodes or E-UTRAN nodes. For Inactive state, whether WAB-gNB ceases to send/receive signal to/from UE need to be studied in RAN2. From the perspective of RAN3, the Xn/NG interface could be kept as the arrival of Xn-C/NG-C signalling or Xn-U/NG-U user plane data may trigger WAB-MT transit from Inactive to Connected state. In a word, the behaviour of WAB-gNB in Idle and inactive state of WAB-MT need to be discussed in RAN2.

Proposal 2: the behaviour of WAB-gNB in Idle and inactive state of WAB-MT need to be discussed in RAN2.

RLF and BFD/BFR

According to the NOTE6 in objective part of WID, the Mobility of the WAB-MTs reuses the legacy UE mobility procedures. This NOTE also implies that the WAB-MT shall supports RLF detection, Beam Failure Detection (BFD) and Beam Failure Recovery (BFR) procedure as normal UE.

NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

Upon the RLF is detected during mobility procedure, WAB-MT performs the RRC re-establishment procedure as legacy UE. During RRC re-establishment procedure, the behaviour of need to be discussed in RAN2 as the WAB-gNB disconnects with BH gNB before the re-establishment procedure is completed. For NCR-MT, the behaviour of NCR-Fwd also need to be specified in stage 2 specification.

Proposal 3: The behaviour of WAB-gNB during RLF or Beam Failure need to be discussed in RAN2 WI phase.

Access Control

In order to avoid another WAB-MT tries to access the cell of WAB-gNB, it is need to specify a mechanism to avoid multi-hop. One potential solution is to introduce a new indicator in SIB1 to indicate the support of WAB. The WAB-MT shall consider the cell broadcast this new indicator as a barred cell while the legacy ignores this indicator. In addition, since IAB and NCR are also alternatives to extend coverage and enhance wireless topological, whether the WAB supports NCR and IAB also need to be decided by RAN2. In our view, WAB does not supports NCR and IAB to avoid multi-hop and stage 2 description on the conclusion is also needed.

Proposal 4: whether the WAB supports NCR and IAB also should be decided by RAN2.

Moreover, the following issues on IAB-MT and NCR-MT were discussed in the scope of RAN2, and we understand RAN2 also need to discuss whether they are also needed for WAB-MT:

- Cell status and cell reservations: Both IAB-MT and NCR-MT ignore the existing cell status and reservations in SIB1, e.g, cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse, intraFreqReselection, and cellReservedForOtherUse.

- Unified Access Control: Unified Access Control does not apply to IAB-MTs and NCR-MTs

- additional allowed cell list and/or forbidden cell list: NCR-MT may be configured with additional allowed cell list and/or forbidden cell list. And The NCR-MT shall consider only the allowed cell list for reselection candidate cells.

Proposal 5: RAN2 needs to discuss whether the Cell status and cell reservations, Unified Access Control and additional allowed cell list and/or forbidden cell list are really needed in WAB.

Xn interface

According to the objective of WID and SA2 conclusion, the WAB-gNB can establish Xn interface with neighbour cells. And Xn-based handover procedure can be also performed via Xn interface as legacy mechanism. As we know, the Xn interface and cell relation configuration are usually fixed in the legacy stationary base station. Operators can manually configure neighbour cell relationships and interfaces, or use tools like ANR to configure them automatically. Per TS23.501, the WAB as mobile base station could be mounted on a moving vehicle and may serve UEs inside or outside the vehicle. Thus, the Xn interface between WAB-gNB and BH gNB may not always exists. Both WAB-gNB and BH-gNB may remove the cell relation and Xn interface upon the base station moves out of the coverage area of the neighbour BH gNB. In order to address this issue, a new indicator information can be contained in Xn Setup/Modifciation message to indicate the survive time of Xn interface.

Proposal 6: a new indicator information can be contained in Xn Setup message to indicate the survive time of Xn interface.

NG interface

According to objective of WID, NG connection management should be considered in WI phase. Per TS23.501, the WAB-gNB and MWAB-MT are authorized and controlled separately. When WAB-gNB tries to establish with CN, it should provide the WAB-gNB Indicator via NG Setup Request message. And the CN needs to check authorized and provide authorized result in NG Setup Response/Failure message. In case the status changed from “authorized” to “not authorized”, AMF could signal “authorized” or “not authorized” in AMF CONFIGURATION UPDATE message. Then, NG connection removal procedure need to be triggered by WAB-gNB after all served UEs have been handed over to neighbour cells.

Proposal 7: a new WAB-gNB Indicator should be contained in NG Setup Request message.

Proposal 8: AMF could signal “authorized” or “not authorized” in AMF CONFIGURATION UPDATE message.

Proposal 9: NG connection removal procedure need to be triggered by WAB-gNB in case the status changed from “authorized” to “not authorized”.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: WAB-MT supports all RRC States and all types of SRBs.

Proposal 2: the behaviour of WAB-gNB in Idle and inactive state of WAB-MT need to be discussed in RAN2.

Proposal 3: The behaviour of WAB-gNB during RLF or Beam Failure need to be discussed in RAN2 WI phase.

Proposal 4: whether the WAB supports NCR and IAB also should be decided by RAN2.

Proposal 5: RAN2 needs to discuss whether the Cell status and cell reservations, Unified Access Control and additional allowed cell list and/or forbidden cell list are really needed in WAB.

Proposal 6: a new indicator information can be contained in Xn Setup message to indicate the survive time of Xn interface.

Proposal 7: a new WAB-gNB Indicator should be contained in NG Setup Request message.

Proposal 8: AMF could signal “authorized” or “not authorized” in AMF CONFIGURATION UPDATE message.

Proposal 9: NG connection removal procedure need to be triggered by WAB-gNB in case the status changed from “authorized” to “not authorized”.

Reference

- RP-242395，New WID on additional topological enhancements for NR，NTT DOCOMO, INC., AT&T

- S2-2409245，Mobility of UEs served by MWAB,Huawei, HiSilicon, Nokia, Ericsson, ZTE