3GPP TSG-RAN WG3 #126								R3-247628

Orlando, USA, 18th – 22th Nov 2024

Agenda Item:	12.2

Source:	China Telecom

Title: 	Discussion on Multi-hop Prevention and Authorization for WAB

Document for:	Discussion

Introduction

The new WI on additional topological enhancements for NR was approved in RAN#105 meeting [1]. The mechanisms to avoid multi-hop WAB topology need to be specified in WI phase. According to the outcome of the online discussion at last RAN3 meeting, the following candidate solutions need to be down selection:

- Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters.

- Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed.

- Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

- Solution5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

This contribution aims to further discuss on Authorization for WAB and evaluate these candidate solutions.

Discussion

Down Selection of Multi-hop Prevention Solutions

- Solution 1

The solution 1 implies the WAB can only be served in a dedicated network with dedicated resource. Currently, we have only deployed a dedicated network for railway. Per the justification of WID, WAB need to support multiple application scenarios, e.g., aircrafts, cruise ships, helicopters, and vehicles [1]. Obviously, it is not possible to configure dedicated network for all scenarios. Furthermore, operators had allocated all NR frequencies and PCI for 5G commercial network. If Solution 1 is agreed, operators have to adjust their network planning strategies which is also impossible at this stage. Thus, the Solution 1 is not feasible in commercial network.

- Solution 2

For Solution 2, UE may still try to access any cell and be denied during the initial access procedure. Therefore, the drawback of Solution 2 is to introduce much more latency during the initial access procedure. In addition, the most of commercial UEs can only support one slice/default slice as Operation System of commercial UEs could not support to be configured with multiple slices. This is not an issue with the 3GPP protocol but rather with the terminal ecosystem. We should not expect to change this situation in the short term. Therefore, the Solution 2 is feasible in theoretically feasible, but practically not possible in commercial network.

Observation 1: Solution 1 and Solution 2 are not feasible in commercial network.

- Solution 3

Solution 3 can be regarded as a straightforward solution to avoid multi-hop but it has RAN2 impact as a new indication needs to be introduced in SIB1 message and corresponding behaviour of WAB-MT in inactive /idle state. Compare to Solution 2, access control is performed during cell (re)selection procedure and WAB-MT regards one cell with access indicator broadcast as a forbidden cell. During handover procedure, UE may not check SIB1 provided by target cell via HO command. Upon handover failure occurs, WAB-MT reselect other cells to initiate RRC establishment procedure. In addition, this enhancement on air interface has no impact on the existing commercial UEs as they ignore this new indicator. Thus, according to the above analysis, Solution 3 has impact on RAN2 specifications, e.g., TS38.331 and TS38.304. In order to support some special cases, it is proposed that OAM may configure a forbidden/allowed cell list for WAB-MT as NCR-MT in IDLE/INACTIVE state. The following texts on access control mechanism for NCR-MT are quoted below:

NCR-MT may be configured with additional allowed cell list and/or forbidden cell list, see TS 38.300 [2]. The NCR-MT shall consider only the allowed cell list, if configured by OAM as candidates for cell reselection (ignore above exclude-listed cells and/or allow-listed cells). The NCR-MT shall not consider the cells for cell reselection in the forbidden cell list, if configured by OAM.

Observation 2: Solution 3 is a straightforward solution to avoid multi-hop.

- Solution 5

This solution proposes an access control mechanism only applied during handover procedure. The basis of this solution is that the source and target BH RAN need to aware of WAB. Therefore, we should discuss whether WAB awareness is needed first. From the perspective of signalling enhancement, we prefer to not introduce any new indicator over RRC message. SA2 had also concluded that there is no need to specify a new indication in RRC connection establishment that the MWAB-UE intends to operate as MWAB. Moreover, the authorization information for WAB-MT can be send via NAS signalling. As for UE capability, we also do not see the need to introduce a set of new UE capabilities for WAB-MT. To sum up, source BH RAN could not derive the identification of WAB-MT from RRC/NAS signalling. In order to address this issue, companies proposed to configure URSP or S-NSSAIs. Since the source node does not have the knowledge of whether the target cell is a WAB-gNB, WAB-MT still may suffer handover failure upon the handover request is denied by the target cell. Therefore, Solution 5 is not a unified solution for all cases and WAB-MT still may suffer handover failure upon the handover request is denied by the target cell.

Observation 3: Solution 5 is not an unified solution for all cases and WAB-MT still may suffer handover failure upon the handover request is denied by the target cell.

Proposal 1: WAB-gNB-cells broadcast a new indicator in SIB for access control.

WAB authorization

According to the conclusion of SA2, the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs. Therefore, it is need to introduce a new indicator in NG Setup Request/RAN Configuration Update Message to indicate it is a WAB node. This indicator cannot be changed after NG interface was established. Without this new indicator contained in NG Setup Request/RAN Configuration Update Message, the AMF will take this node as legacy gNB.

Proposal 2: the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs.

According to RAN3 agreements in last meeting, authorization of the WAB-MT is different from the WAB-gNB service authorization/configuration/activation by e.g. OAM/SeGW.

If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. As regards whether SCTP connection for Xn and NG interface is still maintained, we understand it depends on pre-configuration or OAM configuration. If the WAB-MT authorization status changes from “not authorized” to “authorized” , since BH PDU session and its corresponding DRBs resource can be reallocated to WAB-gNB, it could perform the normal WAB-gNB setup procedure.

Proposal 3: If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection

If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the authorization information can be delivered via NGAP message., NG Setup Response/RAN Configuration Update ACK message. The WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. In this case, the SCTP connection for Xn and NG interface should be also removed. While the authorization information is triggered by OAM, the WAB-gNB could also not continue to provide service to any UEs and needs to ceases to receive/send signal from/to UE and remove Xn/NG connection. Since the BH PDU session can be still kept and, the SCTP connection can be maintained.

Proposal 4: If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB could not continue to provide service to any UEs and needs to ceases to receive/send signal from/to UE and remove Xn/NG connection. If the authorization status changes is triggered by CN, the SCTP connection shall be removed.

Proposal

Based on the above analysis, we have the following observations and proposals:

Observation 1: Solution 1 and Solution 2 are not feasible in commercial network.

Observation 2: Solution 3 is a straightforward solution to avoid multi-hop.

Observation 3: Solution 5 is not an unified solution for all cases and WAB-MT still may suffer handover failure upon the handover request is denied by the target cell.

Proposal 1: WAB-gNB-cells broadcast a new indicator in SIB for access control.

Proposal 2: the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs.

Proposal 3: If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection

Proposal 4: If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. If the authorization status changes is triggered by CN, the SCTP connection shall be removed.

Reference

- RP-242395，New WID on additional topological enhancements for NR，NTT DOCOMO, INC., AT&T