3GPP TSG-RAN WG3 Meeting #126	R3-247631

Orlando, US, 18 - 22 Nov, 2024

Title: 	Discussion on renewable energy based LBO

Source: 	Huawei

Agenda item:	8.1

Document Type:	 Discussion

# 1. Introduction

The last SA5-127bis meeting sent an LS on renewable energy-based LBO as follows [1].

1. Overall Description: 3GPP SA5 (Management, Orchestration and Charging) has initiated its Release 19 study on energy efficiency and energy saving aspects of 5G networks and services. In this context, 3GPP SA5 identified a use case where neighbor gNBs, which are candidates for distributed LBO, are powered using different types of energy sources, some of which being renewable, some others being not, and where the decision for choosing the target gNB to receive the load transferred from source gNB is taken based on such considerations. As an illustration, in the figure below, gNB A and gNB C are both candidates to receive traffic load transferred from gNB B, the only difference between gNB A and gNB C being that gNB A is fully powered by renewable energy. With the guidance of renewable energy usage policy, gNB B gives gNB A higher priority and chooses gNB A. To support this capability, the procedure of Distributed SON LBO would need enhancements to consider new energy related information, such as e.g. carbon emission, carbon emission efficiency and renewable energy usage. 3. Actions: To RAN3: SA5 would like to ask RAN3 to provide SA5 with your feedback on the use case and potential solution described above.

This contribution provides our views on the received LS.

# 2. Discussion

## 2.1 Background

The received LS is related to the SA5 R19 energy work Study item (FS_Energy_OAM_Ph3).  The overall study is captured in TR 28.880, where the use cases and requirements are related to energy related information, and define the new energy related information, new Energy Consumption (EC) KPIs, new Energy Efficiency (EE) KPIs and new Energy Saving (ES) solutions for 5G.

## 2.2 Incoming LS analysis

The LS requests RAN3 feedback on the use case and potential solutions respectively. Neighbouring gNBs may be powered by different types of energy sources, where some are powered by the renewable, such as solar PV (i.e. Photovoltaic), while others are not.

From [1], the use case is basically described as:

- gNB A uses renewable energy from Energy Grid or generated by its attached solar PV

- gNB C uses the traditional powering method - fossil energy from the Energy Grid

- guidance of renewable energy usage policy, gNB B gives gNB A higher priority and chooses gNB A.

We think this use case is a useful and valuable scenario for energy saving purpose. Due to the fact that the gNB A is powered by renewable energy source, it is beneficial to offload traffic to the gNB A instead of gNB C, as long as the UE link performance is ensured. This can allow the gNBs with the traditional powering method to enter into energy saving state. This clearly brings more energy efficiency, less carbon emission, and high renewable energy etc.

RAN3 to feedback that from RAN3 perspective the use case mentioned in the LS is valuable for energy saving purpose.

In the incoming LS [1], it is also mentioned that “the procedure of Distributed SON LBO would need enhancements to consider new energy related information”.

First, we consider the Mobility Load balancing (MLB) is a correct tool to tackle the use case. As copied below, MLB can be used to offload traffic other cells, to achieve the network energy saving, as well as load balancing. Therefore, MLB can be typically used to meet the requirement of the use case mentioned in [1].

#### 15.5.1.1 General The objective of mobility load balancing is to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cell or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to achieve network energy saving. This can be done by means of optimization of cell reselection/handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks. Intra-RAT, intra-system inter-RAT and inter-system load balancing scenarios are supported. In general, support for mobility load balancing consists of one or more of following functions: - Load reporting for intra-RAT and intra-system inter-RAT load balancing; - Load balancing action based on handovers; - Adapting handover and/or reselection configuration; - Load reporting for inter-system load balancing.

- Observation 1:  The MLB can be typically used to offload users from one cell, cell area, carrier or RAT to achieve network energy saving, hence meeting the requirement of the use case.

Second, the current SON MLB mechanism in RAN3 is sufficient to meet the requirements for the use case. MLB has various of functions, including the load reporting, handover-based load balancing etc. Typically, take the figure in [1] -  the gNB A powered by the renewable energy from the attached PV, while gNB C is powered by the fossil energy from the energy grid.

Example 1:

- gNB A powerd by the renewable energy, can indicate to the gNB B its composite available capacity based on how much solar energy is retrieved from PV in a progressive way. On the contrary, the gNB C can only provide very conservative available capability to gNB B. All this can be implemented based on implementation or OAM configuration.

- gNB B can determine offloading to the gNB A based on the configuration policy/implementation.

Example 2:

- For intra-frequency mobility, gNB A can provide its own mobility parameters or proposed gNB B mobility parameters in the Mobility Change Request message to set the HO trigger so that it can attract more offloaded traffic. On the contrary, the gNB C would not do much mobility setting change optimization.

- the gNB B can decide handover UEs to the gNB A based on the configuration policy/implementation.

Example 3:

- the gNB A can adjust its cell reselection parameters including frequency priority, hysterias threshold etc to attract more idle/inactive UEs to camp while gNB B could do the opposite, based on the configuration etc.

For all above examples, it should be highlighted that:

- there is no need to indicate the energy type (e.g., renewable, traditional etc) among neighbouring gNBs over network interfaces.

- the overall UE radio link performance should not be deteriorated during the MLB actions, which is well covered by the existing RAN specifications.

Hence RAN3 may feedback that the current SON MLB mechanism in RAN3 is sufficient to meet the requirements for the use case. RAN3 could also feedback that it is up to SA5 to consider how the policy on renewable energy is configured to the RAN node, which is out of RAN3 scope.

RAN3 to feedback that the current SON MLB mechanism in RAN3 is sufficient to meet the requirements of the use case, and no intention to indicate the energy type (e.g., renewable, traditional etc) among neighbouring gNBs over the network interfaces. It is up to SA5 to consider how any policy on renewable energy is configured to the RAN node.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- RAN3 to feedback that from RAN3 perspective the use case mentioned in the LS is valuable for energy saving purpose.

- Observation 1:  The MLB can be typically used to offload users from one cell, cell area, carrier or RAT to achieve network energy saving, hence meeting the requirement of the use case.

RAN3 to feedback that the current SON MLB mechanism in RAN3 is sufficient to meet the requirements of the use case, and no intention to indicate the energy type (e.g., renewable, traditional etc) among neighbouring gNBs over the network interfaces. It is up to SA5 to consider how any policy on renewable energy is configured to the RAN node.

The draft reply LS is provided in [3].

# 4. Reference

- S5-246028 LS to RAN3 on renewable energy based LBO, SA5

- 3GPP TR 28.880 V1.1.0 Study on energy efficiency and energy saving aspects of 5G networks and services

(Release 19)

- R3-247632	[draft] Reply LS on renewable energy based LBO	Huawei