3GPP TSG-RAN WG3 Meeting #126			 R3-247647

Orlando, USA, 18th – 22th Nov 2024

| Agenda item: | 20.2 |
| --- | --- |
| Source: | Baicells |
| Title: | Discussion on support of AI/ML assisted positioning (Case 3a) |
| Document for: | Discussion and Decision |

Introduction

Accurate positioning is an essential and integral component of many industrial use cases and verticals. In R18, a SI Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1] investigated the necessity, feasibility, and potential enhancement of using AI/ML for positioning as one of the target use cases. Under this background, a new R19 WI [2] on AI/ML for NR Air Interface has been approved at RAN #102. This WI aims to provide normative support for general framework for AI/ML for air interface and use cases such as positioning accuracy enhancement in the preceding R18 study. In addition, some study objectives will be continued to deal with outstanding issues identified in R18 study.

The following objectives regarding AI/ML based positioning are included in the WID that was updated at RAN #105:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Figure 1-1  Sub-use cases with different priorities

Discussion

In RAN #102, the five sub-use cases have been assigned with different priorities as shown in Figure 1-1. In our understanding, RAN3 should focus on the NW side cases with 1st priority (Case 3a and Case 3b) regarding LCM for positioning, or what have been agreed by RAN1 or RAN2.

RAN3 to focus on:

NW side cases with 1st priority (Case 3a and Case 3b)

Works have been agreed by RAN1 or RAN2

In RAN3#125bis, the following Case 3a related agreements were reached.

The AI/ML model (inference and training functions) is at the gNB. Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Table 2-1 shows the model input and model output related information for Case 3a.

Table 2-1  model input and model output (label) related information for Case 3a

|  | Measurements for model input | CIR/PDP | By TRP |
| --- | --- | --- | --- |
|  | Ground-truth label | Intermediate measurement: -New measurement report: e.g., ToA, path phase -Existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP -Enhancement of existing measurement report: e.g., soft information/high resolution of RSTD | By network entity with known PRU location |

Data collection for model training

It’s still open that whether LMF can be used for gNB-side model training for Case 3a. If LMF can train a gNB-side model, it means measurement and related data need to be sent from gNB to LMF, which may have similar procedure as Case 3b.

Compared with LMF training gNB-side model, it’s more straightforward for a gNB to train gNB-side models with assistance information from LMF or CN in Case 3a. In addition, it can greatly reduce the workload of normative phase regarding training data collection since the whole procedure can have minimal specification impact. Thirdly, NW/LMF-side training burden can also be released if a gNB or a RAN-side node can train gNB-side models by itself, which is a distributed training pattern instead of centralized training. Therefore, we think the primary focus of normative work for Case 3a data collection for model training can be the training mechanism at gNB side. Using gNBs for positioning model training can be a starting point and the group may discuss how labels are provided to gNB by NW entity (e.g., LMF). With labels and measurements gathered at the gNB, the model training can be performed.

Regarding the signaling design, we think a request and response of the ground truth label is needed. The detail design of the request and response can be further discussed.

Support a request and response procedure for gNBs to obtain the ground truth label.

Data collection for monitoring

In RAN1 #118, the following agreement regarding monitoring was reached.

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

The Case 3a monitoring can be either at gNB side or LMF side. If the monitoring is at gNB side, on one hand, minimum specification effort can be expected as it can be totally up to implementation. For example, using model-input-based monitoring, or other methods which do not rely on labels. On the other hand, if the gNB-side monitoring method needs label information from network entities (with known PRU position), specification impact can be expected same as model training. For example, labels or related information need to be sent from a network entity to gNB. Therefore, we think model training and monitoring can use the same procedure acquiring label-related information from NW for Case 3a.

For Case 3a model monitoring at gNB side, label-related information is sent from network entity to gNB. In such case, model training and monitoring can share similar procedure.

If LMF performs the monitoring, monitoring data or metrics can be provided to LMF via NRPPa. Then LMF performs the monitoring, detects anomalous data, and reports the anomalies to gNB. In this logic, we may discuss issues such as what’s the monitoring data and metrics, what/how to collect them, and when/what/how to report anomalies or other information.

For Case 3a model monitoring at LMF side, further discuss the data to be monitored, metrics that may result in monitoring decisions, and related procedures.

Conclusions

Proposal 1.	RAN3 to focus on:

	NW side cases with 1st priority (Case 3a and Case 3b)

	Works have been agreed by RAN1 or RAN2

Proposal 2.	Support a request and response procedure for gNBs to obtain the ground truth label.

Proposal 3.	For Case 3a model monitoring at gNB side, label-related information is sent from network entity to gNB. In such case, model training and monitoring can share similar procedure.

Proposal 4.	For Case 3a model monitoring at LMF side, further discuss the data to be monitored, metrics that may result in monitoring decisions, and related procedures.

- References

3GPP TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), V2.0.0, 2023.

[2] RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface RAN#102 December 2023.

- Appendix

- Agreements on inferencing

RAN2 #125bis Agreement There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

- Agreements on data collection/training

In the SI - Model Training: - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM. - Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. RAN2 #125bis Agreement RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management). RAN1 #116bis Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope.

- Agreements on monitoring

In the SI - Monitoring: - The UE monitors the performance of its UE-side model. - For monitoring at the gNB side, and if needed, calculated performance metrics or data required for performance metric calculation, can at least be generated by the gNB. - For monitoring at the LMF side, the gNB or UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination points for these metrics is the LMF. RAN2 #125bis Agreement For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3. RAN1 #118 Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.