3GPP TSG-RAN WG3 Meeting #126	R3-247672

Orlando, USA, 18th – 22th Oct 2024

Title: 	Discussion on transferring Energy Cost for split architecure

Source: 	CMCC, Samsung

Agenda item: 	11.4

Document Type:	Discussion

- Introduction

In RAN3 #124 meeting, agreements as following have been achieved for split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

Measured Energy Cost can be transferred from DU to CU.

And in RAN3 #125-bis meeting, it has been agreed that:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

.2	Discussion

Here from our understanding, both the Measured Energy Cost and measured EC are common concept, referred to the information transferred through Xn interface. Meanwhile, it is never carefully discussed what kind of format should be transferred between gNB-DU and gNB-CU to represent the Measured Energy Cost.

In R18, consensus has achieved on that: EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy.

And RAN3 also reply to the LS from SA5 on the mapping rules in R3-240998[3]:

“It is up to the Operator to define the unified “Energy Consumption – to – Energy Cost” mapping rule. ”

“RAN3 assumes that a NG-RAN node is capable to self-measure its Energy Consumption and then derive the corresponding Energy Cost, the latter being a dimensionless index which is derived on the basis of averaged measurements of the NG-RAN node’s consumed energy. It is up to the OAM to configure the Energy Consumption values corresponding to the minimum and maximum Energy Cost index values. By means of configuration the OAM provides a recommended time interval within which to perform the average.”

“n Rel-18 the Energy Cost index concept is limited to non-split NG-RAN nodes.”

“RAN3 wants to further clarify that it is up to the Operator to define the area where the mapping rule is applicable.”

And in RAN3 #125-bis, SA5 also repled the LS and provide the two CRs according to the LS: Rel-18 and Rel-19 CRs to 28.541 to Align with TS 38.423 to introduce NRM definitions to support NR energy cost Reporting. Which means the mapping rules provision can be supported now for non-split and split architecture.

Observation 1: It is clear that gNB shall self-measure the overall Energy Consumption and mapped it into the Energy Cost according to the mapping rule provisioned by OAM, no matter it is split or non-split gNB.

However RAN3 never discuss the details on how to self-measure the Energy Cost for split gNB, which is important for AI/ML Energy Saving use cases.

Since via Xn interface, only node level gNB Measured Energy Cost can be provided to the neighbouring gNB. It is important and fair to explicitly describe how to self-measure the Energy Cost for the split gNB in the same way. In split architecture, both gNB-CU(gNB-CU-CP and gNB-CU-UP) and gNB-DUs under it will consumes energy, and the gNB’s Energy Cost shall consider all the Energy Cost on gNB-CU and gNB-DU entities reasonably. It is fair and simple to calculate and add up the Measured Energy Cost for all the gNB-DU and gNB-CU in the requested gNB, as the overall Measured Energy Cost for the gNB.

However, currently in SA5, only non-split Energy Consumption is defined. The definition of Energy Consumption for split gNB needs to be confirmed in SA5.

Proposal 1: It is proposed to discuss how to measure the overall Measured Energy Cost in split architecture in RAN3 and send LS to SA5 on helping define the Measured Energy Cost for split gNB.

Currently, in stage2 and stage3 specifications, nothing was captured to describe how to

And also in RAN3 #125-bis, CR to 38.300 R3-245773[4] has been agreed to add the OAM description for AI/ML Energy Saving use case.

16.20.x		OAM Requirements

OAM configures the following:

- the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area; and

- the recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node’s consumed energy.

Proposal 2: The OAM requirements for split gNB on AI/ML Energy Saving shall be described clearly in 38.401, as proposed in the following TP to 38.401.

Since the gNB-CU finally report the Measured Energy Cost to requested gNB via Xn interface, it is also gNB-CU who shall calculate the overall Measured EC. Generally, there are two ways for a gNB-CU to calculate the overall Energy Cost for this gNB:

Option 1: Each gNB-DU maps its own energy consumption into Energy Cost Index value and gNB-CU adds its own EC and all the EC for each gNB-DUs under it.

Option 2: Each gNB-DU transfer its own energy consumption to gNB-CU, and gNB-CU adds its own energy consumption and all the energy consumption from gNB-DU under it, then maps it into Energy Cost Index value.

If RAN3 choose Option1, the index is transferred through F1, it means the same mapping rules shall be provisioned from OAM to all gNB-CU, gNB-DUs repeatedly. However, for a given gNB, all its CU and DUs belong to the same area and follow the same mapping rules. It is redundant to provision the same mapping rules on both CU and DUs.

Another challenge for Option 1 is that in the mapping rule, minimum and maximum EC are currently defined for gNB node level. The number of gNB-DUs under the gNB may change (active/deactive), and each gNB-DU is different, how to provision the minimum and maximum EC in the mapping rules for each gNB-DU is also a challenge for operators. And to calculate the node-level EC, the measured value for gNB-CU and gNB-DUs shall be in the same time window, which is determined by gNB-CU by implementation, it is also hard for OAM to provision and coordinate the time window for all these gNB-CU and gNB-DUs.

For Option2, since F1 only transfer the information from DU to CU, there is no sensitive information leak issue between gNB-DUs from different vendors.

So we think option2 is an easy and feasible way to get the Measured EC for a split gNB.

Proposal 3: It is suggested to use option 2 to calculate the overall Measured Energy Cost for a gNB with split architecture.

Proposal 4: The mapping rule only needs to be configured by OAM to gNB-CU. Capture it in TP to 38.401.

In RAN3#125-bis, consensus has been gotten as captured from Chairman Note:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

Based on above suggestions, the Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU can be same as those over Xn interfaces defined in TS38.423. But introduce Energy Consumption IE instead of Energy Cost IE.

Proposal 5: It is proposed to transfer the energy consumption from gNB-DU to gNB-CU when CU request the Measured EC via F1.

And the time window to measure the consumed energy shall be the same for all gNB-DU and gNB-CU per node level EC measurement when gNB-CU send requests to all gNB-DUs under it. Only the replied consumed energy measured in the same time window can be added up as the node level Measured EC.

And according to the OAM requirement which has been agreed in CR to 38.300 R3-245773: the recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node’s consumed energy. It means the time window to average the consumed energy in gNB-DU may be choosed by gNB-DU itself by implementation, which can’t be guaranteed to be same for all gNB-DUs under the same split gNB.

So we think it is better to let gNB-CU decide the time window and need to guarantee it is the same in the request to gNB-DU. gNB-CU can set the time window in request message.

Proposal 6: It is suggested to discuss how to guarantee the time window used to average consumed energy to be the same for all gNB-DUs and gNB-CU under a split gNB, and whether to carry the time window from gNB-CU to gNB-DU.

If one of these gNB-DU response failure to gNB-CU on the energy consumption, gNB-CU shall also fail to calculate the overall Measured EC for this split gNB.

Proposal 7: gNB-CU shall finally add up the energy consumption received via F1 from all its gNB-DUs in the same time window. Then map the sum of energy consumption into Energy Cost Index value, as the overall Measured EC for this split gNB.

3	Summary

In this contribution, we provide additional input considerations on the remaining issues on AI/ML for NG-RAN split architecture use cases, following observations and proposals are made:

Observation 1: It is clear that gNB shall self-measure the overall Energy Consumption and mapped it into the Energy Cost according to the mapping rule provisioned by OAM, no matter it is split or non-split gNB.

Proposal 1: It is proposed to discuss how to measure the overall Measured Energy Cost in split architecture in RAN3 and send LS to SA5 on helping define the Measured Energy Cost for split gNB.

Proposal 2: The OAM requirements for split gNB on AI/ML Energy Saving shall be described clearly in 38.401, as proposed in the following TP to 38.401.

Proposal 3: It is suggested to use option 2 to calculate the overall Measured Energy Cost for a gNB with split architecture.

Proposal 4: The mapping rule only needs to be configured by OAM to gNB-CU. Capture it in TP to 38.401.

Proposal 5: It is proposed to transfer the energy consumption from gNB-DU to gNB-CU when CU request the Measured EC via F1.

Proposal 6: It is suggested to discuss how to guarantee the time window used to average consumed energy to be the same for all gNB-DUs and gNB-CU under a split gNB, and whether to carry the time window from gNB-CU to gNB-DU.

Proposal 7: gNB-CU shall finally add up the energy consumption received via F1 from all its gNB-DUs in the same time window. Then map the sum of energy consumption into Energy Cost Index value, as the overall Measured EC for this split gNB.

4. Reference

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM)

- RP-242385 WID Enhancements for Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN (ZTE, NEC), 3GPP RAN#105

- R3-240998, Reply LS on Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index (Nokia), 3GPP RAN#123

- R3-245773, OAM Requirements for EC stage2 CR(Nokia), 3GPP RAN#125bis

- TS 38.401, Architecture description

- TS 38.473, F1 application protocol (F1AP)