3GPP TSG-RAN WG3 Meeting #126	R3-247674

Orlando, USA, 18th – 22th Oct 2024

Title: 	(TP to 38.300) Discussion on Supporting MBS broadcast service for NR NTN

Source: 	CMCC

Agenda item: 	14.2

Document Type:	Other

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for NR Phase 3 has been approved in RP-240775.

For RAN3, one of the objectives is to specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

In RAN3 #125-bis, TP was captured to use Mapped Cell ID(s) for intended MBS service area for NTN.

For the geographical service area information, it was agreed that:

RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu.

And LS on Supporting MBS broadcast service for NR NTN in R3-245794 has been sent to RAN2, SA2 and CT4. Also TS38.300 has been added with the following description: To support the intended service area of a broadcast service (e.g. MBS broadcast) in case the satellite footprint covers a larger area, the intended service area may be in a form of cell ID list, TAI list, or the geographic area or any combination as defined in TS 38.413.

2	Discussion

However, Whether and how to define the geographical area information over NG interface? needs to be further discussed.

RAN2 has gotten consensus that the geographical information can be introduced in the intended broadcast service area from last meeting. Anyway, RAN2 doesn’t use the GNSS description:

The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

RAN3 can begin the discussion to introduce more information in Intended Service Area, but need clear use cases to decide what and how to include the extra information than Mapped Cell ID.

In some scenarios, the MBS broadcast intended geographical location can’t be exactly mapped to Cell coverage area. Especially when a NTN cell’s coverage is much more larger than the intended MBS broadcast service’s geographical area. For example, a NTN cell covers multiple countries. And the MBS broadcast service is only intended to a specific country’s geographical location in the area. UE may roam to another PLMN which is also covered by the same cell, then AMF may need to carry extra information to RAN, to indicate the intended service area.

It is not only to improve the accuracy of the MBS service area, but also useful in some NG enhancement solutions, such as Store&Forward for IoT NTN, or NG suspend/resume or Xn Handover enhancements for specific UEs. The small granularity location information in MBS Service Area can help the satellite (gNB on board) make decisions to choose different solution/enhancement for a given UE. These UE may be under the same cell coverage area but needs different solutions according to their geographical location.

Proposal 1: It is proposed to turn the assumption into agreements, to introduce geographical information under MBS intended service area, to provide smaller granularity service area or different service area than Mapped cell.

Proposal 2: It is proposed to introduce referenceLocation and radius information in MBS intended service area, as RAN2, to present the geographical information, and provide smaller granularity service area than cell level.

Proposal 3: Captured the description in TP to TS38.300 as below.

3	Summary

In this contribution, we provide considerations on the NTN broadcast service and PWS service in RAN3, following observations and proposals are made:

Proposal 1: It is proposed to turn the assumption into agreements, to introduce geographical information under MBS intended service area, to provide smaller granularity service area or different service area than Mapped cell.

Proposal 2: It is proposed to introduce referenceLocation and radius information in MBS intended service area, as RAN2, to present the geographical information, and provide smaller granularity service area than cell level.

Proposal 3: Captured the description in TP to TS38.300 as below.

4	Reference

[1] 	TS 38.413, NGAP

[2]       TS 38.300, NG-RAN stage2

[3]       BLCR R3-245861 (BL CR to 38.300) Support for Regenerative Payload in NR NTN

[4]       3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM)

5	TP to TS38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

To support the intended service area of a broadcast service (e.g. MBS broadcast) in case the satellite footprint covers a larger area, the intended service area may be in a form of cell ID list, TAI list, or the geographic area or any combination as defined in TS 38.413 [26]. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Editor’s Note: The above paragraphs may be further reformulated.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>