3GPP TSG-RAN WG3 Meeting #126          	          R3-247689

Orlando, USA, 18 Nov - 22 Nov 2024

Agenda item:	16.4

Source:	CMCC, Huawei, ZTE, CATT, Xiaomi, Nokia, LG Electronics, China Telecom

Title:	(TP to TR 38.769) locating ambient IoT devices

Document for:	Discussion && Approval

1	Introduction

This contribution provides the TP to TR 38.769 for locating ambient IoT devices based on the discussion paper.

- Text Proposal to TR 38.769

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.9	Locating ambient IoT devices

### 6.9.x	General

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

NOTE:	If and how to further provide the location information to the consumer of the A-IoT service by the A-IoT CN is out of the scope of this study.

### 6.9.y	Topology 1

For topology 1, an A-IoT RAN may hold one or multiple readers, and the reader(s) are assumed to be stationary. A-IoT RAN node ID and probably identifier of the reader can be considered as a location of A-IoT device.

To support of A-IoT device location report, if A-IoT RAN serves multiple readers, the A-IoT RAN may report an identifier of the reader in the Inventory Report and/or Command Response message to the A-IoT CN.

The A-IoT CN can determine the location of a reader supported by an A-IoT RAN by OAM configuration or coordination between A-IoT RAN and A-IoT CN.

NOTE:	How to define the reader identifier could be further discussed in normative phase.

### 6.9.z	Topology 2

For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

To support of A-IoT device location information report:

-	In RRC based solution, UE ID is reported to the A-IoT CN in the Inventory Report and/or Command Response to enable the A-IoT CN to determine the location of the A-IoT-enabled UE to which the A-IoT device responded.

-	In NAS/UP based solutions, the A-IoT CN knows by default from which A-IoT-enabled UE the A-IoT device responded.

The A-IoT CN can determine the location of the A-IoT-enabled UE via OAM configuration or via legacy LCS procedure.

NOTE:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” may need further consideration.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>