3GPP TSG-RAN WG3 Meeting #126				                     R3-247702

Orlando, USA, 18th - 22th Nov 2024

Agenda Item:       10.2

Source:	ZTE Corporation

Title: 		(TP for TS 38.401 TS38.473) Discussion on MRO

Document for:	Discussion and Approval

1	Introduction

During the RAN2#127bis meeting, RAN2 agreed to narrow the scope of RAN2's work for Rel-19 SON/MDT. The agreements are listed below.

Agreements 1: Continue the work of MRO for LTM 1st priority. 2: Focus CHO with candidate SCGs (focus on agreed scenarios by RAN3) with 2nd priority 3: For Rel-18 leftovers, regarding SON for SDT, focus on RSRP and data volume in SON reports. Other enhancements (including the additional failure cause) shall be paused unless time permits. 4: Close the stage-2 work on MRO for MR-DC SCG failure. 5: Pause subsequent CPAC and wait if any for RAN3 LS. 6: Pause and wait if any for RAN3 LS for SON/MDT for Slicing and NTN. 7: Send an LS to RAN3 that RAN2 want to prioritize RAN3-based solutions on P5 and P6. 8: Pause and wait for RAN3 LS on MHI Enhancement for SCG Deactivation/Activation.

Additionally, RAN2 sent an LS to RAN3 on the way forward for Rel-19 SON/MDT, to request RAN3 to prioritize RAN3-based solutions that have no RAN2 impacts on MRO for SCPAC, NTN, and slicing.

This contribution considers RAN2's LS and related agreements, providing further discussion on enhancements for MRO in LTM, CHO-CPAC, and subsequent CPAC.

2	MRO for SCPAC

Based on the agreements/LS from RAN2, RAN2 has paused SCPAC and requested that RAN3 prioritize solutions that are RAN3-based and have no impact on RAN2.

In past RAN3 discussion, the issue was raised of whether to support forwarding the SCG UHI to the target node during the S-CPAC procedure to avoid ping-pong immediately. However, we believe this type of real-time optimization introduces significant complexity of SCPAC reconfiguration at network side and  is not aligned with the principles of MRO, which focuses on long-term optimization. Additionally, it would require modifications to the UE's SCPAC configuration whenever a ping-pong is detected. We suggest that this optimization not be considered.

Proposal 1:  The real-time ping-pong optimization for SCPAC, e.g., forwarding the SCG UHI to the target node during the S-CPAC procedure, is not aligned with the principles of MRO, which focus on long-term optimization. As RAN2 has paused SCPAC, we propose that RAN3 close the discussion on this optimization.

At the RAN3#125 meeting, it was agreed that "Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.". In Rel-18 MRO, the MN may include the CPC candidate cell list and CPC execution conditions to the SN via the SCG FAILURE INFORMATION REPORT over Xn for failure analysis.To differentiate the CPC configuration over Xn, RAN3 discuss whether to introduce  SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

Proposal 2:RAN3 to discuss whether to introduce the SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

3	MRO for CHO-CPAC

Following was the progress during RAN3#124 & RAN3#125：

RAN3#124 progress: Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases) Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

RAN3#125 progress: MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed. MRO for case 4/5/6 is in the scope. FFS whether there is any specification impact for case 4/5/6. Case 1c, 2c, 2c, 3c, 7c, 8c are FFS. The RLF report includes: - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met. - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered - FFS whether to optimize the association of the PCell and PSCell.

RAN3#125bis progress: RLF report is enhanced to including - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered. - The fulfilled events before the RLF is encountered?

3.1 Failure case :

3.1.1 Case 1c, 2c, 2c, 3c, 7c, 8c:

During past meeting, cases such as 1a and 1b have been identified in the scope of Rel-19. For case 1a, the network could optimize either PCell or PScell based on the failure information received from UEs. While for cases such as 1c, we think it could also be captured in Rel-19 scope. We assume the case represent the situation that PCell failure and PScell failure happen simultaneously. The SCG failure reports could not been send to the network because the radio link between UE and MN is not valid. In this case, the SCG failure could be captured in the same RLF report. With such information, the network could optimize PCell and PScell at the same time.

Proposal 3: RAN3 to consider the Case 1c, 2c, 2c, 3c, 7c, 8c in Rel-19.

3.1.2 Case 9:

Following case 9 has been agreed to address in past meeting. It means CHO execution condition and the associated CPAC execution condition cannot be met simultaneously.

| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |
| --- | --- | --- |

During the RAN3#125bis meeting, the following enhancement of RLF were agreed:

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Regarding the above enhancement, if applied to Case 9, this would seems that before an RLF encountered, the UE might record a list of candidate cells that individually met their conditions, while their paired candidate cells failed to meet. Clearly, some cells in this list would likely appear multiple times.

However, this approach seems unnecessary. the UE only needs to record the number of times each specific candidate cell met the conditions individually before an RLF encountered. This way, MRO can identify which candidate cell had a higher number, helping to analyze why its configured paired cell was consistently unable to meet the conditions at the same time.

Proposal 4: In order to support MRO for case 9, enhance RLF report to include:

- The identifier of candidate PCell(s) or PSCell(s) that individually fulfilled the execution conditions one or more times before the RLF is encountered, while their paired candidate cells failed to meet those conditions, along with the number of times each cell meet the execution conditions.

3.1.3 Case 10:

During past meeting, Companies propose to consider case 10: UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure, which may be different from case 9.

Since RAN3 has agreed on the following for RLF enhancement: "The identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered." However, for Case 10, it is important to emphasize that the recorded candidate cell information should represent a pair of non-associated candidate PCells and PSCells that fulfilled the execution conditions simultaneously. Therefore, we suggest revising the above agreement to: "The identifiers of the non-associated candidate PCell and PSCell pair(s) that simultaneously fulfilled the execution conditions before the RLF is encountered.".  Similar to Case 9, we only need to record each pair once but with the number of times the pair met the conditions.

Proposal 5: RAN3 to agree support MRO for case 10, revising the existing agreement "The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered" for RLF report as the following.

- The identifiers of the non-associated candidate PCell and PSCell pair(s) that simultaneously fulfilled the execution conditions one or more times before the RLF is encountered, along with the number of times each pair meet the execution conditions.

3.2 Near failure case:

Based on current specification, in addition to CHO-CPAC configuration, the network can provide a complementary CHO-only configuration /execution condition to the same UE.  And in case the CHO-CPAC execution conditions are not meet, instead the CHO-only configuration can be used when the execution condition is meet. In this case, no failure report will create while the  CHO-CPAC configuration seems useless. From RAN3 point view, the useless of CHO-CPAC with CHO-only can be taken as one type of near failure case.

Proposal 6: CHO only executed as  complementary for  configuration of CHO with candidate SCG(s) can be concluded as near failure case.

3.3 Other

Additionally, since a CHO-CPAC failure could potentially cause connection failed in both MCG and SCG simultaneously, there is a need to discuss how to correlate the failure reports related to MCG and SCG.

Near failure cases are also needed to be considered for CHO-SCPAC, We suggest to reuse SHR/SPR for sub-optimization. Since nearby failure could be happened in  both MCG and SCG simultaneously, we believe network needs to consider the correlation of SHR/SPR for MRO optimization .There is a need to discuss how to correlate SHR/SPR reports related to a specific CHO-CPAC execution.

Additionally, considering the possibility that one execution in the MCG or SCG might succeed while the other fails, in this case, there is a need to perform the correlation between the failure report and the success report.

Proposal 7： RAN3 to discuss how to correlate the SON reports of MCG and SCG, including the following cases:

- Correlates the failure reports of MCG and SCG

- Correlates the SHR/SPR for near failure;

- Correlates the success/failure report for CHO-CPAC, in the case of  one execution in the MCG or SCG might succeed while the other fails.

Either CHO or CPAC, any execution condition not being met, could lead to a 'too late failure'. However, even when the execution of CHO-CPAC is successful, we need to minimize the time between the fulfilment of the CHO and CPAC execution conditions, ideally not exceeding a certain threshold. In other words, if this threshold is exceeded, we could classify this CHO-CPAC execution as near failure. We propose the following:

Proposal 8: Introduce a new trigger condition for SHR/SPR, specifically when the time between the fulfillment of CHO and CPAC execution conditions exceeds a certain threshold.

- 4 MRO for LTM

4.1 Failure case

4.1.1 Reports from the UE

During RAN2#126meeting, the following parameter were agreed to be captured in the  RLF.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD. Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field. Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

During RAN2#127 meeting, the following agreements achieved for LTM failure.

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC. - introduce a new field in RLF report to indicate the LTM recovery cell id. - RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. - We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command. - Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios. - We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases. - We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not. - Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

During RAN2#127 bis meeting, the following agreements achieved for LTM failure.

Agreements The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

During the LTM procedure, the UE switches to the cell and beam indicated by MAC CE, but If the UE is switched to the right cell but wrong beam (or beam that could result in successful LTM cell switch but not stable connection at target beam), and the connection in the target cell would be failed within a short period if beam recovery is also failed. In this case, the UE needs to report the TCI state configuration of the current failed target PCell in the RLF, as well as the failed beam. Based on above discussion, we provide proposal as follows:

Proposal 9：For LTM optimization, RAN3 to enhance RLF report with beam related information :TCI state configuration/failed beam.

4.2 Near failure case

During RAN2#127 meeting, the following agreements achieved for LTM near failure.

- RA-based access will not be a new triggering condition for the SHR report. - HO interruption time will not be a new triggering condition for the SHR report - We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases. - We will not log interruption time in SON reports (e.g. SHR) for LTM. - We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not.

During RAN2#127 bis meeting, the following agreements achieved for LTM near failure.

Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation. UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

If LTM is successful, meaning the UE successfully transmits and receives data based on the beam configured by the cell switch command. However, if the UE experiences beam failure in a short time, it indicates that the beam configured by LTM (the beam indicated in the cell switch command) is not good enough. The UE tries beam failure recovery, which is different from the beam configured by the cell switch command. This case is also a near failure case, and UE shall report failed and successfully recovery beam of target PCell, TCI state configuration of the target PCell, L1 measurement in SHR report, then RAN can use these information to analyse near failure.  Furthermore, beam recovery is need to be add as  new trigger for SHR.

Proposal 10: For MCG LTM,  Introduce a trigger cause for beam recovery into SHR report.

Proposal 11: For MCG LTM, Introduce failed and recovery beam of target PCell, TCI state configuration of the target PCell into SHR report for analysis of near failure.

We believe that the existing T310, T312, and T304 timers can be reused as SHR triggers in LTM near-failure scenarios. Following current MRO principles, upon receiving the SHR, the CU would first analyze the cause of the near-failure and then forward the SHR to the source DU if triggered by T310 or T312 (Configured at source DU), or to the target DU if triggered by T304 (Configured at target DU). We suggest to capture this SHR handling approach  in TS 38.401 stage 2. Additionally, if a trigger cause for beam recovery is introduced in the SHR report, this stage 2 would need to be updated accordingly.

Proposal 12: To capture the SHR handling approach in TS 38.401 stage 2. (The corresponding TP is provided in Annex A.)

4.2 Others

Subsequent LTM.

If subsequent LTM is configured for UE, various report might be generated, in a higher frequency. However, the failure information or the sub-optimal success report (if there are any), might be over-written very soon. If subsequent LTM goes on and on, network might fail to fetch the report in time, and won’t be able to spot the issues in a subsequent LTM (and therefore to optimize it).  (Note: a similar issue also exists in SCPAC)

Proposal 13: RAN3 to discuss the issue of frequently over-written report for MRO in subsequent LTM.  (Note: a similar issue also exists in SCPAC).

In past meeting, companies raised the question of whether LTM-related UHI should be transferred to the source CU, the target DU, or both. In R3-245230, a company provided the following discussion: "The ping-pong purpose could be dived into two sub-cases: ping-pong event detection and mobility parameter optimization, ping-pong avoidance. For the former sub-case, CU forwards the UHI to source DU after the source DU makes the cell switch command. For the latter sub-case, CU forwards the UHI to the target DU as assistance information for target DU's further handover decision once the UE accesses to the target DU. RAN3 discuss which case is the ping-pong purpose.".

We believe that only the latter sub-case, CU forwards the LTM related UHI to the target DU, should be considered necessary, while the former sub-case, CU forwards the LTM related UHI to source DU is redundant.

If the CU sends the LTM related UHI to the target DU during each cell switch (e.g., via CU-DU CELL SWITCH NOTIFICATION), the target DU can immediately use this information to make better decisions for future handovers , and can also use the UHI for long-term parameter optimization. If ping-pong between the source and target DU do not continue, then optimization at the source DU becomes unnecessary. However, if the ping-pong does persist and the UE is switched back to the source DU, the source DU will still receive the UHI at that time.

Proposal 14: RAN3 only to consider the scenario where the CU sends the LTM related UHI to the target DU during each cell switch (e.g., via the CU-DU CELL SWITCH NOTIFICATION) to optimize ping-pong for subsequent LTM.  (The corresponding TP for TS38.473 is provided in Annex B).

- 4 Conclusion:

The following proposals are provided for MRO for SCPAC:

Proposal 1:  The real-time ping-pong optimization for SCPAC, e.g., forwarding the SCG UHI to the target node during the S-CPAC procedure, is not aligned with the principles of MRO, which focus on long-term optimization. As RAN2 has paused SCPAC, we propose that RAN3 close the discussion on this optimization.

Proposal 2:RAN3 to discuss whether to introduce the SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

The following observations and proposals are provided for MRO for CHO-CPAC:

Proposal 3: RAN3 to consider the Case 1c, 2c, 2c, 3c, 7c, 8c in Rel-19.

Proposal 4: In order to support MRO for case 9, enhance RLF report to include:

- The identifier of candidate PCell(s) or PSCell(s) that individually fulfilled the execution conditions one or more times before the RLF is encountered, while their paired candidate cells failed to meet those conditions, along with the number of times each cell meet the execution conditions.

Proposal 5: RAN3 to agree support MRO for case 10, revising the existing agreement "The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered" for RLF report as the following.

- The identifiers of the non-associated candidate PCell and PSCell pair(s) that simultaneously fulfilled the execution conditions one or more times before the RLF is encountered, along with the number of times each pair meet the execution conditions.

Proposal 6: CHO only executed as  complementary for  configuration of CHO with candidate SCG(s) can be concluded as near failure case.

Proposal 7： RAN3 to discuss how to correlate the SON reports of MCG and SCG, including the following cases:

- Correlates the failure reports of MCG and SCG

- Correlates the SHR/SPR for near failure;

- Correlates the success/failure report for CHO-CPAC, in the case of  one execution in the MCG or SCG might succeed while the other fails.

Proposal 8: Introduce a new trigger condition for SHR/SPR, specifically when the time between the fulfillment of CHO and CPAC execution conditions exceeds a certain threshold.

The following proposals are provided for MRO for LTM:

Proposal 9：For LTM optimization, RAN3 to enhance RLF report with beam related information :TCI state configuration/failed beam.

Proposal 10: For MCG LTM,  Introduce a trigger cause for beam recovery into SHR report.

Proposal 11: For MCG LTM, Introduce failed and recovery beam of target PCell, TCI state configuration of the target PCell into SHR report for analysis of near failure.

Proposal 12: To capture the SHR handling approach in TS 38.401 stage 2. (The corresponding TP is provided in Annex A.)

Proposal 13: RAN3 to discuss the issue of frequently over-written report for MRO in subsequent LTM.  (Note: a similar issue also exists in SCPAC).

Proposal 14: RAN3 only to consider the scenario where the CU sends the LTM related UHI to the target DU during each cell switch (e.g., via the CU-DU CELL SWITCH NOTIFICATION) to optimize ping-pong for subsequent LTM.  (The corresponding TP for TS38.473 is provided in Annex B).

Annex A: 	TP to TS38.401 for LTM SHR handing

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the gNB-DU responsible for the failure.

The gNB-CU receives a SHR report associated to an LTM mobility event from the UE or via the Access and Mobility message over Xn or via Downlink RAN configuration transfer procedure over NG. In case of the SHR triggered by the T310, T312, or T304 timer, the gNB-CU may forward the SHR report to the gNB-DU responsible for configuring the corresponding timer.

Annex B: 	TP to TS38.473 for Subsequent LTM

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.3.10	CU-DU Cell Switch Notification

8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

If the UE History Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it for future LTM cell switching.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| UE History Information | O |  | OCTET STRING | UE History Information related to LTM, as same definition in TS 38.413 [3] | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>