3GPP TSG-RAN WG3 Meeting #126	R3-247720

# Orlando, Florida, U.S, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1230 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restoration of N3mb Failure for MBS multicast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 received an LS from CT4 in R3-247005 that CT4 agreed a CR for TS 23.527 in C4-244212 to introduce a restoration procedure for Multicast MBS session N3mb path failure for unicast transport. This CR adds the necessary protocol additions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new MBS NG-U Failure Indication is added to the Multicast Session Update Request Transfer IE. Upon receiving it, the NG-RAN node shall behave as specified in TS 23.527. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it introduces protocol additons for the Multicast MBS function only The CR has impact from functional and protocol point of view. The CR is backwards compatible from a functional and protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | N3mb recovery not supported by NGAP as specified in TS 23.527. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8,18.5.2, 9.3.5.13, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.483 CR0150 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.18.5	Multicast Session Update

#### 8.18.5.1	General

The purpose of the Multicast Session Update procedure is to request the NG-RAN node to update the NG-RAN MBS session resources or area related to a multicast MBS session. The procedure uses non-UE associated signalling.

#### 8.18.5.2	Successful Operation

Figure 8.18.5.2-1: Multicast Session Update, successful operation.

The AMF initiates the procedure by sending a MULTICAST SESSION UPDATE REQUEST message to the NG-RAN node.

Upon receipt of the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall, if requested, update the MBS QoS profile and/or MBS Service Area for the multicast service and send the MULTICAST SESSION UPDATE RESPONSE message to the AMF.

For location dependent multicast session, the AMF shall include the MBS Area Session ID IE in the MULTICAST SESSION UPDATE REQUEST message, and the NG-RAN node shall provide the same value of the MBS Area Session ID IE in the MULTICAST SESSION UPDATE RESPONSE message.

In case the MBS Service Area Information IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall update the stored MBS Service Area Information for that service, as specified in TS 23.247 [44].

In case the MBS QoS Flows To Be Setup or Modify List IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall setup or modify the MBS QoS profile accordingly.

In case the MBS QoS Flows To Be Release List IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall release the indicated MBS QoS flows.

In case the MBS Session TNL Information 5GC IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall take the information into account and update related NG-U resources.

If the MBS NG-U Failure Indication IE is included in the MULTICAST SESSION UPDATE REQUEST message within the Multicast Session Update Request Transfer IE and set to "N3mb path failure", the NG-RAN shall, if supported, behave as specified in TS 23.527 [57].

#### 8.18.5.3	Unsuccessful Operation

Figure 8.18.5.3-1: Multicast Session Update, unsuccessful operation.

If the NG-RAN node is not able to perform any of the requested update, it shall respond with a MULTICAST SESSION UPDATE FAILURE message with an appropriate cause value.

#### 8.18.5.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.5.13	Multicast Session Update Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.208 |  | YES | reject |
| MBS QoS Flows To Be Setup or Modified List | O |  | MBS QoS Flows To Be Setup List 9.3.1.236 |  | YES | reject |
| MBS QoS Flow To Release List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS flow Identifiers of the MBS QoS flows to be released. | YES | reject |
| MBS Session TNL Information 5GC | O |  | 9.3.2.15 |  | YES | reject |
| MBS NG-U Failure Indication | O |  | 9.3.2.18 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSQoSFlows | Maximum no. of QoS Flows allowed within one MBS session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MulticastSessionUpdateRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionUpdateRequestTransferIEs} },

...

}

MulticastSessionUpdateRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea					PRESENCE	optional		}|

{ ID id-MBS-QoSFlowsToBeSetupModList		CRITICALITY reject	TYPE MBS-QoSFlowsToBeSetupList		PRESENCE	optional		}|

{ ID id-MBS-QoSFlowToReleaseList			CRITICALITY reject	TYPE QosFlowListWithCause				PRESENCE	optional		}|

{ ID id-MBS-SessionTNLInfo5GC				CRITICALITY reject	TYPE MBS-SessionTNLInfo5GC				PRESENCE	optional		}|

{ ID id-MBSNG-UFailureIndication			CRITICALITY reject	TYPE MBSNG-UFailureIndication			PRESENCE	optional		},

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>