3GPP TSG RAN WG3 Meeting #127-bis	R3-251516

Wuhan, China, 07-11 April, 2025

3GPP TSG-RAN WG2 Meeting #129	R2-2501396

Athens, Greece, 17 – 21 of February 2025

Title:	Reply LS on QMC Coordination for RRC Segmentation in NR-DC

Response to:	R2-2500022/R3-247888

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:

Contact Person:

Name:	Zhihong Qiu

E-mail Address:	qiu.zhihong@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN3 for the LS. After discussion, below are RAN2 responses to Questions 1-4:

- Question 1: Can the UE receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter?

- Question 2: Can the UE receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter?

RAN2 response to Q1/2: It is specified in the field descriptions as below that rrc-SegAllowedSRB4-r17 is only configured for an MCG and rrc-SegAllowedSRB5-r18 is only configured for an SCG.

rrc-SegAllowedSRB4 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB4. The field is only configured for an MCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

rrc-SegAllowedSRB5 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB5. The field is only configured for an SCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

Based on it, there are two different understandings in RAN2:

- Understanding 1: rrc-SegAllowedSRB4-r17 is only used for an MCG, and rrc-SegAllowedSRB5-r18 is only used for SCG. But there is no restriction that MN should not include rrc-SegAllowedSRB5-r18 in AppLayerMeasConfig-r17 or SN should not include rrc-SegAllowedSRB4-r17 in AppLayerMeasConfig-r17. In order words, UE can receive both rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 in a common AppLayerMeasConfig-r17 from any node.

- Understanding 2: rrc-SegAllowedSRB4-r17 is only configured in an MCG configuration and rrc-SegAllowedSRB5-r18 is only configured in an SCG configuration.

- Question 3: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled?

Question 4: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

RAN2 response to Q3/4: According to current ASN.1 design, rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 are optional fields with need code Need R. Based on it, there are two different understandings of UE behaviors in RAN2:

- Understanding 1: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, UE considers the RRC segmentation function for SRB5 is disabled. If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, UE considers the RRC segmentation function for SRB4 is disabled.

- Understanding 2: rrc-SegAllowedSRB4-r17 cannot be included in a configuration for an SCG and absence of the field in an SCG configuration means no action by the UE as the field cannot be present. Similarly, rrc-SegAllowedSRB5-r18 cannot be included in a configuration for an MCG and absence of the field in an MCG configuration means no action for the UE as the field cannot be present.

RAN2 has discussed whether to change the current design on rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 and it was not agreed as it would lead to non-backward compatible changes in RAN2 specifications. Therefore RAN2 reached below agreement:

We do not correct this in RAN2 and let RAN3 make corrections

2. Actions:

To RAN WG3:

RAN2 respectfully asks RAN3 to take the aforementioned RAN2 decisions into account in their future work.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#129-bis	from 2025-04-07	to 2025-04-11		Wuhan, CN

RAN2#130	from 2025-05-19	to 2025-05-23		Malta, MT