3GPP TSG-RAN WG3#127bis 	R3-251534

Wuhan, China, 7-11 April 2025

Agenda item:	13.3 Support for Conditional LTM

Source:	Nokia

Title:	Discussion on Conditional LTM in disaggregated gNB architecture

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-250339 NR mobility enhancements Phase 4), where the objectives relevant to conditional LTM (CLTM) are described as:

• Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC • Specify RRM requirements related to the above objectives as necessary [RAN4]

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the cell-switch command at right radio condition which may lead to radio link failure. Compared to CHO, the conditional LTM is expected to bring the benefits of robustness along with the interruption time reduction benefits of LTM. In this contribution, we discuss base requirements from RAN3 perspective to support Intra-CU Conditional LTM in disaggregated gNB architecture, considering the agreements so far in RAN2.

# 2	Discussion

Considering the four phases of LTM, we can divide our analysis of Conditional LTM as follows.

- Conditional cell switch condition configuration

- Conditional cell switch condition evaluation

- Early synchronization

- Conditional cell switch execution

Further, RAN3 has also agreed the following at last meeting which we build upon for our proposals.

RAN3#127: The CU decides to configure C-LTM. The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM. Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition. UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition. UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition. Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail. For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused. Continue to work on the solutions comparison and details…

## 2.1	Conditional cell switch condition configuration

Conditional LTM follows Rel-18 LTM principle which reserves cell and RACH resources when configuring LTM. Likewise, if Conditional LTM configured at a cell is to be based on L1 measurements, the gNB-CU needs to request gNB-DU to generate the L1 execution conditions, which are reported back to the gNB-CU. Further, this process is required for each candidate LTM cell where Conditional LTM is to be configured. After the L1 execution conditions have been signalled to the gNB-CU, it can generate the RRC message containing the L1 execution conditions of each cell and signal it to the UE. For this purpose, we have agreed to reuse existing UE CONTEXT SETUP REQUEST and UE CONEXT MODIFICATION REQUEST messages with a corresponding flag indicating the preparation concerns Conditional LTM. However, Stage-3 was not agreed. For the Indicator we believe an IE as follows “Conditional-LTM (ENUMERATED(true,…))” is sufficient.

Proposal 1: For L1 measurement based Conditional LTM, introduce a new “Conditional-LTM (ENUMERATED (true,…))” IE in F1AP UE CONTEXT SETUP and UE CONTEXT MODIFICATION REQUEST messages to trigger generation of Conditional LTM L1 execution conditions.

Proposal 2: The gNB-DU signals the L1 execution conditions via the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages. Whether the L1 executions conditions should be embedded with the CSI report config or require to be explicitly signaled via F1AP via new IEs is pending RAN2 progress.

Similarly, if Conditional LTM configured at a cell is based on L3 measurements we do not foresee F1AP impact, given that the condition is determined by the gNB-CU. Hence, the existing Rel-18 LTM framework is sufficient and there is no additional interaction needed between the gNB-CU and Source/Candidate gNB-DU(s) at the preparation phase.

## 2.2	Early Synchronization

L3 based Conditional LTM considerations

Conditional LTM is meant to support early synchronization. This results in two possible cases, one for L1 measurements based, and another for L3 measurements case.

In our view, for L1 measurement based conditional LTM, the existing Rel-18 LTM framework is sufficient for triggering early sync related procedures and we foresee no F1AP specification impact for this case.

However, for the L3 measurement based conditional LTM, there is need for supporting a mechanism for the gNB-CU to request a Source gNB-DU to trigger only early sync related procedures but not the LTM cell switch command.  However, given there is ongoing RAN3 discussion for Rel-18 LTM for L3 based LTM overall, we propose to postpone this decision and build upon the agreed solution in Rel-18 when it concludes.

Proposal 3: For L3 measurement based Conditional LTM, the gNB-DU needs capability to trigger early sync related procedures independently (i.e. without having the gNB-DU trigger also an LTM cell switch) via F1AP. Impact to Conditional LTM shall be revisited after Rel-18 L3 based LTM framework concludes.

Beam indication considerations for Conditional LTM

Upon LTM cell switch change, the Rel-18 LTM framework signals the activated TCI States to the Candidate LTM cell in order to be prepared for the incoming UE in the appropriate resources. This is signalled via the DU-CU/DU-CU CELL SWITCH NOTIFICATION messages.

For Conditional LTM a similar handling is required. However, given that the cell switch execution is determined by the UE, the Candidate LTM cell is unaware of the TCI States that the UE will utilize. Hence, F1AP signalling is needed so that when DL synchronization takes place, the Source-DU provides the TCI State IDs to the Candidate-DU LTM cells (via the gNB-CU) indicating these have been activated for Conditional LTM.

Proposal 4: For a UE configured with Conditional LTM, the Source gNB-DU signals over F1AP to the Candidate gNB-DU (via the gNB-CU) about the activated TCI States.

Proposal 5: The F1AP CU-DU CELL NOTIFICATION and CU-DU CELL SWITCH NOTIFICATION messages are reused to indicate the activated TCI States to the Candidate gNB-DU for Conditional LTM.

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU ® gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Conditional LTM | O |  | ENUMERATED(true, …) | Indicates whether the message relates to a Conditional LTM configuration. | YES | ignore |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

## 2.2	Conditional LTM Cell Switch Execution

Awareness at Source DU that Conditional LTM was executed

In case of Conditional LTM, given that the LTM cell switch is triggered by the UE, the Source gNB-DU is unaware that the mobility has taken place. Hence, we see need for a mechanism to notify the Source gNB-DU and stop data transmission toward the UE.

One alternative is to handle this from the network side and reuse existing messages. That is, have the Candidate gNB-DU notify the Source gNB-DU that the UE has accessed the cell. In legacy LTM, the Candidate gNB-DU already indicates to gNB-CU via the ACCESS SUCCESS message over F1 that the UE has accessed the new cell. Likewise, the gNB-CU needs trigger to stop data transmission on the older cell at the Source gNB-DU via the UE CONTECT MODIFICATION REQUEST message. However, RAN3 could further discuss whether it is beneficial to introduce a new IE to explicitly differentiate the Conditional LTM scenario from other L3 mobility where the same message is used to stop transmission or rely on existing IEs.

Proposal 6: For Conditional LTM cell switch, the Candidate gNB-DU signals to the Source gNB-DU via the gNB-CU that Conditional LTM cell switch has taken place. Existing ACCESS SUCCESS and UE CONTEXT MODIFICATION REQUEST messages are reused.

Proposal 7: RAN3 to discuss whether a new IE for explicit differentiation of Condititional LTM Cell Switch triggered is needed in UE CONTEXT MODIFICATION REQUEST message or reuse existing IEs.

# 5	Conclusions

Proposal 1: For L1 measurement based Conditional LTM, introduce a new “Conditional-LTM (ENUMERATED (true,…))” IE in F1AP UE CONTEXT SETUP and UE CONTEXT MODIFICATION REQUEST messages to trigger generation of Conditional LTM L1 execution conditions.

Proposal 2: The gNB-DU signals the L1 execution conditions via the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages. Whether the L1 executions conditions should be embedded with the CSI report config or require to be explicitly signaled via F1AP via new IEs is pending RAN2 progress.

Proposal 3: For L3 measurement based Conditional LTM, the gNB-DU needs capability to trigger early sync related procedures independently (i.e. without having the gNB-DU trigger also an LTM cell switch) via F1AP. Impact to Conditional LTM shall be revisited after Rel-18 L3 based LTM framework concludes.

Proposal 4: For a UE configured with Conditional LTM, the Source gNB-DU signals over F1AP to the Candidate gNB-DU (via the gNB-CU) about the activated TCI States.

Proposal 5: The F1AP CU-DU CELL NOTIFICATION and CU-DU CELL SWITCH NOTIFICATION messages are reused to indicate the activated TCI States to the Candidate gNB-DU for Conditional LTM.

Proposal 6: For Conditional LTM cell switch, the Candidate gNB-DU signals to the Source gNB-DU via the gNB-CU that Conditional LTM cell switch has taken place. Existing ACCESS SUCCESS and UE CONTEXT MODIFICATION REQUEST messages are reused.

Proposal 7: RAN3 to discuss whether a new IE for explicit differentiation of Condititional LTM Cell Switch triggered is needed in UE CONTEXT MODIFICATION REQUEST message or reuse existing IEs.

# References

- 3GPP TS 38.401

- RP-250339 Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom