3GPP TSG-RAN3 Meeting #127bis	R3-251586

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Inventory and Command procedures for Topology 1 of AIoT

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on how to support Inventory and Command procedures for Topology 1 of A-IoT.

# Discussion

## Inventory-only

The following working assumptions were agreed in RAN3#127

WA: Introduce a NGAP Class 1 Inventory Request procedure.

Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE.

Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE.

Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE.

WA: Introduce a NGAP Class 2 Inventory Report procedure.

Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE.

The above Transfer IEs are transparent to AMF in case of indirect communication.

The following call flow was captured in the TP to 38.300:

Figure 16.xx.x3-1: Inventory procedure

### Inventory Request

The following is captured in SA2 spec:

7.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, and assistance information to the selected AIoT RAN.

Editor's note:	AIoT Device Identification information needs further definition.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

In TR 38.769, it was captured that the Inventory Request from AIOTF to A-IoT RAN node may contain an A-IoT Device Identification (details up to SA2 and RAN2) and a Scope of Inventory Request (e.g., a certain area in which the inventory is to be triggered) which is also referred as “AIoT Service Area” in our companion paper R3-251585. We propose to confirm to include the above details in the Inventory Request.

Further as already agreed in RAN3, AIOTF may indicate the A-IoT Service Area as an area and/or a list of Readers to the gNB. We therefore propose to also include a list of Readers in the Inventory Request.

Further, the following was captured in the Conclusions in SA2 TR:

The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

- AIoT service type (e.g. Inventory, Command);

- approximate number of AIoT devices based on AF request;

- approximate D2R message size based on AF request.

We think that the gNB can distinguish whether it is an inventory or command based on the type of message it receives from AIOTF (Inventory Request vs. Command Request). There is no need to explicitly indicate the type of A-IoT service request (inventory or command).

The other two information (approximate number of AIoT devices and approximate D2R message size) can be provided from AIOTF to gNB and can be used as assistance information for planning resources for inventory.

Further, there were few proposals last meeting whether gNB needs to know whether there is a follow-up command after an inventory i.e., whether and how the gNB can distinguish between an Inventory-only vs. “Inventory and Command”. We think such a “Follow-up command indicator” flag can be useful and can also be included in the Inventory Request.

Further, the following open issue was identified in last RAN3 meeting:

FFS For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

In our understanding, a gNB can receive multiple parallel Inventory Requests from the same AIOTF and each Inventory Request can be uniquely identified by a Correlation ID.

Further, a gNB can receive multiple parallel Inventory Requests from different AIOTFs and each Inventory Request can be then uniquely identified by the pair {AIOTF ID, Correlation ID}.

A gNB can receive multiple parallel Inventory Requests from the same AIOTF which can be uniquely identified by a Correlation ID.

A gNB can receive multiple parallel Inventory Requests from the different AIOTFs which can be uniquely identified by the pair {AIOTF ID, Correlation ID}.

Therefore, the following is proposed:

Inventory Request sent from A-IoT CN node to gNB may contain the following

  - A-IoT Device Identification. Details up to SA2 and RAN2

  - A-IoT Service Area

  - Reader ID(s)

  - Approximate number of A-IoT devices

  - Approximate D2R message size

  - Correlation ID

  - AIOTF ID

  - Follow-up command indicator

### Inventory Response

The following is captured in SA2 spec:

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

We therefore propose to confirm the above and support the corresponding signalling:

Inventory Response sent from gNB to A-IoT CN node may contain the following:

  - Correlation ID

  - AIOTF ID (only in case of indirect; for routing by AMF to the appropriate AIOTF)

### Inventory Report

The following is captured in SA2 spec:

10.	AIoT RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

As mentioned in SA2 spec, the Inventory Report may contain the Correlation ID, and the Reader ID associated to the A-IoT device(s) which are inventoried, so that the AIOTF can use this information for any subsequent command signaling.

We therefore propose the following:

Inventory Report sent gNB to AIOTF may contain the following:

  - Correlation ID

  - AIOTF ID (only in case of indirect; for routing by AMF to the appropriate AIOTF)

  - A list of AIoT NAS containers, each carrying the A-IoT device ID of the inventoried A-IoT device

  - Reader ID

## Inventory followed by Command

The following working assumptions were agreed in RAN3#127:

WA: Introduce a NGAP Class 1 Command Request procedure

Introduce COMMAND REQUEST message, which includes Command Request Transfer IE.

Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE.

Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE.

The above Transfer IEs are transparent to AMF in case of indirect communication.

The following call flow was captured in TP to 38.300:

Figure 16.xx.x4-1: Command procedure

### Signaling design for Command

The following open issues were identified in RAN3#127:

FFS: Command request procedure is a per device procedure or to be used for multiple devices?

FFS: The need for a Command Report procedure?

Option 1: Command Request is a device associated message and is used by the AIOTF to send commands for each A-IoT device sequentially.

Cons: Longer delay to complete Command procedure for multiple A-IoT device(s). Need to wait for Command Response of one A-IoT device before triggering Command for another A-IoT device.

Pros: No need of a “Command Report” procedure. No need of aggregation of A-IoT NAS Command Response PDUs (thereby simpler behavior at gNB).

Command procedure over the A-IoT radio for an A-IoT device may take multiple seconds e.g., due to poor R2D/D2R link conditions, low harvested energy at A-IoT device, D2R/R2D scheduling, D2R segmentation of Command Response

Using just a class-1 Command Request/Response without a class-2 Command Report would mean that the AIOTF would need to have a timer long enough to receive the Command Response message after sending Command Request.

Option 2: Command Request can be used by the AIOTF to send commands for multiple A-IoT device(s)

Pros: Shorter delay to trigger Command for multiple A-IoT device(s) and complete the Command procedure. No need to wait for Command Response of one A-IoT device before triggering Command for another A-IoT device.

Cons: Need of a “Command Report” procedure. Need to support aggregation of A-IoT NAS Command Response PDUs; but this should not be an issue as gNB anyway need to support aggregation of Inventory Reports.

We prefer Option 2 because of its latency benefits and therefore propose the following:

Command Request from A-IoT CN node to gNB can be used to send commands for multiple A-IoT device(s) towards the gNB i.e., it can include multiple A-IoT NAS Command messages.

Introduce a class-2 Command Report procedure from gNB to A-IoT CN node, via which the gNB can send the (aggregated) D2R Command Responses of one or more A-IoT device(s).

### Command Request

The following is captured in SA2 spec:

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, NAS Command Request) to the AIoT RAN. The correlation identifier is corresponding to this AF service operation request and is used for the AIOTF to correlate the service operation responses to the request.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to AIoT RAN.

FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

Regarding Correlation ID and AIOTF ID, we make a similar observation as for Inventory.

A gNB can receive multiple parallel Command Requests from the same AIOTF (e.g., a 1st Command Request for Device 1 and 2 and a 2nd Command Request for Device 3 and 4)

A gNB can receive multiple parallel Command Requests from two different AIOTFs (e.g., a 1st Command Request for Device 1 and 2 from AIOTF1 and a 2nd Command Request for Device 3 and 4 from AIOTF2)

FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

In case there are multiple “Inventory and Command” procedures at the same time, it will be good to know at the gNB whether a Command Request received by the gNB is related to which Inventory Request received previously. One way to achieve this is by using the same Correlation ID for Inventory Request and its follow up Command Request. Therefore, the following is proposed:

AIOTF indicates the same Correlation ID for Inventory Request and its follow up Command Request to correlate the two procedures

Other assistance information (e.g., the Reader ID of the last reader associated to the A-IoT device for which the command is sent and Approximate D2R message size) can also be included in Command Request.

Command Request message sent from A-IoT CN node to gNB may contain the following:

  - Correlation ID

  - A list of {A-IoT Device ID + AIoT NAS container}

  - Reader ID

  - Approximate D2R message size

### Command Response

Similar to Inventory Response, Command Response should at least include the Correlation ID and indicate the success/failure of receiving the Command Request.

Command Response sent from gNB to A-IoT CN node may contain the following:

  - Correlation ID

### Command Report

The following is captured in SA2 spec.

For Direct path as shown in clause 4.2.2.2, step 15a is performed.

15a.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

For Indirect path via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed

15b.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds an Naiotf_AIoT_Command Notify message (AMF ID, correlation identifier, Command Response (correlation identifier, Reader ID, NAS Command Response)) to the AIOTF.

Command Response in the SA2 spec corresponds to the Command Report indicated in this section. We therefore propose the following:

Command Report sent from gNB to A-IoT CN node may contain the following:

  - Correlation ID

  - A list of {AIoT NAS Command Responses}

  - Reader ID

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Inventory-only

2.1.1	Inventory Request

Observation 1:	A gNB can receive multiple parallel Inventory Requests from the same AIOTF which can be uniquely identified by a Correlation ID.

Observation 2:	A gNB can receive multiple parallel Inventory Requests from the different AIOTFs which can be uniquely identified by the pair {AIOTF ID, Correlation ID}.

Proposal 1:	Inventory Request sent from A-IoT CN node to gNB may contain the following

a.	A-IoT Device Identification. Details up to SA2 and RAN2

b.	A-IoT Service Area

c.	Reader ID(s)

d.	Approximate number of A-IoT devices

e.	Approximate D2R message size

f.	Correlation ID

g.	AIOTF ID

h.	Follow-up command indicator

2.1.2	Inventory Response

Proposal 2:	Inventory Response sent from gNB to A-IoT CN node may contain the following:

a.	Correlation ID

b.	AIOTF ID (only in case of indirect; for routing by AMF to the appropriate AIOTF)

2.1.3	Inventory Report

Proposal 3:	Inventory Report sent gNB to AIOTF may contain the following:

a.	Correlation ID

b.	AIOTF ID (only in case of indirect; for routing by AMF to the appropriate AIOTF)

c.	A list of AIoT NAS containers, each carrying the A-IoT device ID of the inventoried A-IoT device

d.	Reader ID

2.2 Inventory followed by Command

2.2.1	Signaling design for Command

Observation 3:	Command procedure over the A-IoT radio for an A-IoT device may take multiple seconds e.g., due to poor R2D/D2R link conditions, low harvested energy at A-IoT device, D2R/R2D scheduling, D2R segmentation of Command Response

Observation 4:	Using just a class-1 Command Request/Response without a class-2 Command Report would mean that the AIOTF would need to have a timer long enough to receive the Command Response message after sending Command Request.

Proposal 4:	Command Request from A-IoT CN node to gNB can be used to send commands for multiple A-IoT device(s) towards the gNB i.e., it can include multiple A-IoT NAS Command messages.

Proposal 5:	Introduce a class-2 Command Report procedure from gNB to A-IoT CN node, via which the gNB can send the (aggregated) D2R Command Responses of one or more A-IoT device(s).

2.2.2	Command Request

Observation 5:	A gNB can receive multiple parallel Command Requests from the same AIOTF (e.g., a 1st Command Request for Device 1 and 2 and a 2nd Command Request for Device 3 and 4)

Observation 6:	A gNB can receive multiple parallel Command Requests from two different AIOTFs (e.g., a 1st Command Request for Device 1 and 2 from AIOTF1 and a 2nd Command Request for Device 3 and 4 from AIOTF2)

Proposal 6:	AIOTF indicates the same Correlation ID for Inventory Request and its follow up Command Request to correlate the two procedures

Proposal 7:	Command Request message sent from A-IoT CN node to gNB may contain the following:

a.	Correlation ID

b.	A list of {A-IoT Device ID + AIoT NAS container}

c.	Reader ID

d.	Approximate D2R message size

2.2.3	Command Response

Proposal 8:	Command Response sent from gNB to A-IoT CN node may contain the following:

a.	Correlation ID

2.2.4	Command Report

Proposal 9:	Command Report sent from gNB to A-IoT CN node may contain the following:

a.	Correlation ID

b.	A list of {AIoT NAS Command Responses}

c.	Reader ID