3GPP TSG-RAN3 Meeting #127bis	R3-251590

Wuhan, China, Apr 7th – 11th, 2025

Agenda item:	10.3.2

Source:	Qualcomm Incorporated

Title:	SON MDT for network slicing

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss aspects related to SON/MDT for network slicing based on agreements and open issues from previous meetings.

# Discussion

  - Slice-aware MRO

RAN3 agreements:

Work on the Slice aware MRO scenarios.

For successful handover, FFS if solutions are needed to optimize handovers in the case of slicing discontinuity, e.g., target cell not supporting slice.

The following is captured in TS 38.300

To make mobility slice-aware in case of Network Slicing, S-NSSAI is introduced as part of the PDU session information that is transferred during mobility signalling. This enables slice-aware admission and congestion control.

NG handovers are allowed regardless of the slice support of the target NG-RAN node i.e. even if the target NG-RAN node does not support the same slices as the source NG-RAN node.

Xn handovers are allowed only when at least one of the slices with active PDU sessions for the UE at the source gNB is supported at the target gNB.

The source gNB performs this check as part of the Handover Preparation.

Even though Xn handovers are allowed even if one of the slices with active PDU session is supported at target gNB, this leads to discontinuity in the slice service and interruption if there is another active PDU session with a different slice.

To detect such scenarios and take correction action in the future, it was proposed by few companies last meeting to add slice-related information in SON reports (e.g., in UHI or SHR) e.g., include UE’s ongoing slice(s) in the source and target cell in UHI, collect SHR when a certain handover resulted in a slice service interruption.

Currently, a gNB already knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn. During handover preparation, the source gNB will know that a PDU session (which is associated to a slice) it is offloading to a certain target gNB might get interrupted due to non-support of that slice in the target gNB.

We therefore are not sure whether any enhancements to SON reports (e.g., collecting SHR in case of such handover to target cell supporting the ongoing slice) to do non-real optimizations are really needed when such things are known already at the gNB.

Also, a certain gNB via implementation can already take slice aware mobility decisions and mitigate slice service discontinuity. For example, a gNB can prioritize a target cell that supports UE’s ongoing slice in the source cell over another target cell that doesn’t support the UE’s ongoing slice (if radio conditions of the two target cells are comparable).

A gNB knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn.

Based on the knowledge of slice(s) supported at neighboring gNBs, a source gNB has knowledge whether a PDU session (which is associated to a slice) will get interrupted or not upon offloading to a target gNB.

There are no enhancements needed from UE in SON reports to assist the gNB to optimize handovers with slice service discontinuity (e.g., due to target cell not supporting UE’s ongoing slice). gNB already has sufficient knowledge to identify such scenarios.

# Conclusion

Observation 1.	A gNB knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn.

Observation 2.	Based on the knowledge of slice(s) supported at neighboring gNBs, a source gNB has knowledge whether a PDU session (which is associated to a slice) will get interrupted or not upon offloading to a target gNB.

Proposal 1:	There are no enhancements needed from UE in SON reports to assist the gNB to optimize handovers with slice service discontinuity (e.g., due to target cell not supporting UE’s ongoing slice). gNB already has sufficient knowledge to identify such scenarios.

# 4. References