3GPP TSG-RAN WG3 Meeting #127bis	R3-251593

Wuhan, China, 7 – 11 April, 2025

Agenda item:	10.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 38.473, TP for SON BL CR for TS 38.300, TP for SON BL CR for TS 38.423) MRO Enhancements for LTM and CHO with Candidate SCG(s)

Document for:	Discussion

# 1	Introduction

This is a contribution related to MRO for LTM and CHO with candidate SCG(s) in Rel-19.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN3 and RAN2

The agreements achieved in RAN3 #127 [1]:

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM?

Network based solution is used for LTM candidate cell list.

Network based or UE based solution for TA acquisition type?

Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

The agreements achieved in RAN3 #126 [2]:

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

For TP to BL CR to TS 38.401:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

Whether network based solution is needed? What’s the UE context to be stored in RAN side?

UHI and ping-pong:

- Which node is responsible for avoiding LTM cell switch ping-pong (CU or DU)?

- CU sends UHI to DUs?

The agreements achieved in RAN2 #128 [4]:

- No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues.

- Reuse the existing ra-InformationCommon for the RA-based LTM failure.

- If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

### 2.1.2 Connection Failure Monitoring for MCG LTM

In the context of RAN3 # 126 meeting also the solution for TA acquisition type in RLF report was discussed. Majority of companies agreed with network-based solution as source gNB can store the Early UL sync configuration and UE-based TA configuration and can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in Cell Switch Command by “retrieving” the UE context upon receiving the RLF Report.

During the RAN3 #127 the discussion continued. Some of the companies prefer supporting network-based solution in case if TA acquisition type is always available in CU to avoid some extra effort to retrieve this information from the DU. Therefore, the following was marked to be clarified in the next meeting:

Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

In case of inter DU LTM cell switch the TA Information List as part of the DU-CU CELL SWITCH NOTIFICATION sent from source DU to CU as specified in TS 38.473 contains the list of candidate cell IDs together with the acquired TA values. The target cell, i.e. the cell the UE will attempt as the first one in target DU is also a candidate cell so may be part of this list together with the acquired TA value (if any) which was also communicated to UE via Cell Switch Command message. Based on presence of the target cell together with the TA value in the TA Information List the CU shall deduce the gNB indicated TA. In case the target cell is not included in the TA Information List but the RLF report contains an indicator about RACH-less LTM cell switch, the CU shall deduce the UE based TA.

Observation 1-1: In case of inter DU LTM cell switch the TA acquisition type shall always be available in the CU.

In case of intra DU LTM cell switch TA Information List is not part of the DU-CU CELL SWITCH NOTIFICATION sent from DU to CU as specified in TS 38.473. The CU does not have any info about acquiring the TA in the DU.

Observation 1-2: In case of intra DU LTM cell switch the TA acquisition type is not available in the CU.

Proposal 1-1: RAN3 to discuss the network-based solution for TA acquisition type for intra DU LTM cell switch.

To solve this problem and make the TA type available in CU also for intra DU LTM cell switch it is proposed to include new binary indicator into Cell ID IE (representing target cell) of the DU-CU CELL SWITCH NOTIFICATION message (TS 38.473) informing CU about gNB indicated TA.

The new binary indicator “gNB indicated TA” is set to “YES” if TA value obtained via Early TA acquisition procedure was added into Cell Switch Command and sent from gNB to UE. Otherwise, it is set to “No”.

Proposal 1-2: RAN3 agrees to add new binary indicator into Cell ID IE (representing target cell) of the DU-CU CELL SWITCH NOTIFICATION message (TS 38.473) informing CU about gNB indicated TA.

In RAN3 # 126 meeting the following was agreed:

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

The reasoning why the C-RTNI is added shall be part of stage 3. It is therefore proposed to update the Semantics description for the C-RNTI in TS 38.473 as proposed in the attached TP.

Proposal 1-3: RAN3 agrees the attached TP for TS 38.473 in Annex 1 to update the Semantics description for the C-RNTI.

There may be a plurality of problems causing the RLF. Besides the wrong cell selection at cell switch and wrong cell switch timing the problem may be related to wrong or missing TA in the Cell switch command.  In general, it may be related to an issue in Early TA Acquisition procedure which oversees source DU. Despite the TA was included in Cell Switch Command it may become outdated because the UE significantly moved between the time of the Early TA Acquisition and the time of the LTM execution. The UE then fails to attempt the target LTM cell and had to switch RACH based access via LTM recovery/re-establishment procedure when new TA value is provided from network in RAR message. We think that reporting of the information about the difference between the TA reported in the Cell switch command for which the RA less LTM failed, and TA received consequently in RAR for recovery/re-establishment cell may help to optimize the wrong cell switch timing.

- Proposal 1-4: RAN3 to discuss the following information to be included in the RLF report:

- TA difference between the TA provided by network in Cell Switch Command for which the RA less LTM failed, and TA received in RAR for recovery/re-establishment cell.

- In case of invalid TA, the UE may re-connect or re-establish into the same target LTM cell to which original access failed. Even such failure case is related to MRO it neither fits to too late, not too early nor to LTM to wrong cell. It may therefore be desirable to introduce new failure cause “Invalid TA” which CU communicates to source DU.

- Proposal 1-5: RAN3 agrees on new failure cause “Invalid TA” to be communicated from CU to the source DU.

- During the RAN2 # 129 meeting also the following issues were discussed:

- BFR shortly after successful LTM cell switch to the wrong beam (“near failure”), and

- LTM cell switch failure due to wrong beam

On the former issue, RAN2 have acknowledged the issue but agreed not to define a UE-based solution. RAN2 respectfully asks RAN3 to consider defining a network-based solution for this.

- As the outcome of the RAN2 # 129 meeting RAN2 send LS to RAN3 (R3-251514) where RAN2 respectfully asks RAN3 to consider defining a network-based solution for the issue of BFR shortly after successful LTM cell switch (“near failure”). Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the case of LTM cell switch failure due to wrong beam.

Regarding the first issue, the scenario under discussion is a beam failure shortly after a successful LTM cell switch, as depicted in Figure 1. The different steps involved in the problem scenario are as follows:

- UE reports event triggered L3 measurement of the neighbor cells to the source cell (Cell-1)

- Source CU decides to initiate LTM configuration. This involves getting the candidate LTM configurations from the neighbor cells. Based on the information from one or more neighbor cells, the source prepares a common LTM configuration for the UE and configures the UE using RRC signaling.

- The UE starts reporting L1 beam measurements as configured by the LTM measurement configurations

- The source DU decides to execute LTM for the UE and sends a Cell Switch Command to UE including transmission specific information called TCI state (e.g. which beam, say Beam 2-1).

- The UE attaches with the target DU (Cell-2) via Beam 2-1

- Shortly afterwards, the UE experience a beam failure and performs Beam Failure Recovery (BFR), i.e. a RACH-based access to another beam.

- The BFR is successful, and the UE connects to Beam 2-2 of Cell-2

Figure 1: Problem scenario with BFR shortly after successful LTM HO

- From the steps above, we observe that both the LTM HO as well as the BFR procedures are successful, therefore the UE does not experience any Radio Link Failure (RLF). However, the overall latency and interruption experienced by the UE during the RACH-based BFR could be seen as part of the overall HO and, therefore the HO is negatively affected as the UE has to change the beams shortly after the HO. In principle, LTM is a RACH-less procedure, but BFR is a RACH-based procedure, this sequence of the two procedures nullifies the advantage of LTM compared to a Layer-3 (L3) handover.

- Proposal 1-6: RAN3 to support the optimization of BFR shortly after successful LTM cell switch.

- As the BFR was successful in target DU (Cell-2), we can say that the beam management in the target DU is working properly, however there could be a problem at the source (Cell-1) that led to a non-optimal beam selection during the LTM cell switch. The beam to be used by the UE in the target cell for LTM HO is indicated to the UE by the source DU in the cell switch command (see step 4 above). The source DU makes this decision based on the UE measurement reports (Step 3).

- Observation 1-3: as BFR if working properly in this scenario, we can assume that the problem is related to LTM HO configurations at the source cell (DU/CU) that led to a wrong beam selection for the LTM cell switch execution

- In beam-based cell deployments, it is normal to have some beam failures and if the BFR is working properly then the UE would be able to continue its services without any radio link failures.  For the problem scenario, it is important to note that the problem is only related to BFRs that may occur “shortly” after the LTM cell switch. For example, if the UE experience this beam failure after staying for a “relatively long” period of time in the target cell, then this would not a problem associated with the LTM cell switch configurations. Therefore, it is important for the network to categorize such problems.

- Observation 1-4: For proper identification of the problem scenario, it is important that the network can properly categorize the BFR(s) that happen “shortly” after the LTM cell switch and can differentiate them from the BFR(s) that may happen after the incoming UE has already stayed in the target cell for some longer time period.

- To avoid these beam failures shortly after LTM cell switch, the source DU needs to know that it selected the wrong beam (TCI state) for the cell switch and how it can modify its cell switch decisions for the future. As the BFR happens in the target DU, the information about the beam on which the beam failure happened and on which beam the UE successfully recovered can be forwarded by the target DU to the source DU. However, as the radio conditions at beam level may change quite rapidly, for proper analysis and corrective action at the source DU, it may not be sufficient to just get the beam information from the target DU. For this we propose that the source DU also stores the Layer-1 measurements reported by the outgoing UE that the source DU used for the LTM cell switch decision. Once the source DU gets a notification along with the beam information from the target DU that the incoming UE experienced a BFR shortly after the LTM cell switch, then the source DU can analyse the stored L1 measurements of that UE. For example, the source DU can use those measurement to see which other beams were observable by the UE and what were their signal strengths at the time of LTM cell switch decision.

- Proposal 1-7: Target DU to signal BFR shortly after LTM cell switch information to the CU, and the CU forwards this information to source DU.

- Proposal 1-8: Source DU stores the L1 radio measurements reported by the outgoing UE that it used for the cell switch decision. Source DU may use those measurements for the root cause analysis and to decide about any corrective actions for the cell switch command decision.

Regarding the second issue, LTM cell switch failure due to wrong beam we think that providing the RLF report is sufficient. There is already an RAN2 agreement on enhancement of the RLF report to log the L1 measurements for serving cell, target cell and other LTM candidate cells.

Observation 1-5: No extra information needs to be reported in RLF report to optimize LTM cell switch failure due to wrong beam.

Proposal 1-9: RAN3 agrees the enhanced RLF report in rel. 19 for LTM is sufficient to optimize LTM cell switch failure due to wrong beam and no extra network-based solution is needed.

## 2.2 MRO for CHO with Candidate SCG(s)

### 2.2.1 Agreements achieved in RAN3

The agreements achieved in RAN3 #127 [1]:

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

FFS on whether Handover Report message is needed for case 7a.

FFS on the additional information included in handover report.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

When to send the message? Immediately or not?

Check whether respective MN is only a candidate?

In addition to SCGFailureInformation, information needed in the new message?

- CPC failure type

The agreements achieved in RAN3 #126 [2]:

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The failure types defined for CHO in TS38.300 and for CPAC defined in TS37.340 are used as baseline. Clarification or amendment could be made on top of that if needed.

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

Which message is used from the last serving MN to the candidate MN(s):

- Handover Report

- via new message

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

The agreements achieved in RAN3 #129 [3]:

Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Enhance RLF report for CHO with candidate SCGs to include the associations between CHO and CPAC.

Enhance RLF report for CHO with candidate SCGs to include at least the following information:

-	Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered;

-	Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered;

-	The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

We should avoid specifying the procedure in a way that the UE sends redundant information

The agreements achieved in RAN3 #128 [4]:

RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements.

Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

### 2.2.3 Connection Failure Monitoring for CHO with Candidate SCG(s)

During the RAN3 # 126 the following was agreed:

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

During the RAN3 # 127 consequently the following was agreed:

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

FFS on whether Handover Report message is needed for case 7a.

FFS on the additional information included in handover report.

There may be a need to forward the RLF report also in case not relevant to too late CHO execution. The case 7a when CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions are also fulfilled consequently the CHO execution is unsuccessful while CPA/CPC execution is successful, which may be caused by delayed fulfilment of CPA/CPC execution conditions which may require optimization on CPA/CPC related part. The RLF report received by source (serving) MN may also need to be forwarded to candidate target MN.

Observation 2-1: The agreement that the last serving MN may need to send message to the candidate MN(s) which may need optimization applies also for case 7a.

The part in TS 38.300 which was added based on the above agreement in RAN3 # 126 and consequently updated in RAN3 #127 meeting shall therefore be updated accordingly.

Proposal 2-1: RAN3 agrees the attached TP for TS 38.300 in Annex 2 to update the scenario when the last serving MN may need to send message to the candidate MN(s) which may need optimization also for handover to wrong cell.

Proposal 2-2: RAN3 agrees the attached TP for TS 38.423 in Annex 3 to add Handover to wrong cell with SCG(s) cause.

In RAN3 # 126 the following was agreed:

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

In RAN3 # 127 consequently the following was agreed:

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

When to send the message? Immediately or not?

Check whether respective MN is only a candidate?

In addition to SCGFailureInformation, information needed in the new message?

- CPC failure type

In our understanding this agreement applies to case 3b and 9b when CHO execution conditions are fulfilled and during waiting for CPC/CPA execution conditions fulfilment RLF is declared on SCG. UE sends the SCGFailureInformation to the serving MN (source MN), while optimization on CPC/CPA related part is needed which requires forwarding the SCGFailureInformation to respective candidate MN(s). For case 1b when no one of the execution conditions is fulfilled the serving MN based on SCGFailureInformation may decide some optimization is needed also for CPC/CPA part and may forward the SCGFailureInformation to the candidate MN(s). For case 2b when CPC/CPAC execution conditions are fulfilled but due to waiting for the CHO execution conditions fulfilment there is RLF on SCG when issue may be in the CHO related part which is configured by source (serving) MN there may be no need to forward the SCGFailureInformation to candidate MN(s). Concerning the case 7b when CPC/CPA execution conditions were fulfilled first then with some delay the CHO execution conditions were also fulfilled but CPC/CPA execution is unsuccessful while CHO execution is successful, which may be caused by delayed fulfilment of CHO execution conditions which may require optimization in CHO related part. The SCGFailureInformation received by target (serving) MN may need to be forwarded to source MN. Considering the same case 7b but with CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions but CPC/CPA execution is unsuccessful while CHO execution is successful, which may point to CPA/CPC issue configured by target (serving) MN and thus non need to forward the SCGFailureInformation. Regarding the case 8b when CPC/CPA execution conditions were fulfilled first then with some delay the CHO execution conditions were fulfilled which was followed consequently with successful CHO and CPC/CPA execution, but shortly there is SCG failure which may be caused by delayed fulfilment of CHO execution conditions which may require optimization in CHO related part. The SCGFailureInformation received by target (serving) MN may need to be forwarded to source MN.  Considering the same case 8b but with CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions and consequent successful CHO and CPC/CPA execution, but shortly there is SCG failure which may point of CPA/CPC issue configured by target (serving) MN and thus non need to forward the SCGFailureInformation.

Observation 2-2: The agreement that the serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed applies for case 1b, 3b, 7b, 8b, 9b.

Observation 2-3: The respective MN is candidate MN in case 1b, 3b, 9b while the respective MN is source MN in case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Proposal 2-3: RAN3 agrees the attached TP for TS 38.300 in Annex 2 to add stage 2 text about the SCGFailureInformation forwarding respecting the fact that respective MN may by candidate or source MN.

During the RAN3 # 127 meeting the new message SCG Failure Indication was agreed to forward the SCGFailureInformation from service to respective MN. Discussion started on whether CPAC failure type shall be added as a new IE into this message. The CPAC failure type may be obtained only in candidate MN node which means that the SCGFailureInformation would have to be reported by UE to the candidate MN node which means that the respective MN is source MN. Based on the Observation 2-3 such a scenario applies only for case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Observation 2-4: Sending the CPAC failure type is only applicable from candidate to source MN which is only possible in case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Proposal 2-4: RAN3 discuss and analyses the suitability of including the CPAC Failure type into SCG Failure Indication due to its limited applicability only for case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

# 3	Conclusions

We have the following observations and proposals to enhance MRO for LTM:

Observation 1-1: In case of inter DU LTM cell switch the TA acquisition type shall always be available in the CU.

Observation 1-2: In case of intra DU LTM cell switch the TA acquisition type is not available in the CU.

Proposal 1-1: RAN3 to discuss the network-based solution for TA acquisition type for intra DU LTM cell switch.

Proposal 1-2: RAN3 agrees to add new binary indicator into Cell ID IE (representing target cell) of the DU-CU CELL SWITCH NOTIFICATION message (TS 38.473) informing CU about gNB indicated TA.

Proposal 1-3: RAN3 agrees the attached TP for TS 38.473 in Annex 1 to update the Semantics description for the C-RNTI.

- Proposal 1-4: RAN3 to discuss the following information to be included in the RLF report:

- Proposal 1-5: RAN3 agrees on new failure cause “Invalid TA” to be communicated from CU to the source DU.

- Proposal 1-6: RAN3 to support the optimization of BFR shortly after successful LTM cell switch.

- Observation 1-3: as BFR if working properly in this scenario, we can assume that the problem is related to LTM HO configurations at the source cell (DU/CU) that led to a wrong beam selection for the LTM cell switch execution

- Observation 1-4: For proper identification of the problem scenario, it is important that the network can properly categorize the BFR(s) that happen “shortly” after the LTM cell switch and can differentiate them from the BFR(s) that may happen after the incoming UE has already stayed in the target cell for some longer time period.

- Proposal 1-7: Target DU to signal BFR shortly after LTM cell switch information to the CU, and the CU forwards this information to source DU.

- Proposal 1-8: Source DU stores the L1 radio measurements reported by the outgoing UE that it used for the cell switch decision. Source DU may use those measurements for the root cause analysis and to decide about any corrective actions for the cell switch command decision.

Observation 1-5: No extra information needs to be reported in RLF report to optimize LTM cell switch failure due to wrong beam.

Proposal 1-9: RAN3 agrees the enhanced RLF report in rel. 19 for LTM is sufficient to optimize LTM cell switch failure due to wrong beam and no extra network-based solution is needed.

To support MRO for CHO with candidate SCG(s), we propose what follows:

Observation 2-1: The agreement that the last serving MN may need to send message to the candidate MN(s) which may need optimization applies also for case 7a.

Proposal 2-1: RAN3 agrees the attached TP for TS 38.300 in Annex 2 to update the scenario when the last serving MN may need to send message to the candidate MN(s) which may need optimization also for handover to wrong cell.

Proposal 2-2: RAN3 agrees the attached TP for TS 38.423 in Annex 3 to add Handover to wrong cell with SCG(s) cause.

Observation 2-2: The agreement that the serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed applies for case 1b, 3b, 7b, 8b, 9b.

Observation 2-3: The respective MN is candidate MN in case 1b, 3b, 9b while the respective MN is source MN in case 7 (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Proposal 2-3: RAN3 agrees the attached TP for TS 38.300 in Annex 2 to add stage 2 text about the SCGFailureInformation forwarding respecting the fact that respective MN may by candidate or source MN.

Observation 2-4: Sending the CPAC failure type is only applicable from candidate to source MN which is only possible in case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Proposal 2-4: RAN3 discuss and analyses the suitability of including the CPAC Failure type into SCG Failure Indication due to its limited applicability only for case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

# References

- Report of 3GPP TSG RAN3 meeting #127

- Report of 3GPP TSG RAN3 meeting #126

- Report of 3GPP TSG RAN2 meeting #129

- Report of 3GPP TSG RAN2 meeting #128

# Annex 1: Text proposal to TS 38.473 (MRO for LTM)

First change, ommited text not changed

# 9	Elements for F1AP Communication

[snip]

## 9.2	Message Functional Definition and Content

[snip]

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU ® gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU added in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

Remaining text not changed

# Annex 2: Text proposal to TS 38.300 (MRO for CHO with Candidate SCG(s))

First change, ommited text not changed

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO or CHO with candidate SCG(s) is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover or Handover to wrong cell when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may send a Handover Report over Xn to the candidate NG-RAN node(s) controlling the candidate PSCell(s) whose mobility configuration caused the failure .

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

Retrieval of information needed for problem analysis

In order to retrieve relevant information collected at the network side as part of the UE context, the UE provides C-RNTI used in the last serving cell. If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure. If the NG RAN node controlling that source cell provided the Mobility Information, it is also included in the HANDOVER REPORT message. If used, the Mobility Information is prepared at the source NG RAN node of a handover and may refer to or identify any handover-related data at this NG RAN node. In the Handover Preparation procedure, the source gNB can request the target gNB to provide information on DL LBT failures at the target gNB during handover execution.

Handling multiple reports from a single failure event

In case the RRC re-establishment fails and the RRC connection setup succeeds, MRO evaluation of intra-RAT mobility connection failures may be triggered twice for the same failure event. In this case, only one failure event should be counted.

Next change, ommited text not changed

#### 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure is broughthas been caused by anothera respective M- NG-RAN node configuration, the serving M-NG-RAN node transfers the SCG Failure Information to thate respective M-NG-RAN node where the mobility configuration caused the failure.

Remaining text not changed

# Annex 3: Text proposal to TS 38.423 (MRO for CHO with Candidate SCG(s))

First change, ommited text not changed

# TP for TS38.423 on CHO with candidate SCG(s)

### 8.4.8	Handover Report

#### 8.4.8.1	General

The purpose of the Handover Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

#### 8.4.8.2	Successful Operation

Figure 8.4.8.2-1: Handover Report, successful operation

NG-RAN node1 initiates the procedure by sending the HANDOVER REPORT message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment or re-connected either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

If the Handover Report Type IE is set to "HO to wrong cell for CHO with candidate SCG(s)", then NG-RAN node1 indicates to NG-RAN node2 that, a radio link failure occurred during the handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, and the UE attempted RRC Re-establishment or re-connected at another cell which may be due to incorrect CPAC configuration. The detection of HO to wrong cell for CHO with candidate SCG(s) is based on detection of Handover to Wrong Cell event which is made according to TS 38.300 [9].

If the Handover Report Type IE is set to "Too Late CHO with candidate SCG", then NG-RAN node2 shall deduce that a too late CHO with candidate SCG from a cell of NG-RAN node1 to a cell of NG-RAN node2 occurred which may be due to incorrect CPAC configuration. The detection of Too Late CHO with candidate SCG event is made according to TS 38.300 [9].

Editor’s note: The above text may be refined to capture the usage of the mandatory Target cell CGI IE.

The HANDOVER REPORT message may include:

-	the Mobility Information IE, if the Mobility Information IE was sent for this handover from NG-RAN node2 (in case the NG-RAN node2 provided it more than once, the most recent Mobility Information IE is included in the HANDOVER REPORT message);

-	the Source cell C-RNTI IE.

-	the CHO Configuration IE, if the CHO Configuration IE was sent for this handover from NG-RAN node2.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

If the Handover Report Type IE is set to "Inter-system ping-pong", then NG-RAN node2 shall deduce that a completed handover from a cell of NG-RAN node2 to a cell in another system might have resulted in an inter-system ping-pong and the UE was successfully handed over to a cell of NG-RAN node1 (indicated with Target cell CGI IE).

If the Target cell C-RNTI IE and the Time Since Failure IE are received, NG-RAN node2 uses the information as specified in TS 38.300 [9].

Interaction with the Failure Indication procedure:

If NG-RAN node1 receives a UE RLF Report from an NG-RAN node via the FAILURE INDICATION message, as described in TS 38.300 [9], NG-RAN node1 may also include it in the UE RLF Report Container IE included in the HANDOVER REPORT message.

Next change, ommited text not changed

#### 9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …,Too Late CHO with candidate SCG, HO to wrong cell for CHO with candidate SCG(s)) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cellIf the Handover Report Type is set to “Too Late CHO with candidate SCG”, it is the candidate cell in the NG-RAN node 2. FFS on the new semantics above. | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | UE RLF Report 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node. | YES | ignore |
| Time Since Failure | O |  | INTEGER (0.. 172800, ...) | Corresponds to the TimeSinceFailure IE received from the UE in RLF Report as defined in TS 36.331 [14]. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

Remaining text not changed