3GPP TSG-RAN WG3 Meeting #127bis	R3-251631

Wuhan, China, 7-11 April 2025

Agenda Item:		21.3

Source:		Ofinno

Title:	Support of Multi-Modality Awareness for XR

Document for:		Discussion and Decision

# Introduction

In RAN #107 meeting, the following objectives were added for WID on XR Phase 3 [2]:

-  Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-  Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-  Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

-  QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-  Support of exposure of available data rate [RAN3]

Note: Coordination with SA2, as needed

This contribution discusses the issues on multi-modality awareness for QoS flows in both DL and UL RAN.

# Discussion

In last SA2 meeting, the following agreements were agreed in [4] for 23.501.

### 5.37.2 Policy control enhancements to support multi-modal services … If the AF provides a Multi-modal Service ID for data flows to 5GS, then the PCF may include it, based on operator configuration, in PCC rules, and the SMF forwards it to NG-RAN in N2 SM information when establishing and/or updating the corresponding QoS Flows. NOTE X: Based on an SLA between the network operator and an application service provider, the Multi-modal Service ID can uniquely identify a specific multi-modal communication service instance. The related 5GS behavior is not further specified in this specification and is assumed to be determined by the operator configuration related to the SLA.

Based on the agreement above, SMF may send Multi-modal Service ID to NG-RAN when establishing and/or updating QoS flows. It seems straightforward to enhance PDU SESSION RESOURCE SETUP and PDU SESSION RESOURCE MODIFY messages in NGAP to align with SA2 agreements.

Proposal 1: To enhance PDU SESSION RESOURCE SETUP and PDU SESSION RESOURCE MODIFY messages in NGAP to align with SA2 agreements on Multi-modal Service ID.

In addition, SA2 agreements clearly mentioned that it is applied “when establishing and/or updating the corresponding QoS Flows”. With current structure for PDU SESSION RESOURCE SETUP/MODIFY procedure, i.e., shown as follows:

- PDU SESSION RESOURCE SETUP/MODIFY

- PDU Session Resource Setup/Modify Request Transfer

>> QoS Flow Setup Request List

>>> QoS Flow Identifier

>>> QoS Flow Level QoS Parameters

So, the following stage 3 change for NGAP is proposed to reflect the per QoS flow level establishing and/or updating.

#### 9.3.1.12	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … | … |  | … |  |  |  |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1.. 1800, …) | Indicates the reporting frequency for RAN part delay for QoS monitoring. Units: second | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| MMSID | O |  | INTEGER (1.. 32, …) | Multi-modal Service ID as defined in TS 23.501 [9] | YES | ignore |

Proposal 2: To add MMSID in QoS Flow Level QoS Parameters IE for PDU SESSION RESOURCE SETUP/MODIFY messages in NGAP.

From RAN3 point of view, the QoS flows with the same MMSID may be handed over to another NG-RAN node or offloaded to a Secondary NG-RAN node, thus the target NG-RAN node and the Secondary NG-RAN node may take the MMSID into account for admission control during the handover or dual connectivity procedures.

Proposal 3: To enhance HANDOVER REQUEST, S-NG-RAN NODE ADDITION and S-NG-RAN NODE MODIFICATION messages in XnAP for admission control during the handover or dual connectivity procedures.

With current structure for HANDOVER REQUEST, S-NG-RAN NODE ADDITION and S-NG-RAN NODE ADDITION messages, i.e., shown as follows:

- HANDOVER REQUEST, S-NG-RAN NODE ADDITION/MODIFICATION

- PDU Session Resources To Be Setup List, PDU Session Resource Setup Info – SN or MN terminated

>> QoS Flow To Be Setup List

>>> QoS Flow Identifier

>>> QoS Flow Level QoS Parameters

The stage 3 change for XnAP is given as follows:

#### 9.2.3.5	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … | … | … |  |  |  |  |
| QoS Monitoring Disabled | O |  | ENUMERATED(true, ...) | Indicates to stop the QoS monitoring. | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  | Indicates the PDU Set QoS Parameters. | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.2.3.203 |  | – |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.2.3.203 |  | – |  |
| MMSID | O |  | INTEGER (1.. 32, …) | Multi-modal Service ID as defined in TS 23.501 [9] | YES | ignore |

Proposal 4: To add MMSID in QoS Flow Level QoS Parameters IE for HANDOVER REQUEST, S-NG-RAN NODE ADDITION and S-NG-RAN NODE MODIFICATION messages in XnAP.

# Proposals

Proposal 1: To enhance PDU SESSION RESOURCE SETUP and PDU SESSION RESOURCE MODIFY messages in NGAP to align with SA2 agreements on Multi-modal Service ID.

Proposal 2: To add MMSID in QoS Flow Level QoS Parameters IE for PDU SESSION RESOURCE SETUP/MODIFY messages in NGAP.

Proposal 3: To enhance HANDOVER REQUEST, S-NG-RAN NODE ADDITION and S-NG-RAN NODE MODIFICATION messages in XnAP for admission control during the handover or dual connectivity procedures.

Proposal 4: To add MMSID in QoS Flow Level QoS Parameters IE for HANDOVER REQUEST, S-NG-RAN NODE ADDITION and S-NG-RAN NODE MODIFICATION messages in XnAP.

# Reference

[1] R3-251501, RAN3#127bis Meeting Agenda

[2] RP-240107, Revised WID on XR for NR Phase 3

[3] S2-2502498, Support of multi-modal awareness at NG-RAN

[4] S2-2502463, Provisioning Multi-modal Service ID to NG-RAN