3GPP TSG-RAN WG3 #127bis	R3-251689

Wuhan, China, April 7th – 11th, 2025

Agenda item:		16.3

Title:		Discussion on AIoT location reporting aspects

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The following progress was made during the last RAN3 meeting:

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report. The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Command Report? AIoT RAN node sends the Inventory Report aggregated per Reader. “Global gNB ID + Reader index” is used to uniquely identify the Reader globally. FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

This contribution will discuss the open issues of Ambient IoT.

# 2. Discussion

2.1	Discussion on the transmission of Reader ID

During the last meeting, the Reader ID was defined as “Global gNB ID + Reader index”. Whether the Reader index or the combination of “Global gNB ID + Reader index” should be sent in the Inventory Report requires further discussion.

As RAN3 agreed, the Inventory Report is transmitted over the NG interface. During the establishment of the NG interface, the Global gNB ID should be transmitted to the AMF in the case of an indirect connection or to the AIoTF in the case of a direct connection.

In the direct connection case, the AIoTF can obtain the full Reader ID even if the Inventory Report includes the “Reader index”.

In the indirect connection case:

- If the AMF sends the Global gNB ID to the AIoTF, it is sufficient for the Inventory Report to contain only the “Reader index”.

- If the AMF does not send the Global gNB ID to the AIoTF, the Inventory Report should contain the “Global gNB ID + Reader index” to identify the transmitting readers.

Observation 1: In the direct connection case, the AIoTF can know the Global gNB ID of the transmitting reader. In contrast, in the indirect connection case, whether the AIoTF knows the Global gNB ID of the transmitting reader depends on whether the AMF sends the Global gNB ID to the AIoTF.

Therefore, in the direct connection case, the Inventory Report can only include the Reader Index. In the indirect connection case, it depends on the decision of SA2 whether the AMF sends the Global gNB ID to the AIoTF.

Proposal 1: In the direct connection case, the Inventory Report can only include the Reader Index when sent to the AIoTF.

Proposal 2: In the indirect connection case:

- If the AMF sends the Global gNB ID to the AIoTF, it is sufficient for the Inventory Report to contain only the “Reader index”.

- If the AMF does not send the Global gNB ID to the AIoTF, the Inventory Report should contain the “Global gNB ID + Reader index” to identify the transmitting readers.

Regarding the issue of whether to include the AIoT device's location at the reader ID granularity in the Command Report sent to the AIoTF, as previously discussed in another contribution, the Command Report should be considered in later release. Consequently, we do not support to include the AIoT device’s location at reader ID granularity to the AIoTF in Command Report in Rel-19. Furthermore, we think the Command Response should include the AIoT device's location at the reader ID granularity to reflect RAN3 progress in the TR 38.769 that:

### 6.9.2 Topology 1 For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages. The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded. NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion.

Proposal 3: We prefer not to include the AIoT device’s location at the reader ID granularity in the Command Report sent to the AIoTF in Rel-19.

Proposal 4: The AIoT device’s location at the reader ID granularity should be included in the Command Response message sent to the AIoTF.

2.2	Whether multiple Reader ID(s) can be included in location report

In topology 1, an A-IoT RAN node may serve one or more readers. Some companies proposed that multiple readers could be used to get a higher-accuracy device location. One AIoT device may respond to more than one reader in an AIoT RAN node. For example, the AIoT device may be covered by more than one reader. This means that the network could figure out where the device is by using multiple reader IDs, which is a more accurate location than the "reader ID" granularity location. As RAN3 discussed in previous meetings, it greatly increases the complexity of the AIoT location mechanism, and the benefit is very limited.

However, we still think the inventory report/command response should be able to include multiple reader IDs, but not for AIoT device locations. It is possible that the AIoT RAN may merge the AIoT device responses from the multiple readers into a single Inventory Report/Command Response. And there is another case in which the different readers under an AIoT RAN node may receive the same Inventory Request/Command Response from the AIoTF; a single AIoT device may respond to the same Inventory Request/Command Response to the different readers. In that case, the AIoT RAN node may include multiple reader IDs in the Inventory Report.

Observation 2: There are two scenarios in which the Inventory Report may contain multiple reader IDs. The first is that AIoT RAN may merge the AIoT device responses from the multiple readers into a single Inventory Report. The second is that the different readers under an AIoT RAN node may receive the same inventory request from the AIoTF; a single AIoT device may respond to the same inventory request to the different readers.

Proposal 5: The Inventory Report/Command Response should be able to include multiple Reader IDs, but not for AIoT device location reasons.

# 3. Conclusion

Observation 1: In the direct connection case, the AIoTF can know the Global gNB ID of the transmitting reader. In contrast, in the indirect connection case, whether the AIoTF knows the Global gNB ID of the transmitting reader depends on whether the AMF sends the Global gNB ID to the AIoTF.

Observation 2: There are two scenarios in which the Inventory Report may contain multiple reader IDs. The first is that AIoT RAN may merge the AIoT device responses from the multiple readers into a single Inventory Report. The second is that the different readers under an AIoT RAN node may receive the same inventory request from the AIoTF; a single AIoT device may respond to the same inventory request to the different readers.

Proposal 1: In the direct connection case, the Inventory Report can only include the Reader Index when sent to the AIoTF.

Proposal 2: In the indirect connection case:

- If the AMF sends the Global gNB ID to the AIoTF, it is sufficient for the Inventory Report to contain only the “Reader index”.

- If the AMF does not send the Global gNB ID to the AIoTF, the Inventory Report should contain the “Global gNB ID + Reader index” to identify the transmitting readers.

Proposal 3: We prefer not to include the AIoT device’s location at the reader ID granularity in the Command Report sent to the AIoTF in Rel-19.

Proposal 4: The AIoT device’s location at the reader ID granularity should be included in the Command Response message sent to the AIoTF.

Proposal 5: The Inventory Report/Command Response should be able to include multiple Reader IDs, but not for AIoT device location reasons.

# References