3GPP TSG-RAN WG3 Meeting #127bis														R3-251727

Wuhan, China, 7th – 11th April 2025

Agenda item:	12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.401) Discussion on NG management and Xn management for WAB

Document for:	Discussion and Decision

# 1	Introduction

RAN#105 agreed WI on topological enhancements for NR [1]. The objectives for WAB are following, [1]:

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management. NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB. NOTE 2: NG connection management should take the NTN conclusion into account, avoiding parallel discussions. NOTE 3: No impact on the UE. NOTE 4: Coordination with other WGs (e.g. SA2, RAN2) when needed. NOTE 5: Backhaul link for WAB-MT can be TN or NTN. NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures. NOTE 7: The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase. NOTE 8: Split architecture of the WAB-gNB is out-of-scope for the normative phase. NOTE 9: RAN2 impact should be identified as early as possible, and should be minimal.

This contribution analyses the NG management, Xn management, mobility, and adjusting the QoS based on backhaul type.

# 2	NG interface management

Last meeting agreed TP to capture the NG connection management. Some companies propose introducing a cause value for such NG Removal. Such a cause value is not agreed for NR NTN Regenerative system and as said WAB is similar to the gNB in NR NTN Regenerative system and therefore no cause value is required. Furthermore, this new cause value needs new behaviour at AMF, thus, no cause value is provided for NG removal.

Proposal 1-1: No cause value is provided for NG Removal.

For NG Setup procedure, a new indication may be needed in NG SETUP REQUEST message to tell the AMF that the peer NG-RAN node is a WAB-gNB. This may be used by the AMF, e.g. used in the position procedure for a UE connected with the WAB-gNB, etc.

Proposal 1-2: new “WAB-gNB” indication is needed in NG SETUP REQUEST message, to inform AMF that peer NG-RAN node is a WAB-gNB.

# 3	Xn interface management

Previous meeting RAN3 agreed

Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures.

However, there is no TP captured in the TR 38.799 for Xn management (e.g. Xn establishment) for the WAB-gNB.

Xn Establishment

Xn establishment between WAB-gNB and the BH-gNB serving WAB-MT

When the WAB-MT connects to a BH-gNB serving WAB-MT, the WAB-MT knows the TAI/CGI of the serving cell. After the integration procedure (i.e. WAB-MT connected with the gNB serving WAB-MT, WAB-gNB setup NG with the AMF, BH PDU session establishment), the WAB-gNB can initiate the Xn-C TNL address discovery procedure towards the BH-gNB serving WAB-MT. The WAB-gNB may then initiate the TNL/Xn Setup with the BH-gNB serving WAB-MT.

Proposal 2-1: WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT.

Xn establishment between WAB-gNB and neighboring gNB

The surrounding RAN node can be a stationary gNB, or a WAB-gNB. There is different view on whether the Xn can be setup between the two WAB-gNBs.

In Rel-18, the mobile IAB acts as a gNB-DU/cell. The Xn between the IAB-donor and its neighboring gNB may be already established before mobile IAB connects with IAB-donor. The arrival/departure of a mobile IAB does not affect the Xn between the IAB-donor and its neighboring gNB, other than possibly informing neighbors about newly deployed/released IAB cell(s). From the perspective of a neighboring gNB, the arrival/departure of a mobile IAB can be considered to add/remove cell(s) of the serving IAB-donor.

The WAB brings a new challenge to the inter-gNB neighbor relation. Unlike a new Rel-18 mobile IAB appearing as a new gNB-DU/cell to neighboring gNB, WAB appears as a new gNB to the neighboring gNB. The arrival/departure of mobile WAB causes dynamic setup/release of the Xn between the WAB-gNB and its neighboring gNB(s).

In legacy system, the gNB can initiate the ANR procedure whenever the UE reports an unknown PCI, i.e. a PCI that is not in the NCRT. This can be similar for WAB, e.g. WAB initiate the ANR procedure whenever the UE reports an unknown PCI. However, WAB faces another challenge, for example, when the UE reports a known PCI, the WAB cannot know whether the PCI is related to an existing cell/entry in its NCRT, or the PCI belongs to a new neighboring cell that is not in the NCRT. This is due to the mobility nature of WAB and PCI reuse for cells in different geographical areas.

Figure 1: PCI reuse by cells of gNB1 and gNB3

An example is shown in the above figure.

- When the WAB is at location A, it detects the new cell with PCI#1 (i.e. gNB1’s cell), WAB initiates ANR procedure to add gNB1’s cell (PCI#1) in its NCRT.

- Later, when WAB moves to location B which is covered by gNB3’s cell with same PCI#1, WAB cannot know whether the PCI#1 corresponds to an existing entry in NCRT that is related to gNB1’s cell, or corresponds to a new cell.

So, it is necessary for WAB-gNB to know the neighboring cell, and a neighboring gNB to know WAB cell as early as possible. In legacy system, the Xn setup is either triggered by OAM, or by ANR based on the UE’s measurement report. However, in the mobile WAB case, the WAB-gNB or neighboring gNB may not be able to know the right time that it needs to ask UE to perform a measurement in order to detect the cell using a specific PCI is changed.

Observation 2-1: Due to PCI reuse, it may be difficult for WAB-gNB (or gNB) to determine the right time to ask UE to perform a measurement in order to detect the new neighbouring cell.

One option is to exchange the neighboring cell information over Xn interface, in order for WAB-gNB to know the neighboring cell, and a neighboring gNB to know WAB cell as early as possible.

The WAB-gNB can reuse the existing mechanism to detect the neighboring cell of a gNB. Since the WAB has a gNB functionality, it can initiate the ANR procedure based on the measurement report from a connected UE, or from the measurement report of the co-located WAB-MT. In addition, the WAB-gNB can also use the information received from BH-gNB serving WAB-MT over Xn.  In most of the scenarios, the WAB’s neighboring gNB is also a neighboring gNB to the BH-gNB. Since the BH-gNB may already setup Xn with the neighboring gNB, BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, thus WAB-gNB can directly initiate the Xn setup procedure with neighboring gNB. Alternatively, during the Xn Setup or NG-RAN node configuration update procedure, the BH-gNB serving WAB-MT can provide its neighboring cell information to the WAB-gNB. The WAB-gNB can use the neighboring cell information received from the BH-gNB to update its NCRT, or initiate Xn-C TNL address discovery procedure for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Proposal 2-2: BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbour gNB.

Proposal 2-3: WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Furthermore, in case Xn is to be avoided between WAB-gNBs (as proposed by some companies in the previous meeting due to the nature of WAB being mobile, which would lead frequent Xn setup and Xn release), this could even be avoided during TNL discovery procedure, i.e., as early as possible. For example, source WAB-gNB can inform the target gNB that it is a WAB during TNL discovery, and target can refrain sending TNL information in case the target itself is a WAB-gNB. That way, Xn establishment can be avoided at the earliest time possible and there would be no unnecessary additional signalling.

Proposal 2-4: If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Xn Removal

When the WAB move away, the WAB may need to remove the Xn with previous surrounding gNBs. This can be supported by the existing Xn Removal procedure.

Observation 2-2: Existing Xn Removal procedure can be reused to remove the Xn between WAB-gNB and surrounding gNB.

Support for RRC INACTIVE

A UE connected to the WAB-gNB may transition to RRC INACTIVE, then move to the surrounding gNB’s coverage.  Similarly, a UE connected to the surrounding gNB may transition to RRC INACTIVE, then move to the WAB-gNB’s coverage. The RNA assigned to the UE may include the RAN area ID/cell ID of WAB-gNB and surrounding gNB.

Observation 2-3: A UE’s RNA can consist of RAN area ID(s)/cell ID(s) of WAB-gNB(s) and surrounding gNB(s).

Observation 2-4: An RRC_CONNECTED UE can transition to RRC_INACTIVE via WAB-gNB.

However, to support the Xn Retrieve UE Context procedure, the Xn Removal procedure should be carefully designed to avoid the scenario where UE context cannot be retrieved due to released Xn interface between the target gNB and the WAB-gNB. This is especially an issue considering WAB’s are moving devices, and Xn may be established and removed dynamically as the WAB moves. In our view, there are at least a couple of ways to tackle the issue:

- Avoiding “Too early Xn Removal”, i.e. avoiding that Xn is removed before Xn Retrieve UE Context Procedure is initiated. This could be, e.g., avoided by rejecting the Xn removal request.

- In case Xn is to be released between the WAB-gNB and another gNB, whose cell(s) are in the RNA of at least one of the UEs that WAB-gNB sent to RRC_INACTIVE, UE context of the corresponding UE(s) are provided to the gNB by the WAB-gNB.

Proposal 2-5: RAN3 discuss the enhancement to avoid the UE context retrieval problems when Xn is removed between a WAB-gNB and a surrounding gNB.

# 4	PCI re-configuration for WAB

When the mobile WAB moves (with or without change of UE’s AMF), there may be a potential PCI collision between the cells from different WAB, or between WAB’s cell and a stationary gNB-DU’s cell. The WAB may need to change PCI for its cell. The WAB-gNB instantiates a new logical cell using a new PCI. The new cell need to use new cell ID, but TAC can remain unchanged. The WAB-gNB then initiate the intra-gNB handover for the UEs connected with the old WAB-gNB cell to the new WAB-gNB cell. After all the UEs in RRC_CONNECTED state are handed over, the old logical WAB-gNB cell is removed from service.

Observation 3: It is possible that a WAB-gNB cell’s PCI may need to be changed when the WAB node moves to a new location.

Proposal 3: to support the change of the PCI used by a WAB-gNB’s cell, the WAB-gNB instantiates a new logical cell using a new PCI, new cell ID and old TAC. The WAB-gNB initiate the intra-gNB handover for the UEs connected with the old WAB-gNB cell to the new WAB-gNB cell. After all the UEs in RRC_CONNECTED state are handed over, the old logical WAB-gNB cell is removed from service.

Stage-2 TP can be found in Annex – TP to BL CR for TS 38.401

# 5	WAB-MT mobility

Last meeting agreed TP to describe the handling of the IP address change of the WAB-gNB’s traffic, more specifically, following text:

-	NG-C and Xn-C can be migrated to the new IP addresses via legacy procedures defined in TS 38.412[36] and TS 38.422[37], respectively.

-	NG-U GTP-U tunnels can be migrated via the legacy NGAP PDU Session Resource Modify Indication procedure.

It is FFS whether a WAB gNB can act as an MN or SN in Dual Connectivity.

First, the WAB node may be deployed as a stationary node. In such deployment, the WAB-gNB can act as a MN or SN for a UE. Since the WAB node is stationary, it will not experience the mobility or change of IP address due to WAB node’s mobility.

In case the WAB node is a mobile node, DC may be setup for a UE. For example, when the train stops at the station, DC may be setup using the WAB-node installed on the train and a stationary gNB. But this type of DC will soon be released when the train left, and no need to maintain the DC after WAB-MT’s IP address is changed due to WAB node’s mobility.

One may argue that DC can be setup using two mobile WAB nodes deployed on the same vehicle or ship. This may be a corner case. In case it happens, existing MN/SN initiated modification procedure can be used to update the Xn-U endpoint. Also, MOBIKE can be used to avoid the change of inner IP address. So no enhancement is needed.

Proposal 4-1: In case the WAB-MT’s IP address is changed due to WAB node’s mobility, there is no need to introduce enhancement to handle the DC Xn-U traffic.

The other scenario is Xn HO involving WAB-gNB.

- Handover between a stationary WAB-gNB and neighboring stationary gNB does not have issue.

- Handover between a mobile WAB-gNB and a stationary gNB is a corner case, since it implies the UE is moving to/from a mobile WAB-gNB.

- Handover between mobile WAB-gNBs installed on the same train/ship.

This scenario may happen. In normal handover, the Xn-U tunnel only exist for a short period. However, in case the WAB-gNB initiates CHO with early data forwarding, the Xn-U tunnel may exist for a period. But the issue may be mitigated by only using normal HO or only using CHO with late data forwarding, if the source gNB know the peer gNB is a WAB-gNB (e.g. via a WAB indication over Xn). There may be no strong reason to introduce new enhancement

Proposal 4-2: no need to introduce enhancement to handle the handover Xn-U traffic during WAB node’s mobility.

# 6	WAB-gNB mobility with change of UE’s AMF(s)

Last meeting agreed following TP:

For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location.

The new logical WAB-gNB may activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

Editor’s Note 1:	FFS if the new cell(s) can belong to the WAB-gNB serving the old cell(s).

There is no reason for the new logical WAB-gNB setup NG, but not activate any cell. The new logical WAB-gNB shall activate one or more cell(s).

For the EN, SA2 agreed no consensus on signal WAB-gNB solution. Considering SA2 completed the WI, there is no need to continue this discussion in RAN3. So the EN can be deleted.

Proposal 5: update Stage-2 TP to use “shall” and delete the EN in below Stage-2 TP:

For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location.

The new logical WAB-gNB shall activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

# 7	Report of BH type to CN

There are some contributions proposed in last meeting regarding the NTN backhaul. In current system, the CN need to know the existence of the satellite link used in the BH due to the long delay of the satellite link. By knowing the existence of the satellite link, the CN can take appropriate action, e.g. not to establish a QoS flow with a QoS that cannot be met via the satellite link.

In WAB deployment, there are two cases that the satellite link can be used

- Case 1: the WAB-MT connects to BH-gNB via a satellite link, and the BH-gNB uses a terrestrial link for connecting with CN.

- Case 2: the WAB-MT connects to BH-gNB via a terrestrial network link, but the BH-gNB uses a satellite link for its backhaul connection with CN.

In case 1, it is a TN gNB (i.e. WAB-gNB) using satellite backhaul. Current CN can know the satellite link via existing method defined in TS 23.501 (copied as below):

NOTE: It is assumed that the AMF can determine the Satellite backhaul category for the notification to the SMF based on local configuration, e.g. based on Global RAN Node IDs associated with satellite backhaul.

In legacy gNB using satellite backhaul, its backhaul link is fixed after deployment. So by configuration, the AMF can determine the Satellite backhaul for the gNB.  However, WAB brings more challenge due to the WAB node’s mobility and the WAB-MT may be handover between TN BH-gNB and NTN BH-gNB. So the CN need to be aware of the WAB-gNB’s BH is changed from satellite backhaul to terrestrial backhaul, or vice versa. The existing method by using local configuration (e.g. based on Global RAN Node ID) will not work for WAB-gNB. The WAB-MT can be handover between a TN BH-gNB and NTN BH-gNB, but the gNB ID used by the co-located WAB-gNB remains unchanged. RAN3 need to study how to address this issue.

In case 2, the WAB-gNB/MT cannot know the BH-gNB is using a satellite link. It is suggested RAN3 discuss this issue. Possible solution could be the BH-gNB informs WAB-gNB via Xn or informs WAB-MT via RRC about the satellite link, then the WAB-gNB report it to CN. Using Xn may be better in order to avoid RRC changes.

Proposal 6: RAN3 discuss how a WAB node know the BH-gNB is using a satellite link.  Possible options include BH-gNB informs WAB-gNB via Xn, or informs WAB-MT via RRC.

# 8	Conclusion

In this contribution, we discussed the architecture and other high-level aspects of WAB. Our proposals are:

Proposal 1-1: No cause value is provided for NG Removal.

Proposal 1-2: new “WAB-gNB” indication is needed in NG SETUP REQUEST message, to inform AMF that peer NG-RAN node is a WAB-gNB.

Proposal 2-1: WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT.

Proposal 2-2: BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbors gNB.

Proposal 2-3: WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Proposal 2-4: If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Proposal 2-5: RAN3 discuss the enhancement to avoid the UE context retrieval problems when Xn is removed between a WAB-gNB and a surrounding gNB.

Proposal 3: to support the change of the PCI used by a WAB-gNB’s cell, the WAB-gNB instantiates a new logical cell using a new PCI, new cell ID and old TAC. The WAB-gNB initiate the intra-gNB handover for the UEs connected with the old WAB-gNB cell to the new WAB-gNB cell. After all the UEs in RRC_CONNECTED state are handed over, the old logical WAB-gNB cell is removed from service.

Proposal 4-1: In case the WAB-MT’s IP address is changed due to WAB node’s mobility, there is no need to introduce enhancement to handle the DC Xn-U traffic.

Proposal 4-2: no need to introduce enhancement to handle the handover Xn-U traffic during WAB node’s mobility.

Proposal 5: update Stage-2 TP to use “shall” and delete the EN in below Stage-2 TP:

For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location.

The new logical WAB-gNB shall activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

Proposal 6: RAN3 discuss how a WAB node know the BH-gNB is using a satellite link.  Possible options include BH-gNB informs WAB-gNB via Xn, or informs WAB-MT via RRC.

Stage-2 TP can be found in Annex – TP to BL CR for TS 38.401

# References

- RP-242395, New WID on additional topological enhancements for NR

- TR 38.799 v19.0.0

- R3-250945, (TS38.401 BL CR) Support for Wireless Access Backhaul

# Annex – TP to BL CR for TS 38.401

## X.6 	PCI collision avoidance

In WAB-node deployments, the legacy mechanism for PCI collision avoidance can be reused. PCI space can be partitioned by allocating a range of PCIs to WAB-gNB cells. In case the PCI used by the WAB-gNB’s cell need to be changed, the WAB-gNB instantiates a new logical cell using a new PCI, new cell ID and old TAC. The WAB-gNB initiate the intra-gNB handover for the UEs connected with the old WAB-gNB cell to the new WAB-gNB cell. After all the UEs in RRC_CONNECTED state are handed over, the old logical WAB-gNB cell is removed from service.

## X.7	WAB-node mobility

### X.7.1	WAB-MT mobility

The WAB-MT reuses legacy mobility procedures defined for the UE. During the WAB-node’s movement, when the BH PDU session(s) of WAB-MT are re-established, the co-located WAB-gNB may need to update the IP addresses used for its traffic. In case IPsec tunnel mode is used to protect the WAB-gNB’s traffic, MOBIKE (IETF RFC 4555 [29]) can be used to avoid the change of inner IP addresses used for this traffic. Otherwise, following procedures can be used for handling the IP address change of the WAB-gNB’s traffic:

- NG-C and Xn-C can be migrated to the new IP addresses via legacy procedures defined in TS 38.412[36] and TS 38.422[37], respectively.

- NG-U GTP-U tunnels can be migrated via the legacy NGAP PDU Session Resource Modify Indication procedure.

The continuity of OAM connectivity of the WAB-gNB needs to be ensured as the WAB-node moves across the BH network.

### X.7.2	WAB-gNB mobility

#### X.7.2.1 	WAB-gNB mobility with change of UE’s AMF(s)

Due to the WAB-gNB’s mobility, the UEs moving together with the WAB-gNB may have to change AMF.

For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location.

The new logical WAB-gNB shall activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

Editor’s Note 1:	FFS if the new cell(s) can belong to the WAB-gNB serving the old cell(s).

The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over from a cell served by the old logical WAB-gNB to a cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [7], after which the UE’s AMF is changed to the new AMF.

- A UE in RRC_IDLE or RRC_INACTIVE state camping on a cell served by the old logical WAB-gNB reselects a cell served by the new logical WAB-gNB. The reselection may be triggered by the removal of the old logical WAB-gNB’s cells from service. After cell reselection, the UE performs a Mobility Registration Update as defined in TS 23.502 [7], which is triggered by the new TAC broadcasted by the new logical WAB-gNB’s cell. During this Mobility Registration Update, the UE’s AMF is changed to the new AMF.

After all the UEs in RRC_CONNECTED state are handed over, the NG connection(s) between the WAB-node and the old AMF(s) are removed via NG Removal procedure and the old logical WAB-gNB’s cell(s) are removed from service.

### x.y2	Management of Xn interface for WAB-gNB

The WAB-gNB may establish Xn interface with surrounding gNBs. The BH-gNB may provide the information of neighboring gNB (e.g. gNB ID, TNL address) to the WAB-gNB via Xn interface, which enables the WAB-gNB to establish Xn with the neighboring gNB(s).

The WAB-gNB may release the Xn interface with a neighboring gNB, e.g. when the WAB node move away from the neighboring gNB. The Xn removal should also consider the stored UE context for RRC INACTIVE UE(s), so the Xn interface removal will not cause issue to further XnAP Retrieve UE Context procedure.