3GPP TSG-RAN WG3 Meeting #127-bis	R3-251753

Wuhan, China, 7-11 April, 2025

Title: 	(TP to BLCR for TS 38.473) Discussion on common signal adaptation

Source: 	Huawei, CATT, CMCC

Agenda item:	17.4

Document Type:	 Other

# 1. Introduction

In this contribution, we provide our considerations on the common signal/channel adaptation, including SSB, PRACH and paging.

# 2. Discussion

## 2.1 Paging adaptation

The previous RAN2#129 meeting sent the LS on the paging enhancement, containing the following contents in [1].

For Rel-19 paging adaptation, RAN2 discussed that gNB can decide to transmit paging either at legacy PF(s)/PO(s) or at Rel-19 NES PF(s)/PO(s)and agreed to introduce a new UE capability. Agreements on paging enhancement - A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features. # 2 Actions To TSG RAN3, CT1 ACTION: RAN2 respectfully requests RAN3/CT1 to take into account of RAN2 agreements in their future work.

- The detailed agreements on the paging enhancements in RAN2 previous meetings are made and provided as follows.

Agreements @ RAN2#127bis meeting - Select option-b as baseline for R19 NES paging enhancement. - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. - Legacy UE is not barred. - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration. Agreements @ RAN2#129 meeting 1. For N, values smaller than T/32 are not supported. 2. The maximum possible value for Ns is 8. 3. Introduce a separate PEI configuration. 4. Paging adaptations are configured semi-statically and updated via system information update notification. 5. A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

- From the above, it can be observed that for R19 NES paging enhancement:

- a new NES paging configuration was introduced, and those Rel-19 NES UEs only monitor the Rel-19 paging configuration.

- a new UE Radio capability was introduced.

- Over the NG and Xn interface, the UE Radio Capability for Paging IE is included in paging message and the RAN paging message, and refers to the RRC UERadioPagingInformation as defined in TS 38.331, thus the NG and Xn interface have no impact from RAN3 perspective.

Over the NG and Xn interface, there is no specification impact to support the Paging adaptation/enhancements.

- Over the F1 interface, the UE Paging Capability is included in PAGING message. Since the DU needs to decide whether to page the corresponding UEs in legacy POs or NES POs, a new Paging adaptation/enhancements indication is needed in the UE Paging Capability to indicate the NES POs.

Over the F1 interface, introduce a new Paging adaptation/enhancement indication in the UE Paging Capability.

- For PEI, it was agreed in RAN2 that a separate PEI configuration is needed for R19 NES UEs. RAN3 can be pending till further progress on the detailed separate PEI configuration/capability made in other WGs.

RAN3 is pending on the separate PEI configuration.

## 2.2 SSB adaptation

For SSB adaptation, RAN1 and RAN2 made the following agreements:

Agreement@ RAN1#119 meeting Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell Agreement@ RAN1#120 meeting For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement@ RAN1#120 meeting For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation Agreement@ RAN2#129 meeting 1. RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell. 2. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 3. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

It can be observed that for the SSB adaptation:

- it is only applicable for RRC connected UEs, but not for RRC idle/inactive UEs;

- Only the SSB burst periodicity adaptation for the NCD-SSB is supported.

- The SSB adaption is applicable only for SCells.

From RAN3 perspective, since the NCD-SSB is always broadcasted with the periodicity adaptation, this cell can be considered as a normal cell. In addition, RAN2 agreed that the neighbor cell measurement is based on the legacy SMTC, and additional SMTC configuration would be introduced, which can be decided and encoded by the CU. Hence RAN3 can assume that there is no RAN3 impact, which can be revisited upon further progress.

For SSB adaptation over SCell, RAN3 assumes there is no RAN3 impact, which can be revisited upon further progress.

## 2.3 PRACH adaptation

For PRACH adaptation, significant agreements are made in other WGs and provided as follows.

Agreement@ RAN1#116bis meeting Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Agreement@ RAN1#120 meeting For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration - FFS: Validity time duration for availability is configured by higher layer signaling or predefined - FFS: Location of the reference point defined in the specification - FFS: Value/granularity of the validity time duration. - FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources Agreements @ RAN2#129 meeting 1. Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed. 2a. RAN2 starts CBRA for RACH adaptation. 2b. RAN2 starts 4-step RACH adaptation. 3. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. 4. From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. 5a. From R2 perspective, RACH adaptation is not modelled as RA feature(s). 5b. From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain. 6. Will follow legacy mechanism regarding how to select RACH resource.

In current F1AP and XnAP specification, the NR PRACH Configuration is included in Served Cell information IE, containing NR PRACH Configuration List for the UL carrier or the SUL carrier. Given that the detailed PRACH configuration/signalling are still under discussion, it is not clear whether there is anything missing from the current IE.  Hence RAN3 can be pending till further progress is made in other WGs.

RAN3 is pending on the PRACH adaptation.

# 4. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Over the NG and Xn interface, there is no specification impact to support the Paging adaptation/enhancements.

Over the F1 interface, introduce a new Paging adaptation/enhancement indication in the UE Paging Capability.

RAN3 is pending on the separate PEI configuration.

For SSB adaptation over SCell, RAN3 assumes there is no RAN3 impact, which can be revisited upon further progress.

RAN3 is pending on the PRACH adaptation.

- The TP for TS 38.473 to implement the Proposal 2 is provided in Annex.

# 5. Reference

- R2-2501483 LS on paging enhancement in R19 NES, RAN2

- RP-241650 Revised WID: Enhancements of network energy savings for NR, Ericsson

- Chair notes RAN1#117 eom4.

# TP for TS 38.473

## 8.7	Paging procedures

### 8.7.1	Paging

#### 8.7.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

The PEIPS Assistance Information IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [6].

The RedCap Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging of the RedCap UE or the eRedCap UE.

The Last Used Cell Indication IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, consider the cell identified by the NR CGI IE as the last used cell of the paged UE, and use it as specified in TS 38.331 [8].

The Recommended SSBs List IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, use it to send the paging message over the indicated SSB beams.

The PEI Subgrouping Support Indication IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication as described in TS 38.300 [6] and TS 38.304 [24].

The UE Paging Capability IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, take it into account when paging the UE.

The Extended UE Identity Index Value IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Hashed UE Identity Index Value IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The MT-SDT Information IE may be included in the PAGING message. If present the gNB-DU shall, if supported, use it for MT-SDT paging as specified in TS 38.331 [8].

The NR Paging Long eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present, the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Paging Adaptation IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging adaptation for the concerned UE.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.270	UE Paging Capability

This IE provides the UE Paging Capability information needed for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| INACTIVE State PO-Determination | O |  | ENUMERATED(supported,…) | Corresponds to the inactiveStatePO-Determination contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | - |  |
| RedCap Indication | O |  | ENUMERATED(true,…) | Indicates that the paged UE is a Redcap UE or an eRedCap UE. | YES | ignore |
| Paging Adaptation [FFS] |  |  | ENUMERATED(supported,…) | Corresponds to the [FFS] contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>