3GPP TSG-RAN WG3 Meeting #127-bis	R3-251754

Wuhan, China, 7-11 April, 2025

Title: 	(TP to BLCR for TS 38.413 and TS 38.423 and TS 38.300) Discussion on Low-power wake-up signal and receiver for NR

Source: 	Huawei

Agenda item:	18.2

Document Type:	Other

# 1. Introduction

In this contribution, we continue to discuss the RAN3 related issues on the procedure and configuration for RRC_IDLE/INACTIVE UEs.

# 2. Discussion

## 2.1 LP-WUS Subgrouping

- In the RAN1-120 meeting [1], it was confirmed that the maximum number of subgroups per PO for LP-WUS was set to 31.

Agreement at RAN1#120 Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31.

With this RAN1 agreement, RAN3 can set the maximum number of LP-WUS CN-based subgroup ID as 31.

RAN3 to set the maximum number of LP-WUS CN-based subgroup ID as 31.

In the current spec, the gNB will be provided by the core network or other gNB with an IE of Extended UE Identity Index Value carried in the Core Network Assistance Information for RRC INACTIVE IE in NGAP, or in the XnAP RAN Paging message.

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

The Extended UE Identity Index Value IE comprises 16 least significant bits of 5G-S-TMSI (i.e., UE ID), in which up to 12 bits are to determine the PF and PO (if configured with eDRX) and up to 15 bits (if configured with eDRX) are used for PEI subgrouping as specified in TS 38.304 [2].

In the latest RAN2-129 meeting [3], RAN2 made the conclusion of the design of UE ID based subgrouping as follows.

Agreement at RAN2#128 For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

From TS 38.331, the maximum number of the subgroupNumForUEID is eight. Hence in such case, at least 18 bits (if not configured with eDRX) and 20 bits (if configured with eDRX) are used in UE ID for PO/PF determination, PEI subgrouping and LP-WUS subgrouping, the current Extended UE Identity Index Value IE with 16 bits is definitely insufficient.

Therefore, RAN3 should consider to introduce a new IE with at least a size of 20 bits for LP-WUS UE based subgrouping.

Observation 1: The current size of Extended UE Identity Index Value IE is not sufficient if both PEI and LP-WUS are supported by UE and gNB.

RAN3 to introduce a new IE with a size of at least 20 bits to fully carry the bit information of the 5G-S-TMSI.

## 2.2 Discussion on UE capability

### 2.2.1 LP-WUS capability indication in the F1 paging message

In the RAN1’s LS [4], it is described that the UE may not support LP-WUS reception on all the bands supported by the UE, which means that the LP-WUS capability would be a band-specific capability for the UE. In the latest RAN3-127 meeting, it was agreed that LP-WUS subgrouping support indication should be introduced per-cell in F1 paging message as a working assumption.

Working Assumption at RAN3#127 Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

This was also confirmed by UE feature list [5] from RAN1 as well as the agreement made in RAN2#129 that UE shall report the supported band(s) to the network.

Agreement at RAN1#120 Agreement: Support Alt-2 below: - Alt-2: Introduce the following FG(s), where the supported LP-WUS/WUR waveform and operation are reported in a FG, with FFSs on whether/which to define as a basic FG for LP-WUS/WUR (and potential prerequisite relationship in between), Type and further coupling/decoupling of the FG(s) X-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; [Per Band]; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. X-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported; [Per Band]; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. Agreement at RAN2#129 On separate band issue RAN2 understands that UE can report which band(s) is supported by LR to NW.

Thus, RAN3 can turn the above working assumption into agreement, and remove the FFS in the paging message from the F1AP BLCR.

RAN3 to turn the working assumption to the agreement, i.e., the LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping. And the FFS in the paging message in the F1AP CR can be removed.

### 2.2.2 UE radio capability for paging information

According to the current mechanism, the UE shall report its LP-WUS capability via the UE capability reporting procedure. After receiving the UECapabilityInformation message, the gNB decodes it and accordingly generates the UERadioPagingInformation inter-node RRC message which contains UE’s paging related capabilities including the LP-WUS capability. Then the gNB transfer it to the CN or other gNBs.

Observation 2: LP-WUS related capability needs to be introduced in UE radio capabilities over the Uu interface and UE radio paging capabilities over the network interfaces.

The LS from SA2 [6] discussed the scenario involving the pre-Rel 19 gNBs and Rel-19 gNBs supporting LP-WUS. As illustrated in Fig.1, if the UE reports the LP-WUS capability to a legacy gNB (i.e. the gNB cannot recognize LP-WUS capability), the legacy gNB cannot include LP-WUS capability in UERadioPagingInformation inter-node message, so the LP-WUS capability is actually not maintained by the CN. However, when the UE moves to the cells where LP-WUS is supported, it does not know this situation and will use LP-WUS for paging monitoring later in these cells. However, the gNB does not get any LP-WUS capability from CN for the paged UE, which results in the UE monitoring LP-WUS while the gNB directly sends paging message to the UE in its POs. In such case, the UE will miss the paging message as it does not wake up in its PO, which means the UE is already unreachable.

Fig.1 An example for the LP-WUS capability maintenance and mismatch issue

Observation 3: If the UE reports LP-WUS capability to an old gNB that does not forward LP-WUS capability to CN, the UE will be unreachable under new gNBs, since the UE will use LP-WUS for paging, while the new gNBs does not use LP-WUS due to the missing LP-WUS capability in the UE Radio Capability for Paging.

RAN3 to confirm the non-homogeneous scenario mentioned in the LS, i.e., the UE will be unreachable due to the LP-WUS mismatch between UE and gNB, if UE does not know the gNB fails to be forward the LP-WUS capability to CN.

In the previous RAN2#129 meeting, it was agreed that no LP-WUS capability mismatch issue exists between the UE and the network after Release 17, since the related UE-ID based subgrouping capability can be included in the UE-RadioPagingInfo-r17 container, and this container can be accessed by the gNB beyond Release 17.

Agreement at RAN2#129 Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

While for the pre-17 gNBs, there still exists the mismatch issue. To further consider the solution to address the capability mismatch issue between the UE and the pre-R17 gNB, we provide the following possible solutions.

- Option 1: OAM based - Homogeneous support of LP-WUS

The simplest approach is to ensure the homogenous support of LP-WUS in a given area, e.g., across the entire TA area. Once all gNBs in a given area are upgraded simultaneously, the capability mismatch issue would no longer exist, and this solution does not have any impact on the specifications. However, multi-vendors scenario was emphasized in SA2 discussion, achieving simultaneous upgrades for multi-vendors may be challenging.

Observation 4: Leaving to network deployment does not have any impact on the specifications, but may be challenging for multi-vendors scenario.

- Option 2: UE based solution- UE gets information from gNB whether LP-WUS capability can be forwarded to the CN

As another way, if the UE can know unambiguously whether the reported LP-WUS capability will be forwarded by the gNB and maintained by the CN, the mismatch of LP-WUS capability can be solved. For example, only if the gNB indicates the UE to report LP-WUS capability, or indicates the UE that LP-WUS capability forwarding is supported, the UE can report the LP-WUS capability and can use LP-WUS in a cell where LP-WUS is supported by the gNB. Otherwise, the LP-WUS capable UE cannot use LP-WUS. This option is a universal solution to support the non-homogeneous deployment. Given this solution would have UE impact, the details should be left to RAN2 decision.

RAN3 to discuss the following two options for pre-R17 gNBs:

- Option 1: OAM based - Homogeneous support of LP-WUS

- Option 2: UE based - UE gets information from the gNB whether LP-WUS capability can be forwarded to the CN

## 2.3 Last used cell only

The agreed XnAP BLCR has editor notes whether to introduce the LP-WUS subgrouping capability related information into the XnAP RAN paging message.

For Rel-17 PEI, each cell may broadcast the lastUsedCellOnly IE in SIB1, to indicate that the PEI can only be used in the last used cell. Typically, for an inactive UE moved from the anchor cell to the new cell, if the new cell broadcasts lastUsedCellOnly IE, the UE will not monitor the PEI, and the new gNB will not send PEI for this UE when it receives Xn RAN paging message from the anchor cell. On the contrary, if the new cell does not broadcast the lastUsedCellOnly IE, the UE should monitor the PEI instead.

Due to the fact that the anchor cell does not know whether the neighbor cells broadcast this lastUsedCellOnly IE or not, it should always include the PEIPS Assistance Information in the RAN paging message. Then it is up to the receiving cell to decide to use PEI or not. Hence for LP-WUS, it seems a very same procedure would be adopted as PEI, but this needs to be by RAN2 first.

RAN3 to keep the editor’s note in the XnAP BLCR pending RAN2 progress.

## 2.4 Emergency call back

For the emergency PDU session, RAN2 made the following FFS in RAN2-128:

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

Also, at the previous SA2-167 meeting, the unnecessary restrictions for PEIPS and LP-WUS related to emergency PDU session are removed in the agreed CR [7]. And it seems that SA2 made consensus that the PSAP callback is delivered as normal call in IMS PDU session. There is no need for the UE and network to disable functionalities like PEIPS or LP-WUS. Hence in case of CN paging for idle UE, the LP-WUS CN based subgrouping could be included as the normal call in IMS PDU session. Then for the RAN paging for the RRC inactive UE, the LP-WUS CN based subgrouping can also be used.

If this is acknowledged by the group, the stage 2 BLCR can be updated by removing the restrictions of LP-WUS in case of emergency PDU session for RRC inactive UE.

RAN3 to update the stage2 BLCR by removing the restrictions of LP-WUS in case of emergency PDU session.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- LP-WUS Subgrouping

- RAN3 to set the maximum number of LP-WUS CN-based subgroup ID as 31.

Observation 1: The current size of Extended UE Identity Index Value IE is not sufficient if both PEI and LP-WUS are supported by UE and gNB.

RAN3 to consider to introduce a new IE with a size of at least 20 bits to fully carry the bit information from the 5G-S-TMSI.

- UE capability

RAN3 to turn the working assumption to the agreement, i.e., the LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping. And the FFS in the paging message in the F1AP CR can be removed.

Observation 2: LP-WUS related capability needs to be introduced in UE radio capabilities over the Uu interface and UE radio paging capabilities over the network interfaces.

Observation 3: If the UE reports LP-WUS capability to an old gNB that does not forward LP-WUS capability to CN, the UE will be unreachable under new gNBs, since the UE will use LP-WUS for paging, while the new gNBs does not use LP-WUS due to the missing LP-WUS capability in the UE Radio Capability for Paging.

RAN3 to confirm the non-homogeneous scenario mentioned in the LS, i.e., the UE will be unreachable due to the LP-WUS mismatch between UE and gNB, if UE does not know the gNB fails to be forward the LP-WUS capability to CN.

Observation 4: Leaving to network deployment does not have any impact on the specifications, but may be challenging for multi-vendors scenario.

RAN3 to discuss the following two options for pre-R17 gNBs:

- Option 1: OAM based - Homogeneous support of LP-WUS

- Option 2: UE based - UE gets information from the gNB whether LP-WUS capability can be forwarded to the CN

- Last used cell only

RAN3 to keep the editor’s note in the XnAP BLCR pending RAN2 progress.

- Emergency call back

RAN3 to update the stage2 BLCR by removing the restrictions of LP-WUS in case of emergency PDU session.

The TPs to NGAP and XnAP are provided in the Annex to capture the above agreements. And the F1AP CR is provided in [7].

# 4. Reference

- RAN1#120 chairman notes.

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state

- RAN2#129 chairman notes.

- R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE model;

- R1-2501394 Session Notes of AI 9.15.5

- S2-2411297/R2-2409226, LS Reply on LP-WUS subgrouping.

- S2-2502428 Removing unnecessary restrictions for emergency PDU sessions, Qualcomm

# 5. TP to BLCR for TS 38.413

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| UE Specific DRX | O |  | Paging DRX 9.3.1.90 |  | - |  |
| Periodic Registration Update Timer | M |  | 9.3.3.24 |  | - |  |
| MICO Mode Indication | O |  | 9.3.1.23 |  | - |  |
| TAI List for RRC Inactive |  | 1 |  |  | - |  |
| >TAI List for RRC Inactive Item |  | 1..<maxnoofTAIforInactive> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Expected UE Behaviour | O |  | 9.3.1.93 |  | - |  |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.3.52 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| MICO All PLMN | O |  | 9.3.1.194 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause Indication for Voice Service | O |  | ENUMERATED (supported, …) | This IE indicates whether the UE supports the feature of indication of paging cause for voice service. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.3.3.62 |  | YES | ignore |
| CN MT Communication Handling | O |  | ENUMERATED (supported, …) | This IE indicates the CN support of CN based MT communication handling. | YES | ignore |
| LP-WUSPS Assistance Information | O |  | 9.3.1.x |  | YES | ignore |
| LP-WUSPS UE Identity Index Value | O |  | 9.3.3.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforInactive | Maximum no. of TAIs for RRC Inactive. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	LP-WUSPS Assistance Information

This IE provides the information related to LP-WUS paging subgrouping for a particular UE, as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUS CN Subgroup ID | M |  | INTEGER (0..30, …) |  |

#### 9.3.3.y	LP-WUSPS UE Identity Index Value

This IE is used by the NG-RAN node to calculate the UE_ID based subgroup ID for LP-WUS as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUSPS UE Identity Index Value | M |  | BIT STRING (SIZE(20)) |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

IntersystemSONConfigurationTransfer,

LAI,

LocationReportingRequestType,

LPWUSPSAssistanceInformation,

LPWUSPSUEIdentityIndexValue,

LTE-A2X-ServicesAuthorized,

LTEM-Indication,

LTEUESidelinkAggregateMaximumBitrate,

LTEV2XServicesAuthorized,

MaskedIMEISV,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-IntersystemSONConfigurationTransferDL,

id-IntersystemSONConfigurationTransferUL,

id-LocationReportingRequestType,

id-LPWUSPSAssistanceInformation,

id-LPWUSPSUEIdentityIndexValue,

id-LTE-A2X-ServicesAuthorized,

id-LTE-A2X-UE-PC5-AggregateMaximumBitRate,

id-LTEM-Indication,

id-LTEUESidelinkAggregateMaximumBitrate,

id-LTEV2XServicesAuthorized,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PAGING

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PagingIEs} },

...

}

PagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UEPagingIdentity				CRITICALITY ignore	TYPE UEPagingIdentity					PRESENCE mandatory	}|

{ ID id-PagingDRX						CRITICALITY ignore	TYPE PagingDRX							PRESENCE optional		}|

{ ID id-TAIListForPaging				CRITICALITY ignore	TYPE TAIListForPaging					PRESENCE mandatory	}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority						PRESENCE optional		}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging			PRESENCE optional		}|

{ ID id-PagingOrigin					CRITICALITY ignore	TYPE PagingOrigin						PRESENCE optional		}|

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging			PRESENCE optional		}|

{ ID id-NB-IoT-Paging-eDRXInfo			CRITICALITY ignore	TYPE NB-IoT-Paging-eDRXInfo				PRESENCE optional		}|

{ ID id-NB-IoT-PagingDRX				CRITICALITY ignore	TYPE NB-IoT-PagingDRX					PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction	CRITICALITY ignore	TYPE Enhanced-CoverageRestriction		PRESENCE optional		}|

{ ID id-WUS-Assistance-Information		CRITICALITY ignore	TYPE WUS-Assistance-Information			PRESENCE optional		}|

{ ID id-EUTRA-PagingeDRXInformation		CRITICALITY ignore	TYPE EUTRA-PagingeDRXInformation		PRESENCE optional		}|

{ ID id-CEmodeBrestricted				CRITICALITY ignore	TYPE CEmodeBrestricted					PRESENCE optional		}|

{ ID id-NR-PagingeDRXInformation		CRITICALITY ignore	TYPE NR-PagingeDRXInformation			PRESENCE optional		}|

{ ID id-PagingCause						CRITICALITY ignore	TYPE PagingCause						PRESENCE optional		}|

{ ID id-PEIPSassistanceInformation		CRITICALITY ignore	TYPE PEIPSassistanceInformation			PRESENCE optional		}|

{ ID id-LPWUSPSAssistanceInformation	CRITICALITY ignore	TYPE LPWUSPSAssistanceInformation		PRESENCE optional		}|

{ ID id-LPWUSPSUEIdentityIndexValue		CRITICALITY ignore	TYPE LPWUSPSUEIdentityIndexValue		PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-IntersystemMobilityFailureforVoiceFallback,

id-LastEUTRAN-PLMNIdentity,

id-LastVisitedPSCellList,

id-LocationReportingAdditionalInfo,

id-LPWUSPSAssistanceInformation,

id-LPWUSPSUEIdentityIndexValue,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- C

CoreNetworkAssistanceInformationForInactive-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-EUTRA-PagingeDRXInformation					CRITICALITY ignore	EXTENSION EUTRA-PagingeDRXInformation					PRESENCE optional	}|

{ ID id-ExtendedUEIdentityIndexValue				CRITICALITY ignore	EXTENSION ExtendedUEIdentityIndexValue					PRESENCE optional	}|

{ ID id-UERadioCapabilityForPaging					CRITICALITY ignore	EXTENSION UERadioCapabilityForPaging						PRESENCE optional	}|

{ ID id-MicoAllPLMN									CRITICALITY ignore	EXTENSION MicoAllPLMN											PRESENCE optional	}|

{ ID id-NR-PagingeDRXInformation					CRITICALITY ignore	EXTENSION NR-PagingeDRXInformation						PRESENCE optional	}|

{ ID id-PagingCauseIndicationForVoiceService		CRITICALITY ignore	EXTENSION PagingCauseIndicationForVoiceService		PRESENCE optional	}|

{ ID id-PEIPSassistanceInformation					CRITICALITY ignore	EXTENSION PEIPSassistanceInformation						PRESENCE optional	}|

{ ID id-HashedUEIdentityIndexValue					CRITICALITY ignore	EXTENSION HashedUEIdentityIndexValue						PRESENCE optional	}|

{ ID id-CN-MT-CommunicationHandling					CRITICALITY ignore	EXTENSION 	 CN-MT-CommunicationHandling				PRESENCE optional	}|

{ ID id-LPWUSPSAssistanceInformation				CRITICALITY ignore	EXTENSION LPWUSPSAssistanceInformation					PRESENCE optional	}|

{ ID id-LPWUSPSUEIdentityIndexValue					CRITICALITY ignore	EXTENSION LPWUSPSUEIdentityIndexValue						PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

LoggedMDTTrigger ::= CHOICE{

periodical			NULL,

eventTrigger			EventTrigger,

choice-Extensions		ProtocolIE-SingleContainer { {LoggedMDTTrigger-ExtIEs} }

}

LoggedMDTTrigger-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LPWUSPSAssistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions		ProtocolExtensionContainer { {LPWUSPSAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (0..30, ...)

LPWUSPSUEIdentityIndexValue ::= BIT STRING (SIZE(20))

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-LPWUSPSAssistanceInformation						    ProtocolIE-ID ::= aaa

id-LPWUSPSUEIdentityIndexValue							ProtocolIE-ID ::= bbb

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

# 6. TP to BLCR for TS 38.423

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34], and for eDRX or the UE_ID based subgrouping according to TS 38.304 [33]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards the NG-RAN node2.

When available, the NG-RAN node1 shall include the E-UTRA Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the E-UTRA Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE Specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the NR Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the NR Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the NR Paging eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

When available, the NG-RAN node1 shall include the Hashed UE Identity Index Value IE in the RAN PAGING message towards the NG-RAN node2. If the Hashed UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33] or TS 36.304 [34].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the NR Paging Long eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the LP-WUSPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

Editor’s note: the inclusion of the LP-WUSPS Assistance Information IE is to be finally confirmed based on whether a UE will be configured to monitor LP-WUS, either only in the last used cell, if indication is sent in the system information, or any other cell (after cell reselection).

If the LP-WUSPS UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.304 [33].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE Specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.2.3.161 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.2.3.162 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.2.3.144a |  | YES | ignore |
| MT-SDT Information | O |  | 9.2.3.172 |  | YES | ignore |
| NR Paging Long eDRX Information for RRC INACTIVE | O |  | 9.2.3.195 |  | YES | ignore |
| LP-WUSPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |
| LP-WUSPS UE Identity Index Value | O |  | 9.2.3.y |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x	LP-WUSPS Assistance Information

This IE provides the information related to LP-WUS paging subgrouping for a particular UE, as specified in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUS CN Subgroup ID | M |  | INTEGER (0..30, …) |  |

#### 9.2.3.y 	LP-WUSPS UE Identity Index Value

This IE is used by the NG-RAN node to calculate the UE_ID based subgroup ID for LP-WUS as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUSPS UE Identity Index Value | M |  | BIT STRING (SIZE(20)) |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

ReportingPeriodicityForDataCollection,

NodeAssociatedInfoResult,

SLPositioning-Ranging-Services-Info,

PDUSessionsListToBeReleased-UPError,

UserPlaneFailureIndication,

SRSPositioningConfigOrActivationRequest,

NRPPaPositioningInformation,

LPWUSPSassistanceInformation,

LPWUSPSUEIdentityIndexValue,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-LPWUSPSassistanceInformation,

id-LPWUSPSUEIdentityIndexValue,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue						CRITICALITY reject	TYPE UEIdentityIndexValue								PRESENCE mandatory}|

{ ID id-UERANPagingIdentity							CRITICALITY ignore	TYPE UERANPagingIdentity									PRESENCE mandatory}|

{ ID id-PagingDRX									CRITICALITY ignore	TYPE PagingDRX												PRESENCE mandatory}|

{ ID id-RANPagingArea								CRITICALITY reject	TYPE RANPagingArea											PRESENCE mandatory}|

{ ID id-PagingPriority								CRITICALITY ignore	TYPE PagingPriority											PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging					CRITICALITY ignore	TYPE AssistanceDataForRANPaging						PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging					CRITICALITY ignore	TYPE UERadioCapabilityForPaging						PRESENCE optional }|

{ ID id-ExtendedUEIdentityIndexValue				CRITICALITY ignore	TYPE ExtendedUEIdentityIndexValue						PRESENCE optional }|

{ ID id-EUTRAPagingeDRXInformation					CRITICALITY ignore	TYPE EUTRAPagingeDRXInformation						PRESENCE optional }|

{ ID id-UESpecificDRX								CRITICALITY ignore	TYPE UESpecificDRX											PRESENCE optional }|

{ ID id-NRPagingeDRXInformation						CRITICALITY ignore	TYPE NRPagingeDRXInformation							PRESENCE optional }|

{ ID id-NRPagingeDRXInformationforRRCINACTIVE		CRITICALITY ignore	TYPE NRPagingeDRXInformationforRRCINACTIVE			PRESENCE optional }|

{ ID id-PagingCause									CRITICALITY ignore	TYPE PagingCause											PRESENCE optional }|

{ ID id-PEIPSassistanceInformation					CRITICALITY ignore	TYPE PEIPSassistanceInformation						PRESENCE optional }|

{ ID id-HashedUEIdentityIndexValue					CRITICALITY ignore	TYPE HashedUEIdentityIndexValue						PRESENCE optional }|

{ ID id-MT-SDT-Information							CRITICALITY	ignore	TYPE MT-SDT-Information									PRESENCE optional }|

{ ID id-NRPagingLongeDRXInformationforRRCINACTIVE	CRITICALITY ignore	TYPE NRPagingLongeDRXInformationforRRCINACTIVE	PRESENCE optional}|

{ ID id-LPWUSPSassistanceInformation	CRITICALITY ignore	TYPE LPWUSPSassistanceInformation	PRESENCE optional}|

{ ID id-LPWUSPSUEIdentityIndexValue		CRITICALITY ignore	TYPE LPWUSPSUEIdentityIndexValue		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

...,

suspend-lower-layers,

resume-lower-layers

}

LPWUSPSassistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions				ProtocolExtensionContainer { { LPWUSPSassistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (0..30, ...)

LPWUSPSUEIdentityIndexValue ::= BIT STRING (SIZE(20))

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-LPWUSPSassistanceInformation																		ProtocolIE-ID ::= xxx

id-LPWUSPSUEIdentityIndexValue

ProtocolIE-ID ::= yyy

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

# 6. TP to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 9.2.2	Mobility in RRC_INACTIVE

#### 9.2.2.1	Overview

RRC_INACTIVE is a state where a UE remains in CM-CONNECTED and can move within an area configured by NG-RAN (the RNA) without notifying NG-RAN. In RRC_INACTIVE, the last serving gNB node keeps the UE context and the UE-associated NG connection with the serving AMF and UPF.

For a UE in RRC_INACTIVE with eDRX cycle longer than 10.24 seconds, the NG-RAN node may, based on implementation, send a request to the AMF to perform MT Communication Handling as described in TS 23.501 [3]. If the UE accesses a gNB other than the last serving gNB and upon successful UE context retrieval, the AMF considers that the UE is reachable (i.e., the MT Communicating Handling is deactivated) and triggers the MT DL data/signalling delivery upon receiving the NGAP Path Switch Request message.

If the last serving gNB receives DL data from the UPF or DL UE-associated signalling from the AMF (except the UE Context Release Command message) while the UE is in RRC_INACTIVE, it pages in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s).

Upon receiving the RAN Paging Request message from the AMF while the UE is in RRC_INACTIVE with eDRX beyond 10.24 seconds, the last serving gNB may page in its cells comprised in the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s), in order to trigger the UE to resume connection in RRC_CONNECTED state, or in RRC_INACTIVE state for DL SDT, based on the DL data size if included in the NGAP RAN Paging Request message.

Upon receiving the UE Context Release Command message while the UE is in RRC_INACTIVE, the last serving gNB may page in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s), in order to release UE explicitly.

Upon receiving the NG RESET message while the UE is in RRC_INACTIVE, the last serving gNB may page involved UEs in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s) in order to explicitly release involved UEs.

Upon RAN paging failure, the gNB behaves according to TS 23.501 [3].

The AMF provides to the NG-RAN node the Core Network Assistance Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE, and to assist UE configuration and paging in RRC_INACTIVE. The Core Network Assistance Information includes the registration area configured for the UE, the Periodic Registration Update timer, and the UE Identity Index value, and may include the UE specific DRX, an indication if the UE is configured with Mobile Initiated Connection Only (MICO) mode by the AMF, the Expected UE Behaviour, the UE Radio Capability for Paging, the PEI with Paging Subgrouping assistance information, the LP-WUS with Paging Subgrouping assistance information, the NR Paging eDRX Information, the Paging Cause Indication for Voice Service, the Hashed UE Identity Index Value and the CN support indication for MT Communication Handling. The UE registration area is taken into account by the NG-RAN node when configuring the RNA. The UE specific DRX and UE Identity Index value are used by the NG-RAN node for RAN paging. The Periodic Registration Update timer is taken into account by the NG-RAN node to configure Periodic RNA Update timer. The NG-RAN node takes into account the Expected UE Behaviour to assist the UE RRC state transition decision. The NG-RAN node may use the UE Radio Capability for Paging during RAN Paging. The NG-RAN node takes into account the PEI with Paging Subgrouping assistance information for subgroup paging in RRC_INACTIVE except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used according to TS 24.501 [28]. The NG-RAN node takes into account the PEI with Paging Subgrouping assistance information for subgroup paging in RRC_INACTIVE. When sending the XnAP RAN Paging to neighbour NG-RAN node(s), the PEI with Paging Subgrouping assistance information and the LP-WUS with Paging Subgrouping assistance information may be included. The NG-RAN node takes into account the NR Paging eDRX Information to configure the RAN Paging when the NR UE is in RRC_INACTIVE. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the NR Paging eDRX Information for RRC_IDLE and for RRC_INACTIVE may be included. The NG-RAN node takes into consideration the Paging Cause Indication for Voice Service to include the Paging Cause in RAN Paging for a UE in RRC_INACTIVE state. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the Paging Cause may be included. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the Hashed UE Identity Index Value may be included to determine the start point of PTW. The NG-RAN takes into account the CN support indication for MT Communication Handling when deciding to request the AMF for MT Communication Handling for a UE in RRC_INACTIVE state with long eDRX beyond 10.24 seconds as described in TS 23.501 [3].

At transition to RRC_INACTIVE the NG-RAN node may configure the UE with a periodic RNA Update timer value. At periodic RNA Update timer expiry without notification from the UE, the gNB behaves as specified in TS 23.501 [3].

If the UE accesses a gNB other than the last serving gNB, the receiving gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the last serving gNB and may also trigger an Xn-U Address Indication procedure including tunnel information for potential recovery of data from the last serving gNB. Upon successful UE context retrieval, the receiving gNB shall perform the slice-aware admission control in case of receiving slice information and becomes the serving gNB and it further triggers the NGAP Path Switch Request and applicable RRC procedures. After the path switch procedure, the serving gNB triggers release of the UE context at the last serving gNB by means of the XnAP UE Context Release procedure.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>