3GPP TSG-RAN3 Meeting #127bis                                                                R3-251757

# Wuhan, China, April 07th – 11th, 2025

Agenda Item:	21.3

Source: 	Qualcomm Inc.

Title: 	R19 XR Signaling Enhancements

Document for:	Discussion/Agreement

# Introduction

Following are R19 revised XR WID [1] objectives with RAN3 impacts.

-  Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-  Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-  Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

-  QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-  Support of exposure of available data rate [RAN3]

Note: Coordination with SA2, as needed

In this contribution, we propose several signalling enhancements for above XR objectives.

# MMSID From 5GC to RAN

A multi-modal service is a communication service that consists of several data flows that relate to each other and that are subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources (e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs).

The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service; and SA2 agreed that the 5GC would provide the MMSID to the RAN. 5GC will get MMSID from XR AS.

SMF can provide MMSID to RAN per UE per QoS flow level during PDU Session Setup/Modification procedures. One question is whether SMF provides MMSID to RAN based on RAN indication to support multi-modality handling or whether SMF provides MMSID to RAN irrespective of RAN support of multi modality handling.In order to simplify signalling frame work, SMF can provide MMSID per QoS flow independent of RAN indication and if RAN does not support then it can just ignore MMSID received from SMF.

The 5GC/SMF needs to provide MMSID to CU-CP per QoS flow level. Then CU-CP will provide MMSID per QoS flow level to both CU-UP and DU.

NG-AP protocol: The Multi-modal information needs to be added to PDU Session Management messages and to UE Mobility Management messages.

- In NG-AP, the multi-modal information per QoS flow will be provided in following messages from SMF to RAN:

- PDU SESSION RESOURCE SETUP REQUEST

- PDU SESSION RESOURCE MODIFY REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

Xn-AP protocol: During mobility, the multi-modal information needs to be propagated to the target gNB-CU-CP; also in DC scenarios, the multi-modal information needs to be provided to a secondary gNB-CU-CP.

- In Xn-AP, the multi-modal information per QoS flow level is added to:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

- S-NODE ADDITION REQUEST

- S-NODE MODIFICATION REQUEST

- S-NODE MODIFICATION REQUIRED

E1-AP protocol: As the MMSID can be helpful to the RAN to handle QoS flows of the same multi-modal service, the multi-modal information needs to be propagated to the gNB-CU-UP; this can be realized through the Bearer Context Management messages.

- In E1-AP, the multi-modal information per QoS flow level is added to:

- BEARER CONTEXT SETUP REQUEST

- BEARER CONTEXT MODIFY REQUEST

F1-AP protocol: The multi-modal information needs to be propagated to the gNB-DU through the UE Context Management messages.

- In F1-AP, the new multi-modal information per QoS flow level is added to:

- UE CONTEXT SETUP REQUEST

- UE CONTEXT MODIFY REQUEST

# PDU Set Information Marking without PDU Set QoS

From SA2 agreed CR S2-2502672:

If no DL PDU Set QoS parameters are included in the PCC rule, but a DL Protocol Description is included in the PCC rule received by the SMF and the PSA UPF supports PDU Set Handling (i.e. supports to perform PDU Set Information marking), the SMF may, based on the local operator policy, send to the NG-RAN, a DL PDU Set Information Marking Support Indication indicating that the DL PDU Set Information marking is supported.

If the DL PDU Set Information Marking Support Indication has been sent to the 5G-AN (as described in clause 5.37.5.1) and the 5G-AN has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported, the SMF instructs PSA UPF to perform PDU Set marking and provides the PSA UPF the DL Protocol Description used by the service data flow.

5.37.5.3	Non-homogenous support of PDU set based handling in NG-RAN

The SMF, by sending PDU Set QoS parameters (as described in clause 5.7.7.1) or a DL PDU Set Information Marking Support Indication (as described in clause 5.37.5.1) to the NG-RAN, requests the NG-RAN to activate PDU Set handling for a given QoS Flow and the NG-RAN provides the SMF with a PDU Set Based Handling Support Indication if the PDU Set based handling is supported. Based on this indication, SMF may activate the PDU Set identification and marking in the PSA UPF as described in 5.37.5.2.

During mobility procedures that result in the change of NG-RAN, the target NG-RAN provides to the SMF a PDU Set Based Handling Support Indication if it supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the target NG-RAN indication, the SMF may, upon completion of the mobility procedure, initiate the PDU Session modification procedure to provide PDU Set QoS parameters or the DL PDU Set Information Marking Support Indication to NG-RAN and may configure the PSA UPF to activate the PDU Set identification and marking. If the PDU Set Based Handling Support Indication is not received from the target NG-RAN and PDU Set identification and marking is active in the PSA UPF, the SMF may deactivate it.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header, it shall start doing so from a complete PDU Set.

NOTE:	Based on the PDU Set Based Handling Support Indication from the target NG-RAN, the SMF can update the QoS profile and, if applicable, Alternative QoS Profiles as defined in clauses 5.7.1.2 and 5.7.1.2a to include the PDU Set QoS parameters respectively.

This section discusses the impacts of  SA2 CR to the RAN3 specifications:

- How the 5GC signals to the RAN that it supports PDU Set Information Marking

- How the RAN request the 5GC to activate or deactivate PDU Set Information Marking

## Support of PDU Set Information Marking

The 5GC signals its capability to support PDU Set Information Marking without PDU set QoS to the gNB-CU-CP in NG-AP messages and this indication needs to be passed to other gNBs during HO. Following are list of messages that need to be updated with new PDU Set Information Marking IE.,.

NG-AP protocol:

- The new ‘PDU Set Information Marking Support’ IE is added to the NG-AP messages:

- PDU SESSION RESOURCE SETUP REQUEST

- PDU SESSION RESOURCE MODIFY REQUEST

- PATCH SWITCH REQUEST ACKNOWLEDGE

Xn-AP protocol:

During UE mobility procedures: the source node needs to provide the target node with the new ‘PDU Set Information Marking Support’ IE.

- The new ‘PDU Set Information Marking Support’ IE is added to the Xn-AP messages:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT REQUEST

During dual connectivity scenarios: the master node can provide the secondary node with the new ‘PDU Set Information Marking Support’ IE.

- The new ‘PDU Set Information Marking Support’ IE is added to the Xn-AP messages:

- S-NODE ADDITION REQUEST

- S-NODE MODIFICATION REQUEST

## Request for PDU Set Information Marking

Once the NG-RAN knows whether PDU Set Information Marking without PDU set QoS is supported by the 5GC for a specific QoS Flow, the NG-RAN may request the 5GC to dynamically activate or deactivate the PDU Set Information Marking. gNB may use this PDU Set Information received from UPF via GTP-U header for making decisions like which PDUs to be dropped in case of RAN congestion. When there is no congestion, this PDU Set Information is not needed by gNB and gNB can indicate to 5GC that to stop providing PDU Set Information and that will save additional UPF processing.

- A new PDU Set Information Marking Request IE is added to the NG-AP, Xn-AP protocol messages. It is an optional IE composed of a single bit, whose meaning is:

- Absent: No change

- Present and set to 0: Request for deactivation of PDU Set Information Marking

- Present and set to 1: Request for activation of PDU Set Information Marking

NG-AP protocol:

- The new ‘PDU Set Information Marking Request’ IE is added to the NG-AP messages:

- PDU SESSION RESOURCE SETUP RESPONSE

- PDU SESSION RESOURCE MODIFY RESPONSE

- PATCH SWITCH REQUEST

- PDU SESSION RESOURCE MODIFY INDICATION

- PDU SESSION RESOURCE NOTIFY

Xn-AP protocol: If a UE has been configured with DC, the Secondary Node can request the activation or deactivation of the PDU Set Information Marking through UE-specific messages.

- The new ‘PDU Set Information Marking Request’ IE is added to the Xn-AP messages:

- S-NODE ADDITION REQUEST ACKNOWLEDGE

- S-NODE MODIFICATION REQUEST ACKNOWLEDGE

# Alternative UL/DL PDU Set QoS and Notification control for PDU Set QoS

SA2 agreed ([3]):

- The AF may provide Alternative Service requirements to 5GC which may contain the UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

- SMF provides one or more Alternative QoS Profile(s) with UL and/or DL PDU Set QoS parameters to NG-RAN; if QoS notification is determined as "GFBR cannot be guaranteed".

- The NG-RAN determines the AQP that can be used as a reference to the currently fulfilled QoS considering the priority order.

This section discusses the impacts of these agreements to the RAN3 specifications:

- How the 5GC provides the RAN with a list of alternative PDU Set based QoS profiles,

- How the RAN reports an alternative PDU Set based QoS profile, when the main one can’t be met.

## Alternative PDU Set based QoS profiles Configuration

The alternative PDU Set based QoS Profiles Definition can be defined as the legacy PDU based one, except that PSDB and PSER are used instead of PDB and PER.

- In the NG-AP, Xn-AP, E1-AP and F1-AP protocols, introduce a new ‘Alternative PDU Set based QoS Parameters Set List’ similar  as the existing ‘Alternative QoS Parameters Set List’, except the ‘Packet Delay Budget’ and ‘Packet Error Rate’ IEs, which are changed into ‘PDU Set Delay Budget’ and ‘PDU Set Error Rate’ IEs respectively.

- In the NG-AP protocol, the new ‘Alternative PDU Set based QoS Parameters Set List’ IE is added to the messages per QoS flow level:

- PDU SESSION RESOURCE SETUP REQUEST

- PDU SESSION RESOURCE MODIFY REQUEST

- PATCH SWITCH REQUEST ACKNOWLEDGE

- In the E1-AP protocol, the new ‘Alternative PDU Set based QoS Parameters Set List’ IE is added to the messages per QoS flow level:

- BEARER CONTEXT SETUP REQUEST

- BEARER CONTEXT MODIFICATION REQUEST

- In the F1-AP protocol, the new ‘Alternative PDU Set based QoS Parameters Set List’ IE is added to the messages per QoS flow level:

- UE CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

- In the Xn-AP protocol, the new ‘Alternative PDU Set based QoS Parameters Set List’ IE is added to the messages per QoS flow level:

- S-NODE ADDITION REQUEST

- S-NODE MODIFICATION REQUEST

## Alternative PDU Set based QoS profile Notification

In NG-AP specification PDU Session Resource Notify message is used by RAN to notify PDU session resources or QoS flows which are released or not fulfilled anymore or fulfilled again by the RAN node‘Alternative QoS Parameters Set Notify Index’ IE: The value range of this IE is [0..8], where the value 0 represents that no alternative QoS profile can be met. Value 1 to 8 corresponds to Alternative QoS Parameters Set Index.

To report that an alternative PDU Set based QoS profile is met, three options can be envisioned:

Option 1: The same indexe ‘Alternative QoS Parameters Set Notify Index’ isused, without any change.

This option assumes that:

- The node receiving the message knows whether the QoS flow has been configured either with legacy PDU–based or with PDU Set-based Alternative QoS Parameters Set List.

- The QoS Flow cannot carry legacy PDUs (outside PDU Sets) and PDU Sets

- With these 2 assumptions, upon reception of the index, the receiving node can identify the corresponding Alternative Set unambiguously

Option 2: The same indexe ‘Alternative QoS Parameters Set Notify Index’ is used, but their value range is extended:

- [1..8] points to a legacy PDU based Set, and [9..16] points to a PDU Set based Set.

- This option assumes that the QoS Flow cannot carry legacy PDUs (outside PDU Sets) and PDU Sets, but the node receiving the message no longer needs to be aware of whether the QoS flow has been configured either with legacy PDU–based or with PDU Set-based Alternative QoS Parameters Set List

- Upon reception of the index, the receiving node figures out whether the index belongs to a PDU-based or to PDU Set-based Alternative Set from its value.

Option 3: New indexe ‘Alternative PDU Set based QoS Parameters Set Notify Index’ is introduced.

- This option does not make any of the assumptions that options 1 and 2 require.

Option 2 and Option 3 provide unambiguous way of indicating which alternative PDU Set QoS based QoS parameters are met.

- RAN3 to discuss following 2 options for indicating “alternative PDU Set based QoS Parameters indication” in PDU Session Resource Notify message.

Option A: existing ‘Alternative QoS Parameters Set Notify Index’ IE range will be extended to [9 to 16]

Option B : Introduce new ‘Alternative PDU Set based QoS Parameters Set Notify Index’ IE with range [0 to 8]

# Time To Next Burst and Burst Size Signalling

SA2 agreed ([3]) the following:

Time To Next Burst:

- The AF may include information in the DL Protocol Description provided to the NEF/PCF to determine the Time to Next Burst.

- Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Time to Next Burst and to further notify NG-RAN on the Time to Next Burst.

- Based on SMF instruction, the UPF identifies and marks the Time to Next Burst, based on the input carried in N6 DL packet header. The UPF sends the Time to Next Burst to NG-RAN via DL GTP-U head.

Burst Size:

- The AF may indicate to the NEF/PCF to mark the Burst size.

- Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Burst size and to further notify NG-RAN on the Burst size.

- Based on SMF instruction, the UPF identifies and marks the Burst size, based on the input (i.e. Burst size) carried in N6 DL packet header. The UPF sends the Burst size to NG-RAN via DL GTP-U header

- The NG-RAN may use the received Burst size to assist radio resource management

This section discusses the impacts of these two agreements to RAN3 specifications.

The burst size is typically sent in the first PDU of the current data burst, and TTNB may refer to the time difference between the departure time of the last PDU of the current data burst and the departure time of the first PDU of the next data burst, or a time period during which the traffic source guarantee not to send any delay-sensitive PDUs, as shown below.

Figure 1

At the 3GPP SA4 #131 meeting, RTP header extension for dynamically changing traffic characteristics (i.e., data burst traffic characteristics) was agreed, as shown below:

Figure 2

The semantics of the fields are:

- Reserved [R] (8 bits): This field is reserved for future usage, it shall be set to 0 by the RTP sender and shall be ignored by the RTP receiver.

- Burst Size [BSSize] (24 bits): The Burst Size indicates the total size of the burst to be transmitted (in bytes (including the overhead of the RTP Header).). If the burst size is not known it shall be set to 0

- Time To Next Burst [TTNB] (16 bits): Indicates the approximate time in tenth of milliseconds to the next burst. If the time to next burst is not known, it shall be set the reserved value 65535.

The Time To Next Burst and Burst Size need to be added to GTP-U header.

- New ‘TTNB’ and ‘BSSIZE’ Information Elements are added to the DL PDU SET INFORMATION format of the PDU Set Information User Plane protocol

- These two new Information Elements are optional and two bits of the DL PDU SET INFORMATION format are used to signal their presence.

# Reporting of available data rate value for GBR QoS Flow

From SA2 agreed CR [4]

5.45.x	Available bitrate monitoring

The QoS Monitoring for available bitrate allows the measurement and report of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles at the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation.

NOTE 1:	How the NG-RAN measures the available bitrate is NG-RAN implementation specific.

NOTE 2:	The available bitrate can be between the GFBR and the MFBR. The available bitrate can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles.

For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets.

NOTE 3:	The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value and report the UL and/or DL available bitrate when it matches or exceeds a higher threshold value with some hysteresis.

The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. The Reporting threshold(s) in the PCC rule are not sent to UPF. In UPF, the event for the reporting is the reception of available bitrate information from NG-RAN. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed.

During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration.

5.37.4	Network Exposure of 5GS information

The UL and/or DL available bitrate for a GBR QoS Flow (see clause 5.45.x).

For a GBR QoS Flow, based on the PCC rule from PCF, the SMF requests the NG-RAN to report the UL and/or DL available bitrate. The NG-RAN reports the UL and/or DL available bitrate information to the PSA UPF via the GTP-U header of UL packets.

The SMF further requests the PSA UPF to expose the available bitrate to the AF directly or via local NEF, as described in clause 5.8.2.18.

This section discusses the impacts of these two agreements to RAN3 specifications. Two issues need to be addressed:

- The 5GC signals to the RAN the GBR QoS Flows for which available data rate reporting is requested

- The gNB-CU-CP propagates this information to the gNB-CU-UP

- The gNB-CU-UP reports the available data rate through the GTP-U headers of UL frames

## Available Data Rate Reporting Configuration

The request to activate or to deactivate the available data rate reporting is QoS flow specific; two new IEs need to be introduced, a first one for the downlink data rates and a second one for the uplink data rates.

- Two new IEs ‘Downlink Available Data Rate threshold’ and ‘Uplink Available Data Rate threshold’ are added to the NG-AP, Xn-AP and E1-AP protocols.

NG-AP protocol:

- The two IEs from proposal 19 are added to the NG-AP messages:

- PDU SESSION RESOURCE SETUP REQUEST

- PDU SESSION RESOURCE MODIFY REQUEST

- HANDOVER REQUEST

- PATCH SWITCH REQUEST ACKNOWLEDGE

In these messages, the IEs is introduced at the QoS Flow level.

Xn-AP protocol: During mobility, the available data rate request needs to be propagated to the target gNB-CU-CP; also in DC scenarios, it needs to be provided to the secondary gNB-CU-CP.

- In Xn-AP, The two IEs from proposal 19 are added to:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

- S-NODE ADDITION REQUEST

- S-NODE MODIFICATION REQUEST

- S-NODE MODIFICATION REQUIRED

In these messages, the IEs are introduced at the QoS Flow level.

E1-AP protocol: As it is the gNB-CU-UP which will perform and report the measurement, the gNB-CU-CP needs to provide the request to the gNB-CU-UP.

- In E1-AP, the two IEs from proposal 19 are added to:

- BEARER CONTEXT SETUP REQUEST

- BEARER CONTEXT MODIFY REQUEST

In these messages, the IEs are introduced at the QoS Flow level.

## Available Data Rate Report

As per [3], the gNB-CU-UP reports the available data rate of a GBR QoS flow in the GTP-U header of the UL frames, two new IEs need to be introduced, a first one to carry the downlink data rate, and a second one to carry the uplink data rate.

- Two new IEs ‘Downlink Available Data Rate’ and ‘Uplink Available Data Rate’ are added to the UL PDU SESSION INFORMATION frame of the PDU Session User Plane protocol.

- Each of the two IEs is optional, and for each of them, one bit of the UL PDU SESSION INFORMATION frame is reserved to signal its presence.

# Summary

Based on above discussion, we have the following proposals for XR enhancements.

Proposal 1.	In NG-AP, the multi-modal information per QoS flow will be provided in following messages from SMF to RAN:

–	PDU SESSION RESOURCE SETUP REQUEST

–	PDU SESSION RESOURCE MODIFY REQUEST

–	PATH SWITCH REQUEST ACKNOWLEDGE

Proposal 2.	In Xn-AP, the multi-modal information per QoS flow level is added to:

–	HANDOVER REQUEST

–	RETRIEVE UE CONTEXT RESPONSE

–	S-NODE ADDITION REQUEST

–	S-NODE MODIFICATION REQUEST

–	S-NODE MODIFICATION REQUIRED

Proposal 3.	In E1-AP, the multi-modal information per QoS flow level is added to:

–	BEARER CONTEXT SETUP REQUEST

–	BEARER CONTEXT MODIFY REQUEST

Proposal 4.	In F1-AP, the new multi-modal information per QoS flow level is added to:

–	UE CONTEXT SETUP REQUEST

–	UE CONTEXT MODIFY REQUEST

Proposal 5.	The new ‘PDU Set Information Marking Support’ IE is added to the NG-AP messages:

–	PDU SESSION RESOURCE SETUP REQUEST

–	PDU SESSION RESOURCE MODIFY REQUEST

–	PATCH SWITCH REQUEST ACKNOWLEDGE

Proposal 6.	The new ‘PDU Set Information Marking Support’ IE is added to the Xn-AP messages:

–	HANDOVER REQUEST

–	RETRIEVE UE CONTEXT REQUEST

Proposal 7.	The new ‘PDU Set Information Marking Support’ IE is added to the Xn-AP messages:

–	S-NODE ADDITION REQUEST

–	S-NODE MODIFICATION REQUEST

Proposal 8.	A new PDU Set Information Marking Request IE is added to the NG-AP, Xn-AP protocol messages. It is an optional IE composed of a single bit, whose meaning is:

–	Absent: No change

–	Present and set to 0: Request for deactivation of PDU Set Information Marking

–	Present and set to 1: Request for activation of PDU Set Information Marking

Proposal 9.	The new ‘PDU Set Information Marking Request’ IE is added to the NG-AP messages:

–	PDU SESSION RESOURCE SETUP RESPONSE

–	PDU SESSION RESOURCE MODIFY RESPONSE

–	PATCH SWITCH REQUEST

–	PDU SESSION RESOURCE MODIFY INDICATION

–	PDU SESSION RESOURCE NOTIFY

Proposal 10.	The new ‘PDU Set Information Marking Request’ IE is added to the Xn-AP messages:

–	S-NODE ADDITION REQUEST ACKNOWLEDGE

–	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

Proposal 11.	In the NG-AP, Xn-AP, E1-AP and F1-AP protocols, introduce a new ‘Alternative PDU Set based QoS Parameters Set List’ similar  as the existing ‘Alternative QoS Parameters Set List’, except the ‘Packet Delay Budget’ and ‘Packet Error Rate’ IEs, which are changed into ‘PDU Set Delay Budget’ and ‘PDU Set Error Rate’ IEs respectively.

Proposal 12.	In the NG-AP protocol, the new ‘Alternative PDU Set based QoS Parameters Set List’ IE is added to the messages per QoS flow level:

–	PDU SESSION RESOURCE SETUP REQUEST

–	PDU SESSION RESOURCE MODIFY REQUEST

–	PATCH SWITCH REQUEST ACKNOWLEDGE

Proposal 13.	In the E1-AP protocol, the new ‘Alternative PDU Set based QoS Parameters Set List’ IE is added to the messages per QoS flow level:

–	BEARER CONTEXT SETUP REQUEST

–	BEARER CONTEXT MODIFICATION REQUEST

Proposal 14.	In the F1-AP protocol, the new ‘Alternative PDU Set based QoS Parameters Set List’ IE is added to the messages per QoS flow level:

–	UE CONTEXT SETUP REQUEST

–	UE CONTEXT MODIFICATION REQUEST

Proposal 15.	In the Xn-AP protocol, the new ‘Alternative PDU Set based QoS Parameters Set List’ IE is added to the messages per QoS flow level:

–	S-NODE ADDITION REQUEST

–	S-NODE MODIFICATION REQUEST

Proposal 16.	RAN3 to discuss following 2 options for indicating “alternative PDU Set based QoS Parameters indication” in PDU Session Resource Notify message.

Option A: existing ‘Alternative QoS Parameters Set Notify Index’ IE range will be extended to [9 to 16]

Option B : Introduce new ‘Alternative PDU Set based QoS Parameters Set Notify Index’ IE with range [0 to 8]

Proposal 17.	New ‘TTNB’ and ‘BSSIZE’ Information Elements are added to the DL PDU SET INFORMATION format of the PDU Set Information User Plane protocol

Proposal 18.	These two new Information Elements are optional and two bits of the DL PDU SET INFORMATION format are used to signal their presence.

Proposal 19.	Two new IEs ‘Downlink Available Data Rate threshold’ and ‘Uplink Available Data Rate threshold’ are added to the NG-AP, Xn-AP and E1-AP protocols.

Proposal 20.	The two IEs from proposal 19 are added to the NG-AP messages:

–	PDU SESSION RESOURCE SETUP REQUEST

–	PDU SESSION RESOURCE MODIFY REQUEST

–	HANDOVER REQUEST

–	PATCH SWITCH REQUEST ACKNOWLEDGE

In these messages, the IEs is introduced at the QoS Flow level.

Proposal 21.	In Xn-AP, The two IEs from proposal 19 are added to:

–	HANDOVER REQUEST

–	RETRIEVE UE CONTEXT RESPONSE

–	S-NODE ADDITION REQUEST

–	S-NODE MODIFICATION REQUEST

–	S-NODE MODIFICATION REQUIRED

In these messages, the IEs are introduced at the QoS Flow level.

Proposal 22.	In E1-AP, the two IEs from proposal 19 are added to:

–	BEARER CONTEXT SETUP REQUEST

–	BEARER CONTEXT MODIFY REQUEST

In these messages, the IEs are introduced at the QoS Flow level.

Proposal 23.	Two new IEs ‘Downlink Available Data Rate’ and ‘Uplink Available Data Rate’ are added to the UL PDU SESSION INFORMATION frame of the PDU Session User Plane protocol.

Proposal 24.	Each of the two IEs is optional, and for each of them, one bit of the UL PDU SESSION INFORMATION frame is reserved to signal its presence.

# References

[1] RP-250107: Revised WID on XR (eXtended Reality) for NR Phase 3

[2] TS 23.501: System architecture for the 5G System (5GS); Stage 2

[3] TR 23.700-070: Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2

[4] S2-2502675: KI#9, Support of Available Data Rate