3GPP TSG-RAN WG3 Meeting #127	R3-251762

Wuhan, China, 7 – 11 April 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	Discussion on support of AIoT topology 1

Document for:	Discussion

1	Introduction

In this paper, we would like to discuss the issue and potential solutions to support AIoT topology 1, based on the progress made in RAN3 at the last meeting.

The following sub-topics are discussed:

- Architecture aspects

- NG interface management

- Reader selection

- Device context handling

- Inventory and command operation

2	Discussion

### Architecture aspects

RAN3 127 agreements RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIOTF in Topology 1. WA: Take Option1 (Including AIOTF information in NGAP). WA: The AIoT RAN node may only provide the AIoT radio. AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

By incorporating AIOTF information into NGAP, rather than introducing new protocols, there will be less specification work. Additionally, according to the RANP discussion and conclusion in [1], "RAN3 will continue with existing 38.413," indicating that RAN3 will not introduce new protocol. Therefore, we propose adopting Option 1 (Incorporating AIOTF information into NGAP) as a consensus.

For the indirect path, enhancements between AMF and AIOTF are necessary. For instance, AMF must forward and route AIoT-related information between gNB and AIOTF, akin to SM information. RAN3 needs to send specifications to SA2 and CT4, instructing them to define the interactions between AMF and AIOTF.

Proposal 1,  Turn the WA: Take Option1 (Including AIOTF information in NGAP)  as an agreement.

Proposal 2, Liaise with SA2/CT4 to define AMF-AIOTF interactions, treating AIoT information (e.g., inventory/command) as N2 message containers.

### NG interface management

About Interface management procedures: FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF. FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF. FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

For indirect path communication, we think the existing NG interface management procedures are used between the gNB and the AMF.

For direct path,  the main difference between direct path and indirect path is that new NG or enhanced NG setup procedure needs to be introduced for direct path.

- option 1 reusing existing NG setup but includes incompatible IEs).

- option 2 new AIoT NG setup procedure

If the existing NG setup procedure is re-used, there’re some mandatory IEs marked in yellow below not suitable for AIOTF,

Information in NG setup request

| IE/Group Name | Presence |
| --- | --- |
| Message Type | M |
| Global RAN Node ID | M |
| RAN Node Name | O |
| Supported TA List |  |
| >Supported TA Item |  |
| >>TAC | M |
| >>Broadcast PLMN List |  |
| >>>Broadcast PLMN Item |  |
| >>>>PLMN Identity | M |
| >>>>TAI Slice Support List | M |
| Default Paging DRX | M |

Information in NG setup response

| IE/Group Name | Presence |
| --- | --- |
| Message Type | M |
| AMF Name | M |
| Served GUAMI List |  |
| >Served GUAMI Item |  |
| >>GUAMI | M |
| Relative AMF Capacity | M |
| PLMN Support List |  |
| >PLMN Support Item |  |
| >>PLMN Identity | M |
| >>Slice Support List | M |

A new AIoT NG setup procedure is more suitable for AIoT operation, as the AIoT session is always initiated by AIOTF. The SCTP association between AIOTF and gNB can be preconfigured by OAM, then the AIOTF initiates the AIoT NG setup when needed.

For SCTP association, in NR system, there’re multiple associations between gNB and AMF including UE associations and non-UE associations. For topology 1 in AIoT system, it’s enough to only have non-UE association, so that multiple SCTP association is not needed between gNB and AMF.

Proposal 3, RAN3 agree to introduce a new AIoT NG setup procedures between gNB and AIOTF for direct path.

Proposal 4, RAN3 discuss whether to support multiple SCTP association between gNB and AIOTF.

### Reader selection

AIOTF may be aware of the location of reader via OAM configuration. FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support. FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT? Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area. Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list. FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

In the ongoing discussions within RAN3, it has been acknowledged that AIOTF may be aware of the location of readers via OAM configuration. However, the details of the reader's location, such as room number and GNSS information, are still under further study (FFS). Additionally, the signaling support for this information is also FFS.

We believe it is beneficial for AIOTF to be aware of reader information directly from the gNB, rather than solely relying on OAM. This approach offers several advantages: By obtaining reader information from the gNB, AIOTF can receive more accurate and real-time updates on the reader's location and capabilities. This is particularly important as the gNB may have multiple readers, each with fixed locations but varying serving areas depending on transmitting power and other radio conditions. Allowing AIOTF to access reader information from the gNB can lead to more efficient network operations. The gNB can dynamically adjust the serving area and radio capabilities (e.g., bandwidth) of each reader based on current radio conditions, optimizing overall network performance.

The signalling of reader information exchange, there are two signalling options:

- option 1, via (AIoT) NG setup or RAN configuration update.

- option 2, via dedicated reader information exchange

We slightly prefer option 2, as the reader information is common for both direct arch and indirect arch. AMF does not need to know the reader information, the design of the signalling can be the same as other AIoT operation (e.g., inventory or command).

Proposal 5, RAN3 agree to introduce a class 1 procedure for reader information exchange.

Proposal 6, the following information can be included in the reader information provided by gNB for reader selection:

- Reader ID, used to identify a reader.

- Reader location, used to identify the geographical location of a reader

- AIoT radio capabilities (frequency, bandwidth, power), used to indicate the radio capabilities of a reader.

Upon receiving an inventory request from the core network, the gNB's behavior in selecting readers is as follows:

In case of receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

In case of receiving both Area and Reader List in Inventory Request, the gNB selects all the served readers.

Proposal 7, turn the “WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list” to an agreement.

Proposal 8, Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

### Device context handling in AIoT RAN

There are different service types in AIoT service. In case of inventory-only, as there is no further communication between device and AIOTF, there’s no need to keep device context after inventory report, however, for inventory+command operation, the AIoT RAN needs to keep the device context for further communication, thus, a RAN level ID needs to be introduced in AIoT RAN to identify a specific device in RAN level.

In our view, in case of inventory+command operation, the device context for a specific device should include the following information:
- Correlation ID: A unique identifier to bind requests and responses, ensuring accurate tracking and correlation of messages.

- AS ID: The ID used for AIoT radio communication, which is discussed in RAN2.

- RAN Device ID for NGAP: This ID is used for device specific communication over NGAP, it can be the AS ID or ID provided by the device or a NGAP device ID.

The following is an example for AIoT RAN handle device context

At the initial inventory request, the AIoT CN node will indicate the service type to AIoT RAN, if the service type is “inventory+command”, and the AIoT RAN will create device context after receiving the response from AIoT device.

If there are further communication between AIoT CN and AIoT RAN, the device context will be used for these commutations, as shown from step 7 to step 10. If there’s no further communication, the AIoT CN node should indicate it to AIoT RAN, and then AIoT RAN should release the device context after the last communication.

Proposal 9, for inventory+command operation, the device context should be kept in AIoT RAN and AIoT CN node fur after inventory for further communication.

Proposal 10, RAN device ID can be introduced over NGAP to identify a specific device in command procedures.

Proposal 11, the following information should be introduced to establish and release the device context:

- introduce the service type in inventory request message, so that the gNB establish the device context in case the service type is “inventory and command” .

- introduce no further communication indication in the command request message, so that the gNB release the device context.

- introduce context released indication in the command response message, so that the AIOTF knows that the device context has been released.

### Inventory and command operation

About Inventory Procedures: WA: Introduce a NGAP Class 1 Inventory Request procedure. Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE. Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE. Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE. Introduce a NGAP Class 2 Inventory Report procedure. Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE. The above Transfer IEs are transparent to AMF in case of indirect communication. FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier. About Command Procedure: WA: Introduce a NGAP Class 1 Command Request procedure Introduce COMMAND REQUEST message, which includes Command Request Transfer IE. Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE. Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE. The above Transfer IEs are transparent to AMF in case of indirect communication. FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier. FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure? FFS: Command request procedure is a per device procedure or to be used for multiple devices? FFS: The need for a Command Report procedure? FFS: Device association between gNB and AIOTF is used in Command procedure.

Information in Inventory procedures

Based on the progress in SA2 and RAN2 discussion, we propose the inclusion of the following information in the inventory related message:

| Inventory request | Inventory response | Inventory report |
| --- | --- | --- |
| Correlation ID Reader Selection Info (O) Device Identification (O) Aggregation Information (O) Service Type (O) Resource Allocation Assistance Information (O) | Correlation ID Reader ID (O) UL Device NAS Container List (O) End indication (O) | Correlation ID Reader ID (O) UL Device NAS Container List (O) End indication (O) |

- Correlation ID: A unique ID to bind requests and responses, ensuring accurate tracking and correlation of messages.

- Reader Selection Info: This is optional and can include the target area and/or a reader list. This information will guide the gNB in selecting the appropriate readers.

- Device Identification: This information is transparent to AIoT RAN

- Aggregation Information: Time Interval for Response Aggregation: If provided by the Application Function (AF).

- Service Type: The encoding of this information depends on the device context discussion.

- Resource Allocation Assistance Information: This can include:

Approximate Number of AIoT Devices in the Target Area

Expected D2R Message Size: If available

- End Indication: As noted in SA2 TR NOTE 10, AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID.

Information in Command procedures

| Command request | Command response |
| --- | --- |
| Correlation ID RAN Device ID DL Device NAS container No further communication indication (O) | Correlation ID RAN Device ID UL Device NAS container Device context released (O) |

The following are introduction of new IEs:

- RAN Device ID: This ID is used for further communication, ensuring seamless interaction and management.

- No further communication indication, this indication is used to notify gNB to release the device context.

Proposal 12, the following information is introduced in inventory request message:

- Correlation ID

- Reader Selection Info (O)

- Device Identification (O)

- Aggregation Information (O)

- Service Type (O)

- Resource Allocation Assistance Information (O)

Proposal 13, the following information is introduced in inventory response/report message:

- Correlation ID

- Reader ID (O)

- UL Device NAS Container List (O)

- End indication (O)

Proposal 14, the following information is introduced in command request message:

-Correlation ID

- RA N Device ID

- DL Device NAS container

- No further communication indication (O)

Proposal 15, the following information is introduced in command response message:

- Correlation ID

- RAN Device ID

- UL Device NAS container

- Device context released (O)

3	Conclusion

In this contribution, we discussed the issue and potential solutions to support AIoT in topology 1 and have the following observations and proposals.

### Architecture stacks

Proposal 1,  Turn the WA: Take Option1 (Including AIOTF information in NGAP)  as an agreement.

Proposal 2, Liaise with SA2/CT4 to define AMF-AIOTF interactions, treating AIoT information (e.g., inventory/command) as N2 message containers.

### NG interface management

Proposal 3, RAN3 agree to introduce a new AIoT NG setup procedures between gNB and AIOTF for direct path.

Proposal 4, RAN3 discuss whether to support multiple SCTP association between gNB and AIOTF.

### Reader selection

Proposal 5, RAN3 agree to introduce a class 1 procedure for reader information exchange.

Proposal 6, the following information can be included in the reader information provided by gNB for reader selection:

- Reader ID, used to identify a reader.

- Reader location, used to identify the geographical location of a reader

- AIoT radio capabilities (frequency, bandwidth, power), used to indicate the radio capabilities of a reader.

Proposal 7, turn the “WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list” to an agreement.

Proposal 8, Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

### Device context handling in AIoT RAN

Proposal 9, for inventory+command operation, the device context should be kept in AIoT RAN and AIoT CN node fur after inventory proceudre for further communication.

Proposal 10, a RAN device ID can be introduced over NGAP to identify a specific device in command procedures.

Proposal 11, the following information should be introduced to establish and release the device context:

- introduce the service type in inventory request message, so that the gNB establish the device context in case the service type is “inventory and command” .

- introduce no further communication indication in the command request message, so that the gNB release the device context.

- introduce context released indication in the command response message, so that the AIOTF knows that the device context has been released.

### Inventory and command operation

Proposal 12, the following information is introduced in inventory request message:

- Correlation ID

- Reader Selection Info (O)

- Device Identification (O)

- Aggregation Information (O)

- Service Type (O)

- Resource Allocation Assistance Information (O)

Proposal 13, the following information is introduced in inventory response/report message:

- Correlation ID

- Reader ID (O)

- UL Device NAS Container List (O)

- End indication (O)

Proposal 14, the following information is introduced in command request message:

-Correlation ID

- RA N Device ID

- DL Device NAS container

- No further communication indication (O)

Proposal 15, the following information is introduced in command response message:

- Correlation ID

- RAN Device ID

- UL Device NAS container

- Device context released (O)

4	Reference

[1]	 RP-250455              Rel-19 Ambient IoT WI specifications and other issues  Huawei, HiSilicon