3GPP TSG-RAN WG3 Meeting #127bis	R3-251765

Wuhan, China, 7 – 11 April 2025

Agenda item:	16.3

Source:	Xiaomi

Title:	Discussion on location of the device

Document for:	Other

1	Introduction

In this paper, we’d like to further discuss the location of the device based on the previous progress

RAN3#127:

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIOTF in Inventory Report.

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

2	Discussion

AIOTF Knowledge of gNB ID

The mechanism varies by architecture:

| Architecture | gNB ID Acquisition |
| --- | --- |
| Direct Path | AIOTF obtains gNB ID during NG setup. |
| Indirect Path | AIOTF selects gNB via AMF/OAM, with implicit gNB awareness. The AMF is aware of which gNB is communicating with which AIOTF. The AMF may include the gNB ID in the AIoT-related messages exchanged between the AMF and AIOTF, and vice versa. |

For direct path, it seems that no additional information is required in subsequent communications, as the association is pre-established.

For the indirect path, the AMF is aware of everything and will route and include necessary information, even for routine AIoT operations, the AMF must perform routing and mapping, but it’s up to SA2 discussion.

Observation 1, for both direct path or indirect path, explicit inclusion of gNB ID in operational messages (e.g., inventory reports) is redundant, as the AIOTF inherently identifies the gNB through prior procedures or normal operation between AMF and AIoTF.

Proposal 1, there’s no need to introduce explicit gNB ID in the inventory report.

AIOTF Knowledge of Reader Location

The reader location mapping can be realized in AIOTF via:

- OAM pre-configuration

- Reader information exchange procedures

Figure 1 device location information

The possible call flows can be as below:

1. AIOTF obtains the reader information via reader information exchange or OAM.

2-4. AIOTF selects a TRP (reader) for AIoT operations.

5. Reader ID is included in AIoT response/report messages (e.g., inventory reports).

6. AIOTF maps ReaderID to pre-acquired location data.

Proposal 2, RAN3 agree to include Reader ID in the inventory report message.

3	Conclusion

Based on the above discussion, we have the following observations and proposals.

Observation 1, for both direct path or indirect path, explicit inclusion of gNB ID in operational messages (e.g., inventory reports) is redundant, as the AIOTF inherently identifies the gNB through prior procedures or normal operation between AMF and AIoTF.

Proposal 1, there’s no need to introduce explicit gNB ID in the inventory report.

Proposal 2, RAN3 agree to include Reader ID in the inventory report message.

4	Reference

[1] TR 38.769: Study on Ambient IoT Solutions.