3GPP TSG-RAN WG3 Meeting #127	R3-251767

Wuhan, China, 7th – 11st April 2025

Agenda item:	14.3

Source:	Xiaomi

Title:	(TP to TS 38.300) Support of Regenerative Payload

Document for:	Other

1	Introduction

In this contribution, we’d like to discuss NG connection handling related issue.

2	Discussion

In regenerative payload NTN, especially for the non-GEO satellite, due to the satellite movement, the TNL connectivity between the satellite and the ground segment is dynamic changing. However, this does not mean the NG connectivity between the onboard gNB and on-ground AMF has short window.

We believe that the following two cases shows that the NG connection can remain active even when there is a feeder link switch from one gateway to another:

- In case 1, as shown in figure 1, even if gNB1 loses its feeder link connection with NTN GW, if there are intersatellite links between satellites, the NG connection between gNB1 and AMF can still active.

- In case 2, as shown in figure 2, gNB1 may still connect with AMF via NTN GW2.

Figure 1

Figure 2

In theory, if there’re enough GW deployed and inter-satellite links are available, the NG connectivity can also be always on, however, when the satellite moves to one side of the earth, while the AMF is located in the other side, considering the transmission delay, it may not be necessary to keep the NG connection.

In our view, the NG connection may be always-on, it can also be removed due to unavailable the feeder link or unavailable inter-satellite link or low transmission efficiency. The disconnection time does not depend on a specific connection between on satellite and one gateway, it depends on the on-ground gateway deployment and availability of the inter-satellite link, which means the disconnection time may not last too long.

With the above understanding, we think the following should be discussed before NG removal is performed.

Node level context handling

Considering the disconnection may not last too long and the satellite may re-connect the AMF frequently. We think it would be worth to keep some node level context between gNB onboard and AMF on ground. Our proposal is that, when NG removal is triggered, the gNB on board can decide whether to keep some node level contexts between the gNB and the AMF. NG suspend/resume can be introduced as an optional feature for operators when dealing with frequently removed or re-setup of the NG connections. To minimize specification impacts, it can be implemented as an optional indicator or flag in NG removal requests. For example, if this indicator is included in the NG removal request, the AMF can store node configuration information.

UE level context handling

When a gNB onboard decides to trigger NG removal, before it sends the NG removal request, it should trigger handovers for the RRC_CONNECETED UEs, and relocate/release the contexts for the RRC_INACTIVE UEs.

Observation 1, In theory, the NG connection can be always-on by the well deployed on-ground gateways and inter-satellite links, but it’s not necessary to keep the connection all the time as the AMF may be far away from the satellite gNB.

Observation 2, the disconnection time between onboard gNB and on-ground gateway depends on the on-ground gateway deployment, availability of the inter-satellite link, and transmission latency trade-off, which means the disconnection time may not last too long.

Observation 3, it would be worth to keep the node level context if the gNB onboard will frequently re-connect to the same AMF.

Proposal 1: RAN3 agree to an indicator in the NG removal procedure to retain some node-level configurations in both gNB and AMF.

Proposal 2, before sending the NG removal request, the RRC_CONNECTED UEs can be moves to other AMF/gNB by performing handover. The context of the RRC_INACTIVE UEs can be either released or relocated to other gNBs. RAN3 agree the corresponding TP in the Annex.

3	Conclusion

Based on the discussion, we have the following observations and proposals.

Observation 1, In theory, the NG connection can be always-on by the well deployed on-ground gateways and inter-satellite links, but it’s not necessary to keep the connection all the time as the AMF may be far away from the satellite gNB.

Observation 2, the disconnection time between onboard gNB and on-ground gateway depends on the on-ground gateway deployment, availability of the inter-satellite link, and transmission latency trade-off, which means the disconnection time may not last too long.

Observation 3, it would be worth to keep the node level context if the gNB onboard will frequently re-connect to the same AMF.

Proposal 1: RAN3 agree to an indicator in the NG removal procedure to retain some node-level configurations in both gNB and AMF.

Proposal 2, before sending the NG removal request, the RRC_CONNECTED UEs can be moves to other AMF/gNB by performing handover. The context of the RRC_INACTIVE UEs can be either released or relocated to other gNBs. RAN3 agree the corresponding TP in the Annex.

4	Annex (TP to BL CR 38.300)

<<<<<<<<<<<<<<<<<<<< Change starts>>>>>>>>>>>>>>>>>>>>

### 16.14.x	NG Removal

For the case of regenerative payload, when disconnecting from an AMF is required due to e.g. the satellite movement in a LEO constellation, the gNB on board the satellite may request the removal of the NG interface by initiating the NG Removal procedure toward the AMF. Before initiating the NG removal request, the onboard gNB should initiate handovers to move the RRC_CONNECTED UE(s) associated with the NG connection to other AMF/gNB(s). Concurrently, it should either release or relocate the contexts of RRC_INACTIVE UE(s).

Editor’s Note: NG Removal procedure may also be used for WAB; how to capture this for WAB may need further discussion (pending WAB discussion).

<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>