3GPP TSG-RAN3 Meeting #127bis	R3-251788

Wuhan, China, April 07-11, 2025

Agenda Item:	11.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on UE Performance Reporting for Split Architecture

Document for:	Discussion/Decision

# Background

In RAN3#127, it was discussed whether CU-UP or DU reports the 3 UE performance metrics on UE throughput, packet delay and packet loss. There was no consensus on which node reports UE throughput and packet loss

RAN3#127 Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture. Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture. Stage 3 details of the solution are FFS.

In this paper we discuss our views on the remaining open issues on UE performance reporting in split architecture.

# Discussion

## UE Throughput

If CU-UP reports the UE throughput, the throughput is captured at PDCP layer. When reported by DU, it is captured at RLC layer. It is useful to report UE throughput at both RLC and PDCP layer if the F1-U interface between them suffers packets drops. However F1-U interface is a network interface unlike Uu interface and assumed to have no packet loss. Hence it is sufficient to report UE throughput at PDCP layer from gNB-CU-UP.

TS 28.552 has UE throughput at both RLC layer (Clause 5.1.1.3.1) and at PDCP level (Clause 5.1.1.3.7). Clause 5.1.1.3.7 can be adopted to report UE throughput from CU-UP to CU-CP at per DRB granularity

TS 28.552 ##### 5.1.1.3.7 Average DL UE buffered Throughput per DRB a) This measurement provides the average down link buffered UE throughput per DRB on NRCellCU. The DRBs are mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC. This measurement is intended for throughput per UE and bearer independent of traffic patterns and packet size. The measurement is based on Desired buffer size communicated within DDDS from DU to CU UP and is intended for services with burst duration spanning over the time interval of a couple of consequent DDDSs. For very bursty traffic with burst duration within the interval of one DDDS the measured values can be unprecise and cannot be compared across vendors. Initial buffering time in CU and on F1, meant as time interval the first PDCP SDU of the new burst is received in CU until this first part is received in DU, is excluded. The monitoring is supported also in DC scenario and in NSA option3a and 3x.

Reporting UE throughput at one of the layers RLC or PDCP is sufficient as F1 interface is a network interface and assumed to not incur additional data loss in user plane.

Report UE throughput at PDCP layer as gNB-CU-UP consolidates the feedback from gNB-DU over F1-U and can provide UE throughput at per DRB level as per TS 28.552 Clause 5.1.1.3.7 to gNB-CU-CP over E1 interface.

## UE Packet Delay

As agreed in the last RAN3 meeting, CU-UP provides a consolidated Packet delay metric across different entities in a split architecture with CU-CP, CU-UP and DU. For a split architecture with only CU and DU, DU provides the packet delay metrics to CU.

CU-UP reports Average consolidated Packet Delay to CU-CP over E1 interface as per TS 28.552 Clause 5.1.3.3.1 in a CU-CP, CU-UP and DU split architecture.

DU reports Average Packet Delay to CU over F1 interface as per TS 28.552 Clause 5.1.3.3.3 in a CU-DU split architecture.

## UE Packet Loss

TS 28.552 provides DL packet loss rate at RLC layer in clause 5.1.1.35 as shown below. DL packet loss at RLC layer will be a good metric to know the air interface packet loss which contributes to majority of the packet loss. Packet loss over F1-U interface can be considered negligible as F1-U is a network interface and assumed to have no packet loss.

TS 28.552 #### 5.1.1.35 DL Packet Loss rate on Uu a) This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface for an NR cell. The measurement is split into subcounters per PLMN ID per QoS level (mapped 5QI or QCI in EN-DC) and per supported S-NSSAI.

gNB-DU reports DL Packet Loss rate to CU-CP over F1 interface as per TS 28.552 Clause 5.1.1.35 for a split architecture

Unfortunately, TS 28.552 does not provide UL Packet loss rate at RLC layer. It is a measure of UL Packet loss over Air interface and over F1-U interface. UL Packet loss at RLC layer will be a good metric to know the air interface packet loss which contributes to majority of the packet loss. If we consider F1-U loss as negligible, then UL Packet Loss rate at PDCP is also ok. But to have an apple-to-apple comparison, RAN3 can request SA5 and RAN2 to define UL Packet Loss rate at RLC layer for AI/ML feedback purpose.

TS 28.552 ##### 5.1.3.1.1 UL PDCP SDU Loss Rate a) This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC), and subcounters per supported S-NSSAI.

RAN3 to request SA5 and RAN2 to define UL Packet Loss Rate at RLC layer (gNB-DU) for UE Performance reporting for AI/ML feedback.

## Data Collection over E1 and F1 Interface

A new Data Collection Request Response and Update message shall be introduced over E1 and F1 interface to request and report the UE performance metrics from CU-UP to CU-CP and from DU to CU-CP.

A new Data Collection Request Response and Update message shall be introduced over E1 interface to request and report the UE performance metrics from CU-UP to CU-CP.

A new Data Collection Request Response and Update message shall be introduced over F1 interface to request and report the UE performance metrics from DU to CU-CP.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Reporting UE throughput at one of the layers RLC or PDCP is sufficient as F1 interface is a network interface and assumed to not incur additional data loss in user plane.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Report UE throughput at PDCP layer as gNB-CU-UP consolidates the feedback from gNB-DU over F1-U and can provide UE throughput at per DRB level as per TS 28.552 Clause 5.1.1.3.7 to gNB-CU-CP over E1 interface.

Proposal 2:	CU-UP reports Average consolidated Packet Delay to CU-CP over E1 interface as per TS 28.552 Clause 5.1.3.3.1 in a CU-CP, CU-UP and DU split architecture.

Proposal 3:	DU reports Average Packet Delay to CU over F1 interface as per TS 28.552 Clause 5.1.3.3.3 in a CU-DU split architecture.

Proposal 4:	gNB-DU reports DL Packet Loss rate to CU-CP over F1 interface as per TS 28.552 Clause 5.1.1.35 for a split architecture

Proposal 5:	RAN3 to request SA5 and RAN2 to define UL Packet Loss Rate at RLC layer (gNB-DU) for UE Performance reporting for AI/ML feedback.

Proposal 6:	A new Data Collection Request Response and Update message shall be introduced over E1 interface to request and report the UE performance metrics from CU-UP to CU-CP.

Proposal 7:	A new Data Collection Request Response and Update message shall be introduced over F1 interface to request and report the UE performance metrics from DU to CU-CP.