3GPP TSG-RAN WG3 Meeting #127-bis	R3-251798

Wuhan, China, 7th -11th April 2025

Agenda item:	19.2

Source: 	Samsung

Title: 	Discussion on information exchange among gNBs for CLI handling

Document for:	Discussion and decision

# Introduction

This paper provides further discussion on the information exchange between gNBs for CLI handling, based on the conclusion of RAN3#127 meeting. The detailed discussion includes the open issues about the signalling design remained from last meeting, and also some stage-3 details for the agreement already achieved.

# Discussion

During the meeting of RAN3#127, RAN3 has agreed that basic signalling for exchange the SBFD time and frequency configuration between gNBs and within split architecture, for which the agreements are captured below:

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

The SBFD time and frequency configuration is encoded as container defined by RAN2.

Now that RAN3 has sent an LS to RAN2 for the specific IE design of the time and frequency information, RAN3 can wait for the progress of RAN2. When it comes to the stage 3 details in RAN3 specification, the details can be marked as FFS before we have a clear view of the specific IE in RAN2 spec.

Regarding the exchange of measurement configuration, RAN3 has confirmed that the exchange of SSB information is already supported via the exchanging of Measurement Timing Configuration in RAN3 spec, but there is still no common understanding for the exchange of NZP CSI-RS configuration, as the following open issue reflects:

For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

During the discussion of last meeting, the majority would support to introduce a New IE in Served Cell information, and the detailed content can be referred to 38.331. As a proponent of this proposal, we believe this is the most straight-forward way to handle the NZP CSI-RS configuration —— the exchange of NZP CSI-RS configuration can be directly solved in RAN3, without bothering other WGs. One argument from the opponent is that this kind of standard implementation does not align with the SSB measurement configuration, while we would insist that there is not any requirement for implementing the same IE design for the exchange of SSB configuration and NZP CSI-RS configuration. What RAN1 requires from RAN3 is to check whether the exchange of these two kinds of measurement configuration has been supported, if not, RAN3 can simply add it into the specification. We don’t really need to bother RAN2 to evaluate this issue and discuss whether to introduce any new information in their spec, especially when only three meetings are left for this release.

Proposal 1: For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition.

As for split architecture, similarly, the NZP CSI-RS configuration can be added in the Served Cell Information IE with reference to RAN2 definition, for the DU to send the NZP CSI-RS configuration to the CU.

Proposal 2: For CU-DU split architecture, to add a new IE in Served Cell Information IE with reference to RAN2 definition.

When it comes to the exchange of strongest DL beam information and CLI-mitigation request, RAN3 has the following agreements:

For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

To define CLI-mitigation request as an optional IE.

How to send the two kinds of information over network interface is still not determined, as the following open issue notes:

How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

During the offline discussion of last meeting, the majority of the companies tend to use the same message to transfer the strongest DL beam information and the CLI-mitigation request, no matter what that message is [1]. The transmission of strongest DL beam information and CLI-mitigation request are both used for the purpose of CLI handling, and we don’t see any reason to use two different messages for these two types of information. Besides, even if the two IEs are inserted in the same message, it doesn’t mean the NG-RAN node shall send them together, when to exchange information shall be decided by the NG-RAN node case by case. Anyway, there is no problem with including the two IEs in the same message as far as we can see.

Proposal 3: The strongest DL beam information and the CLI-mitigation request can be added as two optional IEs in the same message over Xn.

With regard to which message can be used for transmitting the two kinds of information, we find the interface management procedures and new procedures both workable. The main difference is that the new procedure of CLI-mitigation is a class-2 procedure and could save some signalling burden, compared to the class-1 NG-RAN node configuration update procedure. But we are open to both of the opens and can have further discussion during the meeting. If we consider the minimized interface impact in the manner of standardization, we would slightly prefer to reuse the interface management procedures.

Proposal 4: Discuss how to transfer the strongest DL beam information and CLI mitigation request between gNBs in RAN3. The interface management procedures for Xn and F1 are slightly preferred.

A TP to the BLCR of F1 is provided in the [2].

# Conclusion

In this contribution, the RAN3 impact of CLI-handling were analysed. The following proposals are provided in this paper:

Proposal 1: For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition.

Proposal 2: For CU-DU split architecture, to add a new IE in Served Cell Information IE with reference to RAN2 definition.

Proposal 3: The strongest DL beam information and the CLI-mitigation request can be added as two optional IEs in the same message over Xn.

Proposal 4: Discuss how to transfer the strongest DL beam information and CLI mitigation request between gNBs in RAN3. The interface management procedures for Xn and F1 are slightly preferred.

# References

[1] R3-250803 the summary of SoD

[2] R3-251799 TP to BLCR of TS38.473 on CLI handling