3GPP TSG-RAN WG3 Meeting #127bis	R3-251809

# Wuhan, China, 7– 11 April 2025

Agenda item:	9.2

Source: 	Samsung, Nokia, ZTE, China Unicom, China Telecom, Lenovo

Title: 	Discussion on QMC coordination for RRC segmentation in NR-DC

Document for:	Discussion and decision

Introduction

The discussion on QMC coordination for RRC segmentation in NR-DC has been discussed for several meetings. In RAN3#126, RAN3 agreed to send an LS to RAN2 in R3-247888 to check with RAN2 on their understanding related to rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 IEs in RRC spec. Before this meeting, RAN3 receives the reply LS from RAN2 in R3-251516 which provides the following information,

RAN2 thanks RAN3 for the LS. After discussion, below are RAN2 responses to Questions 1-4:

- Question 1: Can the UE receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter?

Question 2: Can the UE receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter?

RAN2 response to Q1/2: It is specified in the field descriptions as below that rrc-SegAllowedSRB4-r17 is only configured for an MCG and rrc-SegAllowedSRB5-r18 is only configured for an SCG.

rrc-SegAllowedSRB4 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB4. The field is only configured for an MCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

rrc-SegAllowedSRB5 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB5. The field is only configured for an SCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

Based on it, there are two different understandings in RAN2:

- Understanding 1: rrc-SegAllowedSRB4-r17 is only used for an MCG, and rrc-SegAllowedSRB5-r18 is only used for SCG. But there is no restriction that MN should not include rrc-SegAllowedSRB5-r18 in AppLayerMeasConfig-r17 or SN should not include rrc-SegAllowedSRB4-r17 in AppLayerMeasConfig-r17. In order words, UE can receive both rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 in a common AppLayerMeasConfig-r17 from any node.

- Understanding 2: rrc-SegAllowedSRB4-r17 is only configured in an MCG configuration and rrc-SegAllowedSRB5-r18 is only configured in an SCG configuration.

- Question 3: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled?

Question 4: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

RAN2 response to Q3/4: According to current ASN.1 design, rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 are optional fields with need code Need R. Based on it, there are two different understandings of UE behaviors in RAN2:

- Understanding 1: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, UE considers the RRC segmentation function for SRB5 is disabled. If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, UE considers the RRC segmentation function for SRB4 is disabled.

- Understanding 2: rrc-SegAllowedSRB4-r17 cannot be included in a configuration for an SCG and absence of the field in an SCG configuration means no action by the UE as the field cannot be present. Similarly, rrc-SegAllowedSRB5-r18 cannot be included in a configuration for an MCG and absence of the field in an MCG configuration means no action for the UE as the field cannot be present.

RAN2 has discussed whether to change the current design on rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 and it was not agreed as it would lead to non-backward compatible changes in RAN2 specifications. Therefore RAN2 reached below agreement:

We do not correct this in RAN2 and let RAN3 make corrections

In this contribution, we further discuss the possible corrections to this open issue.

Discussion

According to the reply LS from RAN2 in R3-251516, we can observe that there are two understandings in RAN2 on e.g whether AppLayerMeasConfig-r17 IE for MCG configuration can contain rrc-SegAllowedSRB5-r18 parameter.

However, it should be noted that, in this case, we cannot have two different understandings in our specs, otherwise the inter-operability issue will occur. For example, if an MN having Understanding 2 (which means the MN will NOT contain rrc-SegAllowedSRB5-r18 parameter in MCG configuration) tries to configure a UE having Understanding 1, even though the RRC segmentation is enabled over SRB5 by SN, upon receiving the MCG configuration without rrc-SegAllowedSRB5-r18 parameter, the UE will release the configuration on rrc-SegAllowedSRB5-r18 parameter and consider that the RRC segmentation is NOT enabled over SRB5 by SN.

Observation 1: RAN3 and RAN2 need to achieve only one common understanding from two understandings provided by RAN2, otherwise the inter-operability issue occurs.

In addition, we also observe that RAN2 has achieved an agreement on not to make corrections in RAN2, and let RAN3 do the work. As the leading group, RAN3 needs to decide which understanding to follow.

Observation 2: RAN3 needs to decide which understanding to follow, considering that RAN3 is the leading group and no correction is pursued in RAN2.

As far as we know, RAN2 has discussed the possibility to follow Understanding 2 according to a contribution in R2-2500850, wherein the Need field is proposed to be corrected from “Need R” to “Need S”. The reason to change the Need field is that introducing two separate segmentation allowed parameters (i.e. rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18) is not a normal way of RRC encoding; in other words, if we follow Understanding 2, it is enough to define only one segmentation allowed parameter instead of two separate parameters. Therefore, having two separate segmentation allowed parameters has already indicated that RRC spec does not preclude the case that one MCG/SCG configuration contains both parameters; otherwise the action taken by UE when there’s absence of such parameter needs to be Specified (i.e. Need S), which is also the motivation to propose the correction in R2-2500850.

However, according to the reply LS from RAN2, RAN2 has agreed that such correction is NOT pursued in RAN2 because of the NBC change (i.e. change Need R -> Need S); therefore, it is safer to follow the Understanding 1 because Understanding 1 will not bring NBC change to either RAN2 or RAN3.

Observation 3: If Understanding 2 is followed, it should have been enough to define only one segmentation allowed parameter instead of two separate parameters. However, currently two separate parameters are defined in RRC spec.

Observation 4: RAN2 has discussed whether and how to follow Understanding 2 according to a proposed correction, but the RAN2 correction to follow Understanding 2 is NBC so such correction is not pursued.

Observation 5: Following Understanding 1 is not expected to bring any NBC change to either RAN2 and RAN3.

As a summary, we propose to follow Understanding 1 as the common understanding.

Proposal 1: Use Understanding 1 as the common understanding: i.e. there is no restriction that MN should not include rrc-SegAllowedSRB5-r18 in AppLayerMeasConfig-r17 or SN should not include rrc-SegAllowedSRB4-r17 in AppLayerMeasConfig-r17.

As a step further, the reply LS from RAN2 also asks RAN3 to make corrections. Recalling that there were three options on the table according to the previous discussions in RAN3,

Option 1: The QoE configuring node sends the request explicitly on whether the non QoE configuring node enables the RRC segmentation, and the non QoE configuring node sends the response on whether the non QoE configuring node enables the RRC segmentation (as provided in [1]).

Option 2: The QoE configuring node sends the QMC coordination request which implicitly requests on whether the non QoE configuring node enables the RRC segmentation, and the non QoE configuring node sends the response on whether the non QoE configuring node enables the RRC segmentation (as provided in [2]).

Option 3: Either MN or SN may report to its peer node whether it enables RRC segmentation for QoE report in QoE coordination procedure, i.e. include segmentation allowed information for own SRB in QMC Coordination Request and QMC Coordination Response.

Moreover, for the explicit request in Option 1, there are further two interpretations on such explicit request,

- Option 1-1: The explicit request means the QoE configuring node provides some assistance information to the non QoE configuring node on whether the QoE report is required to be segmented based on previous knowledge. (The interpretation mentioned in R3 245679 [3])

- Option 1-2: The explicit request means the QoE configuring node needs to understand whether the non QoE configuring node enables segmentation, in order for the QoE configuring node to include the correct information on rrc-SegAllowedSRB4/5 to be configured to the UE.

It should be noted that the Option 1-1 and Option 1-2 serve for different purposes. And in our understanding, Option 1-2 is more fundamental which means that without such explicit request, the whole mechanism cannot work.

To down select the options, firstly the Option 2 can be ruled out because no company actually supports this option for now.

For the next step, we need to identify which option is better, Option 1-2 or Option 3. Then we would like to investigate the following case,

If we follow Option 1-2, a round of coordination on segmentation allowed information between MN and SN is always needed before any one of the node would like to send any QoE related configuration to the UE, even when one node only intends to modify the QoE information totally in the charge of itself. While for Option 3, the segmentation allowed information is only needed to be coordinated when one node intends to change the its own segmentation allowed status. Since we expect that the segmentation allowed status is not changed that often, and Option 3 decouples the coordination on RRC segmentation from other QoE coordination which is clearer in node behaviour, we propose to follow Option 3.

Proposal 2: Either MN or SN may report to its peer node whether it enables RRC segmentation for QoE report in QoE coordination procedure, i.e. include segmentation allowed information for own SRB in QMC Coordination Request and QMC Coordination Response.

To further investigate on whether Option 1-1 is additionally needed, according to the discussion during last several meetings, the proponent of Option 1-1 considers the QoE configuring node can be act as the ‘dealbreaker’, i.e.,

- Case 1: If the QoE configuring node found that the QoE reports were not segmented based on previous knowledge (e.g. initially only a single connectivity is established between the gNB and the UE, and there was an S-based QoE configuration for which the QoE reports were not segmented), then during the RRC segmentation coordination in case of NR-DC, the QoE configuring node can explicitly request the non QoE configuring node NOT to enable the RRC segmentation over the non QoE configuring node. And upon the reception of such request, the non QoE configuring node may not enable the RRC segmentation accordingly.

- Case 2: If the QoE configuring node found that the QoE reports were segmented based on previous knowledge, then during the RRC segmentation coordination in case of NR-DC, the QoE configuring node can explicitly request the non QoE configuring node to enable the RRC segmentation over the non QoE configuring node. And upon the reception of such request, the non QoE configuring node should try its best to enable the RRC segmentation.

Regarding the above two cases, in our understanding,

- For both Case 1&2, the pre-condition is that the QoE configuring node is able to obtain the previous knowledge on whether the QoE reports were segmented subject to a QoE configuration. And it is also claimed in [3] that only the QoE configuring node knows whether the QoE report segmentation is required or not.

However, in our understanding, both QoE configuring node and non QoE configuring node has the similar knowledge on one specific QoE configuration (i.e. QoE Reference, MCE IP address, etc.) during the initial configuration stage. So the QoE configuring node actually does not have more knowledge than the non QoE configuring node in many cases.

- If there’s chance that the QoE configuring node is able to obtain the previous knowledge, for Case 1, it is true that the non QoE configuring node may make better RRC segmentation decision by getting suggestions from the QoE configuring node. However, the benefit looks limited. From one aspect, if the non QoE configuring node for QoE configuration 1 also needs to receive the QoE reports for QoE configuration 2 which needs RRC segmentation, then the non QoE configuring node still has to enable the RRC segmentation regardless of the suggestion from the QoE configuring node. From the other aspect, there’s no harm for the non QoE configuring node to still enable the RRC segmentation for such case, since no matter whether the QoE reports will be segmented, enabling the RRC segmentation is always preferred to avoid over-sized QoE reports to be discarded at the UE side.

- If there’s chance that the QoE configuring node is able to obtain the previous knowledge, for Case 2, we don’t find any other reason why the non QoE configuring node chooses not to enable segmentation if it supports segmentation, except the case when non QoE configuring node is overloaded. In fact, if the non QoE configuring node is overloaded, the information on whether the RRC segmentation is required or not from the QoE configuring node does not help much.

Regarding the case raised in [3], which is captured as follows,

- The QoE-configuring node realizes that the QoE reports are large enough to require segmentation (note: this can happen after the node receives one or more reports and decides to request reporting leg switch, but it can also happen before reporting has started, e.g., if the node is unable the reception and forwarding of the reports to the MCE).

- The QoE-configuring node sends the QMC coordination request to the QoE-non-configuring node, asking for reporting leg switch. It does not indicate whether segmentation is needed (because Option 2 does not require the QoE-configuring node to explicitly indicate that it is needed).

- The QoE-non-configuring node determines that it has no possibility of, or is not willing to, enable segmentation, and it indicates it in the response. However, this is a blind, unsolicited response because, as per Option 2, the QoE-non-configuring node does not know whether segmentation is required at all.

- Given that the QoE-non-configuring node cannot/will noy provide, or does not support, report segmentation, the QoE-configuring node decides not to change the reporting leg (because, otherwise, the reports would be lost).

- Meanwhile, the QoE-non-configuring node is unaware about how to proceed, where the following ambiguity occurs:

  - The QoE-non-configuring node may assume that the segmentation is not needed, and that the reporting leg switch will take place and it unnecessarily allocates the resources for that, OR

  - The QoE-non-configuring node may assume that the reporting leg switch will not take place, since it indicated that it will not enable segmentation.

- Given the above ambiguity, the QoE-configuring node either:

  - Needs to execute another round of coordination and indicate to the QoE-non-configuring node that reporting leg will not be switched (e.g., the MN sets the QoE Reporting Path Request IE codepoint value to “srb4”).

    - Problems:

      - Reconfiguring a configuration that was never applied is logically wrong and is a violation of the use of the coordination procedure.

      - This reconfiguration requires an additional round of signalling.

  - Alternatively, the QoE-configuring node does nothing to resolve the confusion and ambiguity at the QoE-non-configuring node.

Regarding 5), when the non QoE configuring node sends QMC Coordination Response, if the non QoE configuring node has responded that the reporting leg is changed to non QoE configuring node, of course the non QoE configuring node will understand that the reporting leg switch will take place. In addition, as explained earlier, we don’t find any other reason why the non QoE configuring node chooses not to enable segmentation if it supports segmentation, except the case when non QoE configuring node is overloaded. If the non QoE configuring node supports segmentation and is not overloaded, it is a bad implementation for the non QoE configuring node to not enable the segmentation.

Regarding 4), after the QoE configuring node receives QMC Coordination Response which (implicitly) indicates not to enable the segmentation, the QoE configuring node of course perceives the reporting leg will be switched and the non QoE configuring node already gets ready. So if the QoE configuring node prefer to switch the reporting leg back, the only way it can do is to execute another round of coordination rather than doing nothing. Note that when the QoE configuring node executes another round of coordination, the update of the QoE configuration has not been transmitted to the UE. So we fail to see any violation to the use of the coordination procedure. And we fail to see why there’s ambiguity.

Observation 7: One of the pros identified to use Option 1-1 depends on the previous knowledge the QoE configuring node had. However, it is unclear whether QoE configuring node can obtain accurate previous knowledge.

Observation 8: Even for cases when the QoE configuring node can obtain the previous knowledge, Option 1-1 only brings limited benefit for a specific case.

Based on the above analyses and observations, we propose not to pursue Option 1-1 as an additional enhancement.

Proposal 3: Agree the CRs in R3-252158 and R3-251810.

Conclusion

In this paper, we provides further considerations for QoE on support of NR-DC. The following observations are provided,

Observation 1: RAN3 and RAN2 need to achieve only one common understanding from two understandings provided by RAN2, otherwise the inter-operability issue occurs.

Observation 2: RAN3 needs to decide which understanding to follow, considering that RAN3 is the leading group and no correction is pursued in RAN2.

Observation 3: If Understanding 2 is followed, it should have been enough to define only one segmentation allowed parameter instead of two separate parameters. However, currently two separate parameters are defined in RRC spec.

Observation 4: RAN2 has discussed whether and how to follow Understanding 2 according to a proposed correction, but the RAN2 correction to follow Understanding 2 is NBC so such correction is not pursued.

Observation 5: Following Understanding 1 is not expected to bring any NBC change to either RAN2 and RAN3.

Proposal 1: Use Understanding 1 as the common understanding: i.e. there is no restriction that MN should not include rrc-SegAllowedSRB5-r18 in AppLayerMeasConfig-r17 or SN should not include rrc-SegAllowedSRB4-r17 in AppLayerMeasConfig-r17.

Proposal 2: Either MN or SN may report to its peer node whether it enables RRC segmentation for QoE report in QoE coordination procedure, i.e. include segmentation allowed information for own SRB in QMC Coordination Request and QMC Coordination Response.

Observation 7: One of the pros identified to use Option 1-1 depends on the previous knowledge the QoE configuring node had. However, it is unclear whether QoE configuring node can obtain accurate previous knowledge.

Observation 8: Even for cases when the QoE configuring node can obtain the previous knowledge, Option 1-1 only brings limited benefit for a specific case.

Proposal 3: Agree the CRs in R3-252158 and R3-251810.

References

[1] R3-244550

[2] R3-244551

[3] R3-245679