3GPP TSG-RAN WG3 Meeting #127bis	R3-251814

# Wuhan, China, 7– 11 April 2025

Agenda item:	16.3

Source: 	Samsung

Title: 	Discussion on Location report for AIoT

Document for:	Discussion and decision

Introduction

Last meeting discussed the location report for AIoT, and the following agreements are captured,

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

In this contribution, we further discuss these open issues.

Discussion

The location report for AIoT was primarily discussed last meeting.

Recall that in TR 38.769, the primary message follow for AIoT inventory and command operations have been agreed and captured as follows,

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

Figure 6.5.2.2-1: Message flow for A-IoT Command in Topology 1

We can observe from the above message flow that, the device location reporting is provided from AIoT RAN to AIoTF in the Inventory Report message and the Command Response message.

In addition, in TR 38.769, for T1 the following text has been captured from the perspective of locating AIoT devices,

For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages.

The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded.

NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion.

NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion.

NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.

As observed by the above quoted text, the first question to check is whether an AIoT RAN node can have multiple Readers. In our understanding, there’s no doubt that an AIoT RAN node can have multiple readers because of the limited coverage for each reader.

The next question, as indicated by NOTE 1, whether multiple Reader IDs can be included in the Inventory Report/Command Response message. In our understanding, even though an AIoT device is located in an overlapping coverage area of multiple readers, the AIoT device only needs to reply once during inventory procedure or command procedure; otherwise there will be more chances for collision during potential random access. Under such case, the AIoT RAN node is only able to report one Reader ID to the AIoTF. And till now we see no clear advantage for the AIoT device to reply multiple times to the same inventory procedure or command procedure. Although whether the AIoT device can only reply once or multiple times depends on RAN2 discussion, RAN3 can firstly assume that at most one Reader ID is included in the Inventory Report/Command Response message.

Proposal 1: RAN3 assumes that at most one Reader ID for a device is included in the Inventory Report/Command Response message.

The next open issue is,

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

We see no strong need to send Global gNB ID additionally in Inventory Report, so only Reader index is enough.

In addition, for the Reader index, the current TRP ID IE as defined in TS 38.455 can be reused.

Proposal 2: Only Reader index in sent in Inventory Report.

Proposal 3: The Reader ID included in the Inventory Report/Command Response message can be TRP ID which uniquely identifies a TRP/Reader within an AIoT RAN node.

Conclusion

In this paper, we provide primary discussion on location reporting for Ambient IoT. The following proposals are provided,

Proposal 1: RAN3 assumes that at most one Reader ID for a device is included in the Inventory Report/Command Response message.

Proposal 2: Only Reader index in sent in Inventory Report.

Proposal 3: The Reader ID included in the Inventory Report/Command Response message can be TRP ID which uniquely identifies a TRP/Reader within an AIoT RAN node.

Reference