3GPP TSG-RAN WG3 Meeting #127bis	R3-251848

# Wuhan, China, 7th - 11th April, 2025

Agenda Item:	17.3

Source:	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

Title:	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Other

# Introduction

The revised Work Item Description (WID) for “Enhancements of network energy savings for NR” [1] in Rel-19 states the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on the impact on the Xn interface.

In particular, we discuss the main principles of the inter NG-RAN node signaling to enable on-demand SIB1 (OD-SIB1) transmission, considering non-split architecture.

# General background for discussion

## General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of its NES Cell to gNB2 in step 1. In step 2, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. In step 3, when needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, receives SIB1 from the NES Cell in step 4.

## Previous agreements

In relation to the impact on the Xn interface, the following agreements have been captured:

- RAN3#125 meeting [2]:

- NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

- RAN3#125-bis meeting [3]:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- RAN3#126 meeting [4]:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

- RAN3#127 meeting [5]:

It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

These agreements (except for the last, highlighted one) can be summarized as follows:

- New class 1 Xn procedure for signaling the UL WUS configuration from NES-Cell gNB to Cell-A gNB

  - The initiating message carries the UL WUS configuration for the NES Cell

  - The response message (successful or unsuccessful outcome) from Cell-A gNB indicates if the UL WUS configuration is to be broadcasted in the Cell A or not.

  - Cell-A gNB stores the UL WUS configuration for the NES Cell (if accepting to broadcast it)

  - The initiating/request message has the choice “start” to trigger Cell-A gNB to broadcast the UL WUS configuration in SIBx

  - After accepting the request, Cell-A gNB broadcasts the UL WUS configuration in SIBx at the first opportunity

- New class 2 Xn procedure for signaling the interruption of UL WUS configuration broadcast by Cell-A gNB

  - Cell-A gNB signals to NES-Cell gNB that it stops the UL WUS configuration broadcast in SIBx

In the following, we address some open issues relating to both the new class 1 and the new class 2 procedures, as well as the implementation aspects of the agreement on “NES-Cell gNB can ´stop´ the UL WUS configuration broadcast in Cell A”.

# Discussion on the new procedures

## General aspects

Before discussing the details of the new procedures, we would like to make a few general comments that apply to the work we are doing in relation to OD-SIB1.

For the design of the new procedures, it is essential to prioritize simplicity to ensure robustness and minimize the potential for errors. Several contributions in the past meetings have proposed a multiplicity of options for the new procedures. While they may offer flexibility, they also introduce certain error cases and complicate the design process, the benefit is not clear. By initially focusing on core functionality, we can establish a solid foundation that is both reliable and easy to implement. With such foundation, enhancements can be introduced in future releases.

RAN3 to agree to design simple and robust procedures for enabling UL WUS configuration provisioning.

In relation to the energy-saving benefits of OD-SIB1, we would like to emphasize that these stem less from the reduced energy expenditure associated with not transmitting SIB1 periodically, but rather from the increased and extended opportunities it provides for putting the hardware into deeper sleep modes. However, these energy savings are contingent upon the absence of concurrent processes that might prevent the hardware from entering deeper sleep modes.

A cell operating in OD-SIB1 mode that continues to transmit or receive signals every few frames will not experience a meaningful reduction in energy consumption compared to transmitting SIB1 periodically. Therefore, we should focus on scenarios characterized by extended periods of low traffic, where only a few UEs sporadically and briefly connect to the network. By targeting these low-traffic scenarios, we can provide a robust framework for the NG-RAN to use the OD-SIB1 feature where it is most beneficial.

A consequence of the previous proposal is that a NES Cell would only enter and exit OD-SIB1 mode a few times per day. The OD-SIB1 mode would be active during the night and other low traffic periods, and the NES Cell may exit this mode if a certain, sustained increase in traffic appears. In summary, we expect that the OD-SIB1 mode would be semi-static, and either active or inactive for very long periods of time.

In the relevant scenarios, the NES Cell is expected to be either in OD-SIB1 mode or periodic SIB1 transmission for very long periods of time.

Finally, we would like to bring to RAN3 attention that, according to RAN1 discussion, the size of the UL WUS configuration is around 30 octets (240 bits) [6][7]. This is a very small amount of data to signal over RAN3 (mostly wired) interfaces.

The size of the UL WUS configuration is around 30 octets (240 bits) according to RAN1. This is a negligible amount of data to signal over RAN3 interfaces.

RAN3 to agree the UL WUS configuration is semi static and due to the small size of UL WUS Configuration, it is no issue to always send it when needed.

## Design of the new Class 1 procedure as start/stop request

### Agreed structure of the new Class 1 procedure

As already agreed, the new Class 1 procedure over Xn will carry the UL WUS configuration information. The UL WUS configuration information is exchanged over Xn only when the NES-Cell gNB intends to put the NES Cell into OD-SIB1 operation and cells of another gNB (Cell A gNB) need to assist..

### No need to have “pause/resume” the UL WUS configuration provisioning

Several contributions in the past meetings have proposed the possibility that the NES-Cell gNB can signal the Cell-A gNB to pause (or resume) the provisioning of the UL WUS configuration in SIBx. We believe that this functionality is not needed as we explain below.

A functionality to “pause/resume” the provisioning of the UL WUS configuration implicitly assumes that the periods when the provisioning is paused are relatively short. It does not make sense to pause the provisioning, e.g., for several hours. This has several disadvantages:

- The Cell-A gNB would have to store in memory the context related to the Class 1 procedure and the UL WUS configuration itself, even when OD-SIB1 mode is off for long periods, which is a burden.

- A Cell-A gNB itself could not remove the said context from memory, and if it would, a resume request from the NES gNB would lead to an error.

- Since the UL WUS configuration may potentially change, the NES gNB would need to remember which version of the UL WUS configuration a Cell-A gNB has stored.

Allowing the new Class 1 procedure to “pause” the broadcasting of the UL WUS configuration for long periods of time results in multiple complications.

As a result, it stands to reason that one could only safely pause the provisioning of the UL WUS configuration for a few seconds, or a few minutes at most. However, as stated in Observation 1, the NES Cell is expected to be either in OD-SIB1 mode or in periodic SIB1 transmission mode for very long periods of time in the relevant scenarios. There is ideally no need to “pause/resume” the provisioning.

Besides, as stated in Observation 2, the size of the UL WUS configuration is very small in RAN3 terms. Even if the NES Cell enters and exits the OD-SIB1 mode several times, the added overhead of signaling the UL WUS configuration over the Xn interface is almost negligible.

Therefore, there is no strong motivation to have the option to “pause” the provisioning of the UL WUS configuration from the point of view of interface overhead.

There is not sufficient motivation to introduce the option to “pause/resume” the provisioning of the UL WUS configuration from the point of view of interface overhead.

In the contributions that proposed having the “pause/resume” functionality, t

•	A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

here are two arguments in favor of it. First, it is argued that when a NES-Cell gNB receives the UL WUS from a UE and starts provisioning the OD-SIB1 in response, it should pause the UL WUS configuration provisioning in the Cell A. This is not necessary since RAN2 agreed during the RAN2#127bis meeting that:

According to RAN2, Cell-A gNB can broadcast the UL WUS configuration of a NES Cell independently of the state of the NES Cell (i.e., both in OD-SIB1 mode or periodic SIB1 mode).

A second argument in favor of this functionality is that, by pausing the UL WUS configuration provisioning in the Cell A, the energy consumption of the Cell-A gNB can be reduced. However, we recall that the basic scenario studied here assumes that the NES Cell is a capacity “booster” cell which is completely immersed in a larger coverage Cell A. Given that Cell A is providing coverage, it may be unlikely that the Cell-A gNB can significantly reduce its energy consumption by not provisioning the UL WUS configuration. Even if the NES-Cell’s UL WUS configuration is the only one in the SIBx of Cell A, which means that the SIBx might not be transmitted if the UL WUS configuration is removed, this might not lead to considerably longer new sleep opportunities. As stated in the objectives of the WID:

-	Cell A: A cell that is periodically transmitting at least its own SIB1

Given that the Cell A is most likely a coverage cell, and it is periodically transmitting its own SIB1, it is unlikely that pausing the provisioning of the UL WUS configuration can result in a meaningful reduction of the energy consumption of the Cell-A gNB.

It is unlikely that pausing the provisioning of the UL WUS configuration would result in a meaningful reduction of the energy consumption of the Cell-A gNB.

Our analysis reveals that there is no motivation or gain to introduce the option to “pause/resume” the provisioning of the UL WUS configuration from any point of view, such as, the relevant scenarios, the interface overhead, RAN2 requirements, or Cell-A gNB energy consumption.

RAN3 to agree not to introduce a “pause/resume” or similar functionality in the new Class 1 procedure for UL WUS configuration provisioning.

As a consequence of Proposal 3, we further propose that the initiating (i.e., request) message for the new Class 1 procedure always carries the UL WUS configuration whenever the NES-Cell gNB requests the Cell-A gNB to start broadcasting the UL WUS configuration information in its SIBx. Therefore, the codepoint “start with UL WUS configuration” should be renamed “start” to reflect this behavior.

RAN3 to agree that the codepoint “start with UL WUS configuration” is renamed “start”, and that the UL WUS configuration information is always included in the “request” message whenever this codepoint is used.

### New codepoint to “stop” the UL WUS configuration provisioning

In the last RAN3#127 meeting, it was agreed that NES-Cell gNB can “stop” Cell A to broadcast UL WUS configuration. However, there was no agreement with respect to which procedure to use to signal the “stop”, i.e., whether to use the new Class 1 or the new Class 2 procedure.

According to the chair’s notes during RAN3#126 meeting, RAN3 agreed that:

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The previous agreement clearly states that the new Class 2 procedure is used by the Cell-A gNB, i.e., the Cell-A gNB is the NG-RAN node that transmits the message defined in the procedure; in this procedure, the NES-Cell gNB receives the information signaled by the Cell-A gNB. The roles of the Cell-A gNB and the NES-Cell gNB are very different in the context of OD-SIB1, and thus it seems inappropriate to reuse this procedure for the NES-Cell gNB to signal information to the Cell-A gNB, effectively reversing their roles.

As outlined in Proposal 1, our focus should be on designing simple and robust procedures. Allowing the same procedure to be used by different NG-RAN nodes with distinct roles contradicts this principle.

Furthermore, it is already agreed that the new Class 1 procedure is initiated by the NES-Cell gNB, i.e., the NES-Cell gNB is the NG-RAN node that transmits the initiating message defined in the procedure. Furthermore, it has already been agreed that the “request” message in this procedure will have multiple codepoints, with the first one being “start with UL WUS configuration” (FFS on the codepoint name). Hence, it is a natural consequence to include a separate "stop" codepoint to allow the NES-Cell gNB to stop the UL WUS configuration broadcast in Cell A.

RAN3 to agree to introduce a new choice to “stop”  the Class 1 procedure to enable the NES-Cell gNB to terminate the UL WUS configuration broadcast in Cell A.

### Further discussion related to the new Class 1 procedure

In summary, we propose to design the procedure as a simple start/stop request, sent by the NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The Cell-A gNB can accept or reject the “start” request. The Cell-A gNB cannot reject the “stop” request, which is similar to, e.g., the PDU session resource release procedure (where the “release” will never fail).

RAN3 to agree to the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

Another detail to consider is that, since the Cell-A gNB can have many cells and cannot judge itself which of its cell(s) are supposed to act as Cell A, as discussed in [8], the NES-Cell gNB shall further indicate in its “start” request to the Cell-A gNB which cell(s) of the Cell-A gNB should assist in providing the UL WUS configuration.

RAN3 to agree that, in the new Class 1 procedure, the NES-Cell gNB indicates to the Cell-A gNB which cell(s) of the Cell-A gNB should act as Cell A (assisting in providing the UL WUS configuration).

We believe, it is sufficient and most appropriate, that the Cell-A gNB responds with “accept” if, and only if, it can provide the UL WUS configuration in all the cells requested (by the NES-Cell gNB) to be Cell A, and “reject” otherwise. The Cell-A gNB cannot judge if broadcasting the UL WUS configuration in a subset of the requested cells will provide good Cell-A coverage for the NES Cell to be in OD-SIB1 mode; thus, the procedure should fail in this situation.

The Cell-A gNB may optionally inform the NES-Cell gNB which cells were responsible for the procedure failing. In this way, the NES-Cell gNB may learn about the unavailability of the failed cells and, potentially, may be able to find a different set of Cell-A cells to assist the NES Cell.

RAN3 to agree that the new Class 1 procedure fails if the Cell-A gNB cannot provide the UL WUS configuration in all the requested cells. The Cell-A gNB may optionally indicate in the failure message which cells were responsible for the procedure failing.

## Design of the new Class 2 procedure to signal the feedback regarding the UL WUS configuration provisioning status

### Changes in Cell-A gNB UL WUS configuration provisioning status

It is crucial that the Cell-A gNB provides feedback to the NES-Cell gNB about the Cell A’s provisioning status. This is to ensure that the NES Cell is still accessible for idle/inactive UEs when the NES Cell is in (or about to enter) OD-SIB1 operation mode. But it is up to the NES-Cell gNB to determine when the NES Cell actually goes into OD-SIB1 operation mode.

Indeed, RAN3 has already agreed to introduce a new Class 2 XnAP procedure so the Cell-A gNB can provide feedback to the NES-Cell gNB that it stops the UL WUS configuration provisioning in its SIBx. The case of a Cell-A gNB providing feedback to the NES-Cell gNB that it starts the UL WUS configuration provisioning requires further discussion.

As specified in TS 38.331 (see IE DownlinkConfigCommonSIB), the SI modification period, expressed in number of radio frames, is: m = modificationPeriodCoeff * defaultPagingCycle. The modificationPeriodCoeff can take values between 2 and 16, and the defaultPagingCycle can take values between 32 and 256 radio frames. This means that the SI modification period m can be up to 4096 radio frames (≈ 41 seconds), and it should not be optimized only for the sake of OD-SIB1 transmission. For the defaultPagingCycle, for example, large values are common – they improve the UE energy efficiency, and the configuration of large values should therefore definitely be assumed.

The modification period for system information can be up to 41 seconds.

The new Class 1 procedure for UL WUS configuration provision should not be kept open for long, certainly not as long as 41 seconds or more (or until the SI update has reached all UEs in Cell A and the UL WUS configuration provisioning in Cell A is ensured). To avoid unfavorable protocol design, the response to the UL WUS configuration provision request should be sent promptly from the Cell-A gNB to the NES-Cell gNB, indicating that the Cell-A gNB will perform (or not) the UL WUS configuration provisioning.

The response to a request for an UL WUS configuration provisioning should be signaled promptly by the Cell-A gNB to the NES-Cell gNB in the Class 1 procedure.

The NES Cell may not go to OD-SIB1 mode immediately after a positive response in the Class 1 UL WUS configuration provision request procedure. As explained above, up to 41 seconds can pass until the SI has been updated by the Cell-A gNB. Without knowing that the broadcasting of the UL WUS configuration has been started by at least one Cell-A gNB, it is not ensured that the NES Cell can continuously serve UEs.

The NES Cell may not go into OD-SIB1 mode without knowing that the UL WUS configuration has been broadcasted by at least one Cell-A gNB.

Therefore, the agreed new Class 2 procedure could be used by the Cell-A gNB to signal any changes in its UL WUS configuration provisioning status to the NES-Cell gNB. That means, the new Class 2 procedure could also be used by the Cell-A gNB to signal that it has started the UL WUS configuration broadcasting.

RAN3 to agree that the new Class 2 procedure is used by the Cell-A gNB to signal any changes in its UL WUS configuration provisioning status to the NES-Cell gNB, including that it has started the UL WUS configuration broadcasting.

### Further discussion related to Cell-A gNB stopping the broadcasting

Another point regarding the new Class 2 procedure relates to the highlighted part in the agreement below:

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

It is of paramount importance that there is no significant time gap between the moment the Cell-A gNB stops broadcasting the UL WUS configuration in its SIBx and the moment the NES Cell reverts to periodic SIB1 transmission (if needed); otherwise, the NES Cell might not be accessible. For this reason, the Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting. This will give the NES-Cell gNB enough time to revert the NES Cell to periodic SIB1 broadcasting if needed.

RAN3 to clarify that Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting.

A final point to consider is the consequence of Cell-A gNB signaling that it stops the broadcasting of the UL WUS configuration in its SIBx. Specifically, does this action permanently terminate the UL WUS configuration provisioning, or does it merely pause it temporarily?

In our view, and in line with Proposal 1, NES-Cell gNB is the only NG-RAN node that can permanently terminate the UL WUS configuration provisioning. Given that NES-Cell gNB signaled the request for UL WUS configuration provisioning to the Cell-A gNB, the cleanest approach is to let the NES-Cell gNB terminate the provisioning. If both NES-Cell gNB and Cell-A gNB can terminate the provisioning, a race condition can occur, which would force us to evaluate possible errors.

To summarize, NES-Cell gNB should be in control of terminating the provisioning, not Cell-A gNB. The signaling of the new Class 2 procedure by Cell-A gNB should only temporarily pause the UL WUS configuration provisioning. In line with Proposal 9, Cell-A gNB may later use the Class 2 procedure to signal that the provisioning is resumed.

RAN3 to clarify that the Class 2 procedure (to signal that Cell-A gNB stops the UL WUS configuration provisioning) only temporarily pauses the UL WUS configuration provisioning.

After Cell-A gNB has signaled that it is stopping the UL WUS configuration broadcasting using the new Class 2 procedure, it would be up to implementation if and when NES-Cell gNB uses the Class 1 procedure to terminate the UL WUS configuration provisioning, e.g., if Cell-A gNB does not resume it.

NES-Cell gNB may terminate the UL WUS configuration provisioning after Cell-A gNB has stopped it.

## Outline of the signaling flow for the two new procedures

We now summarize the signaling steps and the interaction of the two XnAP procedures described above. Figure 2 shows how the procedures can be used by a NES-Cell gNB (gNB1) for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission. The first few steps are as follows:

- In a step 1, gNB1 generates an UL WUS configuration.

- In a step 2, gNB1 sends to gNB2 an UL WUS configuration provision request message comprising the UL WUS configuration and an indication of cell(s) of gNB2 in which gNB1 requests the UL WUS configuration to be provided. In response, gNB1 receives from gNB2 an UL WUS configuration provision response message with an accept/reject indication for the request.

- In case of accept, gNB2 starts providing the UL WUS configuration in requested cell(s) in a step 3, however subject to the SI modification period, as discussed before.

- In a step 4, gNB2 sends to gNB1 an UL WUS configuration provision status message indicating that it indeed has started providing the UL WUS configuration in the requested cell(s).

- In a step 5, gNB1 starts operating the NES Cell in OD-SIB1 transmission mode.

Figure 2: An example of how new procedures can be used for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission.

If the UL WUS configuration provisioning at gNB2 is temporarily interrupted at some point, as in step 6 in Figure 2, gNB2 sends to gNB1 an UL WUS configuration provision status message indicating that it is not providing the UL WUS configuration in the requested cell(s). After receiving such a message, in step 7, gNB1 may switch the NES Cell back to periodic SIB1 transmission mode, e.g., if the loss of Cell A(s) at gNB2 means lack of Cell-A coverage for the NES Cell (meaning no Cell-A redundancy).

Figure 3 shows how the proposed procedures can be used to deactivate OD-SIB1 transmission and revert the NES Cell to periodic SIB1 transmission (until further notice). If gNB1 decides to stop using OD-SIB1 transmission in the NES Cell for a longer period, it can inform gNB2 that the provisioning of the UL WUS configuration is no longer needed. Thus, it can send to gNB2 an UL WUS configuration provision request message with a corresponding indication, to relieve gNB2 from the additional provisioning of the UL WUS configuration to UEs at times when not needed. Unnecessary provisioning of an UL WUS configuration means both an unnecessary allocation of radio resources as well as a continuous increase of energy consumption by Cell-A gNB.

Figure 3: An example of how new procedures can be used to deactivate OD-SIB1 transmission and to revert to periodic SIB1 transmission (until further notice).

RAN3 to agree to capture in a Stage-2 specification the signaling flow for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission as depicted in Figure 2.

RAN3 to agree to capture in a Stage-2 specification the signaling flow for deactivating OD-SIB1 transmission and reverting to periodic SIB1 transmission as depicted in Figure 3.

A TP reflecting the above proposals is provided in Appendix A.

RAN3 to agree on the TP for BL CR for TS 38.423 provided in Appendix A.

# Conclusion

Based on the discussion in the previous sections we make the following observations:

Observation 1	In the relevant scenarios, the NES Cell is expected to be either in OD-SIB1 mode or periodic SIB1 transmission for very long periods of time.

Observation 2	The size of the UL WUS configuration is around 30 octets (240 bits) according to RAN1. This is a negligible amount of data to signal over RAN3 interfaces.

Observation 3	Allowing the new Class 1 procedure to “pause” the broadcasting of the UL WUS configuration for long periods of time results in multiple complications.

Observation 4	There is not sufficient motivation to introduce the option to “pause/resume” the provisioning of the UL WUS configuration from the point of view of interface overhead.

Observation 5	According to RAN2, Cell-A gNB can broadcast the UL WUS configuration of a NES Cell independently of the state of the NES Cell (i.e., both in OD-SIB1 mode or periodic SIB1 mode).

Observation 6	It is unlikely that pausing the provisioning of the UL WUS configuration would result in a meaningful reduction of the energy consumption of the Cell-A gNB.

Observation 7	The modification period for system information can be up to 41 seconds.

Observation 8	The response to a request for an UL WUS configuration provisioning should be signaled promptly by the Cell-A gNB to the NES-Cell gNB in the Class 1 procedure.

Observation 9	The NES Cell may not go into OD-SIB1 mode without knowing that the UL WUS configuration has been broadcasted by at least one Cell-A gNB.

Observation 10	NES-Cell gNB may terminate the UL WUS configuration provisioning after Cell-A gNB has stopped it.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN3 to agree to design simple and robust procedures for enabling UL WUS configuration provisioning.

Proposal 2	RAN3 to agree the UL WUS configuration is semi static and due to the small size of UL WUS Configuration, it is no issue to always send it when needed.

Proposal 3	RAN3 to agree not to introduce a “pause/resume” or similar functionality in the new Class 1 procedure for UL WUS configuration provisioning.

Proposal 4	RAN3 to agree that the codepoint “start with UL WUS configuration” is renamed “start”, and that the UL WUS configuration information is always included in the “request” message whenever this codepoint is used.

Proposal 5	RAN3 to agree to introduce a new choice to “stop”  the Class 1 procedure to enable the NES-Cell gNB to terminate the UL WUS configuration broadcast in Cell A.

Proposal 6	RAN3 to agree to the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

Proposal 7	RAN3 to agree that, in the new Class 1 procedure, the NES-Cell gNB indicates to the Cell-A gNB which cell(s) of the Cell-A gNB should act as Cell A (assisting in providing the UL WUS configuration).

Proposal 8	RAN3 to agree that the new Class 1 procedure fails if the Cell-A gNB cannot provide the UL WUS configuration in all the requested cells. The Cell-A gNB may optionally indicate in the failure message which cells were responsible for the procedure failing.

Proposal 9	RAN3 to agree that the new Class 2 procedure is used by the Cell-A gNB to signal any changes in its UL WUS configuration provisioning status to the NES-Cell gNB, including that it has started the UL WUS configuration broadcasting.

Proposal 10	RAN3 to clarify that Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting.

Proposal 11	RAN3 to clarify that the Class 2 procedure (to signal that Cell-A gNB stops the UL WUS configuration provisioning) only temporarily pauses the UL WUS configuration provisioning.

Proposal 12	RAN3 to agree to capture in a Stage-2 specification the signaling flow for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission as depicted in Figure 2.

Proposal 13	RAN3 to agree to capture in a Stage-2 specification the signaling flow for deactivating OD-SIB1 transmission and reverting to periodic SIB1 transmission as depicted in Figure 3.

Proposal 14	RAN3 to agree on the TP for BL CR for TS 38.423 provided in Appendix A.

# References

RP-242354, Revised WID: Enhancements of network energy savings for NR

R3-244708, Summary of offline disc at RAN3#125, Ericsson (Moderator)

R3-245737, Summary of offline disc at RAN3#125-bis, Ericsson (Moderator)

R3-247806, Summary of offline disc at RAN3#126, Ericsson (Moderator)

R3-250801, Summary of offline disc at RAN3#127, Ericsson (Moderator)

R1-2407057, Study of on-demand SIB1 for UEs in idle/inactive mode for NES, Ericsson

R1-2407103, On-demand SIB1 for NES, Fraunhofer IIS et al.

R3-245498, Xn impact of On-demand SIB1 for UEs in idle/inactive mode, Ericsson

Text Proposal for BL CR for TS 38.423

*************** Start of TP ***************

Editor’s Note: The procedure, message and the IE names are FFS

Editor’s Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

### 8.x.x	UL WUS Configuration Provision (FFS)

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to an NG-RAN node2  and to request NG-RAN node2 to transmit the provided UL WUS configuration information in one or more of its cells.

The procedure is also used to enable NG-RAN node1 to request NG-RAN node2 to discontinue transmission of the provided UL WUS configuration information.

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message (Naming FFS) to the peer NG-RAN node2 to start the provisioning or to stop the provisioning of UL WUS configuration information in one or more cells of the NG-RAN node2. Upon receipt of the message:

- If the Request Type IE is set to “start” , the NG-RAN node2 shall, if supported, store the included UL WUS configuration information, start the broadcast of the UL WUS configuration information in SIBx (SIB naming FFS) of the cells indicated in the Assisting Cells List (Naming FFS) IE, and reply with the UL WUS CONFIGURATION PROVISION RESPONSE (Naming FFS) message.

- If the Request Type IE is set to “stop”, the NG-RAN node2 shall remove the stored UL WUS configuration information associated with the indicated provisioning, stop the broadcast of the UL WUS configuration information in SIBx of the previously admitted cells, and reply with the UL WUS CONFIGURATION PROVISION RESPONSE (Naming FFS) message.

If the Request Type IE is set to "start" in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the UL WUS Configuration (Naming FFS) IE and the Assisting Cells List (Naming FFS) IE shall be included.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot broadcast the included UL WUS configuration information in SIBx (SIB naming FFS) of the cells indicated in the Assisting Cells List (Naming FFS) IE, or a failure occurs during UL WUS Configuration Provision (Naming FFS), the NG-RAN node2 shall send UL WUS CONFIGURATION PROVISION FAILURE (Naming FFS) message. The message shall contain the Cause IE with an appropriate value. The NG-RAN node2 may include the Assisting Cells Failed List IE in the UL WUS CONFIGURATION PROVISION FAILURE message with the list of cells which could not provide the UL WUS configuration information.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Transmission Status Update (FFS)

#### 8.x.y.1	General

This procedure is initiated by an NG-RAN node to inform a neighbouring NG-RAN node about status changes concerning an admitted UL WUS configuration provision.

It is also used to indicate that the UL WUS configuration provisioning is ongoing or interrupted.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

The NG-RAN node2 shall indicate the status of the admitted UL WUS configuration provision in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

### 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Request Type | M |  | ENUMERATED(start, stop, …) | Type of request concerning the provisioning of the UL WUS Configuration. | YES | reject |
| UL WUS Configuration | O |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331. | YES | reject |
| Assisting Cells List |  | 0..1 |  | List of cells of the NG-RAN node2 requested to provide the UL WUS configuration. | YES | reject |
| >Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Indicates the NR CGI at the NG-RAN node2 | – |  |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

### 9.x.x.y	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration will be provided in the requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

### 9.x.x.z	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration cannot be provided by the NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Assisting Cells Failed List |  | 0..1 |  | List of cells of the NG-RAN node2 which cannot provide the UL WUS configuration. | YES | ignore |
| >Assisting Cells Failed Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Indicates the NR CGI at the NG-RAN node2 | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

### 9.x.x.t	UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report a status change concerning an admitted UL WUS configuration provision.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Provision Status | M |  | ENUMERATED (providing, not providing, …) | Indicates the provision status of the UL WUS configuration in the assisting cells. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** End of TP ***************