3GPP TSG-RAN WG3 Meeting #127 bis	   				    R3-251873

Wuhan, China, 7th – 11th Apr 2025

Agenda Item:		14.3

Source:			China Telecom

Title:		(TP for TS 38.300) Support of regenerative payload

Document for:	 	Other

- Introduction

The Rel-19 NTN enhancements WI contains the following objective[1]:

Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we will continue to discuss the potential impact of supporting NTN regeneration payloads from RAN3 perspective.

- Discussion

2.1 Feeder Link Switch over

In the case of soft feeder link switch over, the current control plane mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management. As for NG-U, When the NTN-GW changes, the NG-U channel may also change. The gNB needs to inform the UPF for the change of TNL address for all connected UEs, this can be performed by the gNB initiating the PDU Session Resource Modify Indication procedure or Path Switch procedure for each UE. The Path switch procedure may need to be implemented via the Handover, which requires the satellite to support additional functions (e.g. the same physical gNB implementations two logical gNBs), with additional impact on CN,e.g. connection with two logical gNBs during Handover and one connection with one logical gNB after Handover. Whether non HO Path switch procedure is supported may depend on implementation.

Proposal 1: In the case of feeder link switch over, gNB may provide the updated DL GTP-U tunnels via PDU Session Resource Modify Indication procedure or Path Switch procedure for each UE.

In case of hard feeder link switch over, a satellite/gNB can only be connected to one NTN-GW at the same time. If 5GC does not know the hard feeder link switch over, the 5GC may continue sending the DL NG-C/U packet to the old NTN-GW. The old NTN-GW has no way to deliver it to the gNB or forward it to the new NTN-GW, which will result in packet loss. Although the existing retransmission mechanism at application layer can work, it is better not to cause interruption for some services requiring service continuity (such as voice services). It is beneficial to inform the 5GC to buffer DL data based on actual situation of the UEs and feeder links, so the 5GC can suspend the DL NG-C/U.

Figure 1: Feeder link switchover without AMF change

- The gNB onboard the satellite connects to 5GC via NTN-GW1. In case of hard feeder link switchover, the gNB may informs AMF about the upcoming hard feeder link switchover. It can be per UE procedures or non-UE associated procedures.

2~4. The AMF informs the corresponding SMF(s) about the upcoming feeder link switchover receive from gNB. Based on feeder link switchover information, the SMF(s) controls the UPF(s) to buffer the DL data of the corresponding PDU sessions. It can be existing per UE procedures or new non-UE associated procedures, depending on further SA2 study.

- The gNB onboard the satellite performs Feeder link switchover to NTN-GW2. After the Feeder link is switched, the gNB is assumed to get a new TNLA.

- The gNB connects to AMF via NTN-GW2 by using the new TNLA as described in proposal 3.

7~8. The gNB needs to inform the UPF for the change of TNL address for all connected UEs, this can be performed by using existing procedures as described in proposal 4 or define new non-UE associated procedures.

Proposal 2: It is proposed to study if any enhancement is required for hard feeder link switch scenarios to reduce interruption and packet loss problem.

2.2 Location Based CHO

We have enhanced the Xn/NG interface to support time-based CHO/HO. At the final RAN3 meeting of Rel-18, some companies proposed to enhance location-based HO. Supporting location-based HO includes the following benefits:

- Location-based trigger condition generally do not have an "expiration", as the target gNB cannot cancel the configured HO and may face the risk of allocating resources that cannot be released;

- The early data forwarding adopted in NG HO with time-based trigger condition is also applicable to location-based trigger condition in principle;

- Coarse UE location information can assist target satellites for resource scheduling.

The current RAN3 specifications do not describe how to support CHO/HO with location-based trigger conditions in Xn/NG. For the mobility between gNBs, it is proposed that RAN3 consider whether and how to better support location-based CHO/HO via the Xn/NG interfaces. For example, introducing coarse UE location information or location-based trigger conditions or reusing time-based trigger conditions in the HANDOVER REQUEST message.

Proposal 3: It is proposed to study whether and how to support location-based CHO/HO via the Xn/NG interfaces.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: In the case of feeder link switch over, gNB may provide the updated DL GTP-U tunnels via PDU Session Resource Modify Indication procedure or Path Switch procedure for each UE.

Proposal 2: It is proposed to study if any enhancement is required for hard feeder link switch scenarios to reduce interruption and packet loss problem.

Proposal 3: It is proposed to study whether and how to support location-based CHO/HO via the Xn/NG interfaces.

- References

[1]	RP-241667, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.

- TP to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

16.14.4.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

For regenerative NTN payload, the current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switchover.

16.14.4.3	Procedures

In transparent NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switchover between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switchover is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

In regenerative NTN payload, the NTN Control function determines the point in time when the feeder link switchover is performed. During the feeder link switchover, AMF is not changed for the UEs.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>