3GPP TSG-RAN WG3 Meeting #127bis	R3-251876

Wuhan, China, 07th – 11th April 2025

Agenda Item:	9.2

Source:	ZTE, China Telecom, CATT, China Unicom

Title: 	Discussion on support of Carrier Bandwidth less than 5MHz

Document for:	Discussion

Introduction

In Rel-18, several market requests from vertical industry with the operating bandwidth less than 5 MHz were raised, such as Future Railway Mobile Communication System (FRMCS) in Europe, Smart Grids in USA and Public Safety in Europe. Based on this consideration, the new 3 MHz channel bandwidth, as well as the 12 and 20 PRB transmission bandwidths allow for gradual migration from the present GSM-R systems to 5G NR, while leaving room for a necessary number of GSM-R carriers on the same band (n100).

In the previous meetings, this issue was discussed in [R3-245614] and [R3-247509], however, no consensus is achieved.

This paper continues to discuss this issue based on the reply LS [R2-2500039] from RAN4 and latest agreement from RAN2.

Discussion

Special Carrier Bandwidth less than 5MHz in RAN4 specs

TS 38.101-1: User Equipment (UE) radio transmission and reception - 5.4.3.1 Synchronization raster and numbering The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present. A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The parameters defining the SSREF and GSCN for all the frequency ranges are in Table 5.4.3.1-1 for above 3 MHz channel bandwidth and in table 5.4.3.1-2 for 3 MHz channel bandwidth. For band n100, additional parameters defining the SSREF and GSCN are specified in table 5.4.3.1-3. The resource element corresponding to the SS block reference frequency SSREF is given in clause 5.4.3.2. The synchronization raster and the subcarrier spacing of the synchronization block is defined separately for each band. The synchronization raster and the corresponding SS block do not cover all possible RF channel bandwidths and locations on Enhanced channel raster. Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth Frequency range; SS Block frequency position SSREF; GSCN; Range of GSCN. 0 – 3000 MHz; N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} 1; 3N + (M-3)/2; 22 – 7498. 3000 – 24250 MHz; 3000 MHz + N * 1.44 MHz N = 0:14756; 7499 + N; 7499 – 22255. NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. NOTE 2: GSCN=2 (corresponding to ARFCN-ValueNR = 250) is a reserved value paired with reserved operating band n200.. Table 5.4.3.1-2: GSCN parameters for the global frequency for 3 MHz channel bandwidth Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 1000; N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1); 26638+3N + (M-3)/2; 26640 – 31634. NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. Table 5.4.3.1-3: Additional GSCN parameters for band n100 SS Block frequency position SSREF (MHz); GSCN; Note. 920.73; 41637; Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. 921.45; 41638; Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. TS 38.104: Base Station (BS) radio transmission and reception - 5.4.3.1 Synchronization raster and numbering The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present. A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The parameters defining the SSREF and GSCN for all the frequency ranges are in table 5.4.3.1-1 for above 3 MHz channel bandwidth and in table 5.4.3.1-2 for 3 MHz channel bandwidth. For band n100, additional parameters defining the SSREF and GSCN are specified in table 5.4.3.1-3. The resource element corresponding to the SS block reference frequency SSREF is given in clause 5.4.3.2. The synchronization raster and the subcarrier spacing of the synchronization block is defined separately for each band. The synchronization raster and the corresponding SS block do not cover all possible RF channel bandwidths and locations on Enhanced channel raster. Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 3000; N * 1200 kHz + M * 50 kHz, N = 1:2499, M ϵ {1,3,5}; 3N + (M-3)/2; 22 – 7498. 3000 – 24250; 3000 MHz + N * 1.44 MHz, N = 0:14756; 7499 + N; 7499 – 22255. 24250 – 100000; 24250.08 MHz + N * 17.28 MHz, N = 0:4383; 22256 + N; 22256 – 26639. NOTE 1: The default value for operating bands which only support SCS spaced channel raster(s) is M=3. NOTE 2: GSCN=2 (corresponding to ARFCN-ValueNR = 250) is a reserved value paired with reserved operating band n200.. Table 5.4.3.1-2: GSCN parameters for the global frequency raster for 3 MHz channel bandwidth Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 1000; N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1); 26638 + 3N + (M-3)/2; 26640 – 31634. NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [9] clause 7.4.3.1.. Table 5.4.3.1-3: Additional GSCN parameters for the global frequency raster and for band n100 SS block frequency position SSREF (MHz); GSCN; Note. 920.73; 41637; Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [9] clause 7.4.3.1.. 921.45; 41638; Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [9] clause 7.4.3.1..

In R3-247509, it states that:

The important point to note is that such Synchronisation Raster frequencies have been allocated by RAN4 only to the special cell configuration cases discussed in this paper (as shown in Table 5.4.3.1-3 of TS38.101). Namely: - Synchronisation Raster frequency 920.73 MHz is only used by cells using the band n100 with 3MHz channels and the special configuration of Nrb 12 (at Synch Raster frequency of 920.73 MHz) - Synchronisation Raster frequency 921.45 MHz is only used by cells using the band n100 with 5MHz channels and the special configuration of Nrb 20 (at Synch Raster frequency of 921.45 MHz) Conclusion 2: By knowing the ARFCN of a cell it is possible to derive whether such cell is one of such cases Cell using the band n100 with 3MHz channels and the special configuration of Nrb 12 at Synch Raster frequency of 920.73 MHz, Cell using the band n100 with 5MHz channels and the special configuration of Nrb 20 at Synch Raster frequency of 921.45 MHz

Observation 1: Network uses the special carrier bandwidth configuration (i.e., new ARFCN and new GCSN) to support Less than 5MHz for band n100.

RAN2 agreement

/////////////////////////////// RAN2 #129 meeting (Start) //////////////////////////////

R2-2500039	Reply LS on carrierBandwidth configuration for less-than-5MHz carriers (R4-2420382; contact: ZTE)	RAN4	LS in	Rel-18	NR_FR1_lessthan_5MHz_BW	To:RAN2	Cc:RAN1

Noted

R2-2500556	Less than 5Mhz and carrier configuration	Nokia	discussion	Rel-18	NR_FR1_lessthan_5MHz_BW

Noted

R2-2501220	Clarification on the less than 5M Configuration	ZTE Corporation	discussion	Rel-18	NR_FR1_lessthan_5MHz_BW-Core

Proposal 1: Add the following two clarifications to the field description of the carrierBandwidth for the case where only 12/20 PRB transmission bandwidth can be used:

a)	The network shall configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

b)	The UE shall use 12/20 PRB as the transmission bandwidth, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

-	Qualcomm and Samsung agree with the understanding but it is ok to capture it just in chair notes.    QUlacomm indicates that if we were to capture anything in the spec we should capture only 1a).

Proposal 2: Change the “Cond LessThan5MHz” description as below:

| LessThan5MHz | The field is mandatory present if the carrierBandwidth in SIB1 indicates UL or DL transmission bandwidth other than 15 PRB a cell doesn’t support 12 PRB, 15 PRB or 20 PRB transmission bandwidth, and but the corresponding neighbour cell(s) support(s) 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Otherwise, the field is optional, Need S. |
| --- | --- |

-	Qualcomm is concerned that we are going back to previous agreement where 12 and 20 is optional and it is not agreeable.   ZTE explains this is only for asymmetric case.   Qualcomm explains that the CR doesn’t indicate asymmetric case.

RAN2 confirms the following understanding for the case where only 12/20 PRB transmission bandwidth can be used: - The network shall configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling. - The UE shall use 12/20 PRB as the transmission bandwidth, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

[AT129][017][less5MHz] Offline discussion (ZTE)

Intended outcome: Way forward on whether spec changes are needed

Deadline:  Thursday

R2-2501221	Correction on the Less than 5M Configuration	ZTE Corporation	CR	Rel-18	38.331	18.4.0	5268	-	F	NR_FR1_lessthan_5MHz_BW-Core

R2-2501513	Correction on the Less than 5M Configuration	ZTE Corporation	CR	Rel-18	38.331	18.4.0	5268	1	F	NR_FR1_lessthan_5MHz_BW-Core	R2-2501221

The CR is agreed

/////////////////////////////// RAN2 #129 meeting (End) //////////////////////////////

3GPP TSG RAN WG2#129 R2-2500039 Athens, Greece, 17th - 21st February 2025 Title: Reply LS on carrierBandwidth configuration for less-than-5MHz carriers Response to: R2-2409409 Release: Rel-18 Work Item: NR_FR1_lessthan_5MHz_BW Source: RAN4 To: RAN2 Cc: RAN1 1 Overall description RAN4 would like to thank RAN2 for their LS R2-2409409. RAN4 confirms that it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling. In addition, RAN4 would like to provide the background that 12 PRB and 20 PRB transmission bandwidth configuration are two special cases only for band n100, and only can be configured with SS Block frequency positions in additional GSCN parameters defined in TS 38.101-1 Table 5.4.3.1-3. Therefore, in these two special cases UE can determine 12 or 20 PRB transmission bandwidth configuration should be used instead of 15 or 25 PRB transmission bandwidth configuration, respectively, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

R2-2501513 carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2). For the case that 12PRB/20 PRB transmission bandwidth is used as specified in TS 38.101-1 [15], TS 38.211[16] and TS 38.213[13], the network shall configure the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively and the UE shall use 12PRB/20 PRB as the transmission bandwidth respectively.

Observation 2: In RAN2 spec, it is agreed to introduce clarification (i.e., point to RAN4 spec) on special Carrier Bandwidth configuration.

Whether to introduce the clarification on special carrier bandwidth in RAN3

In this special carrier bandwidth configuration case (i.e., Less than 5MHz for band n100), although the maximum carrier 15PRB/25PRB is the same as legacy, the network configuration is different from legacy by adding new ARFCN and new GSCN for UE to search synch raster.

RAN2 spec has captured above clarification, which is used to point to the RAN4 specs for this special less than 5MHz case. So that, RAN2 experts are easy to read the corresponding chapter in the RAN4's spec, to understand the special 12PRB/20PRB case.

In RAN3, we also need this similar clarification, then RAN3 experts can according to the clarification, be easy to point to the corresponding chapter in the RAN4's spec, to understand the special 12PRB/20PRB case.

On the contrary, without this clarification in RAN3 specs, it is very difficult for RAN3 experts to 1) firstly read RAN2 spec to find the suitable chapter (“carrierBandwidth”), 2) then use this pointer in TS38.331 to the RAN4 specs.

As tradition, we suggest that both RAN2 and RAN3 add the similar clarification to point the RAN4 specs to the Less Than 5MHz case.

So, we do not think it is duplicated that both RAN2 and RAN3 have the similar pointer to the detail chapter in RAN4 specs, we think it is alignment between RAN2 and RAN3.

Proposal 1: Agree to introduce the clarification on special carrier bandwidth configuration in RAN3 specs.

How to introduce the clarification on special carrier bandwidth in RAN3

In the RAN2’s agreed CR R2-2501513, both network configuration and UE behavior are clarified. However, RAN3 only needs to clarify network configuration.

carrierBandwidth

Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2). For the case that 12PRB/20 PRB transmission bandwidth is used as specified in TS 38.101-1 [15], TS 38.211[16] and TS 38.213[13], the network shall configure the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively and the UE shall use 12PRB/20 PRB as the transmission bandwidth respectively.

Moreover, RAN2’s clarification points to the TS38.101 (User Equipment (UE) radio transmission and reception), accordingly, RAN3’s clarification shall point to the TS38.104 (Base Station (BS) radio transmission and reception) other than TS38.101-1.

Proposal 2: Add a note into XnAP and F1AP specs, as below.

In the case that 12PRB/20PRB transmission bandwidth within 3MHz/5MHz channel is used as specified in Table 5.4.3.1-3 of TS 38.104[24], the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) is configured respectively.

Accordingly, the following change is proposed in the RAN3 specs.

| TS38.423 and TS38.473 |  |  |  |  |
| --- | --- | --- | --- | --- |
| >Carrier Bandwidth | M |  | INTEGER (1.. maxnoofPhysicalResourceBlocks, ...) | Width of this carrier in number of PRBs (using the NR SCS IE defined for this carrier). See TS 38.211 [33], clause 4.4.2. NOTE: In the case that 12PRB/20PRB transmission bandwidth within 3MHz/5MHz channel is used as specified in Table 5.4.3.1-3 of TS 38.104[24], the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) is configured respectively. |

In addition, whether this clarification can be applied in EN-DC deployment.

RAN2 #129 meeting also discussed another issue on SSB position restriction as below.

R2-2500950	SSB position restrictions for less-than-5MHz SCells	Qualcomm Incorporated	CR	Rel-18	38.331	18.4.0	5249	-	F	NR_FR1_lessthan_5MHz_BW-Core

-	Qualcomm thinks that because RAN4 does release independent

Have the discussion on the LS from RAN4 first in the context of Rel-19 and then determine whether we start from Rel-18.

[AT129][030][Less5MHz] SSB position restrictions CR (Qualcomm)

Intended outcome: Agree to CR (if agreable for Rel-18)

Deadline:  short

After short email discussion, CR R2-2501389 is nearly agreed. In this CR’s reason for change as below, for the restriction on SSB positions of less-than-5MHz NR SCells, it can be applied in CA, NR-DC or EN-DC

RAN4 sent LS in R4-2420383/ R2-2500040 stating that RAN4 has agreed to restrict SSB positions of less-than-5MHz NR SCells to be always on the sync raster and capture the restriction in Tables 5.4.3.1-2 and 5.4.3.1-3 in TS 38.101-1. Restriction applies independent of whether the operation is in CA, NR-DC or EN-DC.

For the similar reason, we think the special carrier bandwidth configuration for less than 5MHz can also be applied in EN-DC.

Proposal 3: Add a same note into X2AP spec, as below.

In the case that 12PRB/20PRB transmission bandwidth within 3MHz/5MHz channel is used as specified in Table 5.4.3.1-3 of TS 38.104[24], the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) is configured respectively.

Proposal 4: Agree with corresponding XnAP, F1AP and X2AP CRs.

Proposal

Based on the above analysis, we have the following observations and proposals:

Observation 1: Network uses the special carrier bandwidth configuration (i.e., new ARFCN and new GCSN) to support Less than 5MHz for band n100.

Observation 2: In RAN2 spec, it is agreed to introduce clarification (i.e., point to RAN4 spec) on special Carrier Bandwidth configuration.

Proposal 1: Agree to introduce the clarification on special carrier bandwidth configuration in RAN3 specs.

Proposal 2: Add a note into XnAP and F1AP specs, as below.

In the case that 12PRB/20PRB transmission bandwidth within 3MHz/5MHz channel is used as specified in Table 5.4.3.1-3 of TS 38.104[24], the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) is configured respectively.

Proposal 3: Add a same note into X2AP spec, as below.

In the case that 12PRB/20PRB transmission bandwidth within 3MHz/5MHz channel is used as specified in Table 5.4.3.1-3 of TS 38.104[24], the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) is configured respectively.

Proposal 4: Agree with corresponding XnAP, F1AP and X2AP CRs.

Reference

- R3-245614, Discussion on Transmission Bandwidth less than 5MHz, China Telecom, ZTE

- R3-247509, On the special case of Nrb 12 and Nrb 20 Transmission Bandwidth configurations, Ericsson, Strict, Qualcomm

- R3-251877, Clarification on Carrier Bandwidth less than 5MHz in TS38.423

- R3-251878, Clarification on Carrier Bandwidth less than 5MHz in TS38.473

- R3-251879, Clarification on Carrier Bandwidth less than 5MHz in TS36.423