3GPP TSG-RAN WG3 Meeting #127bis	R3-251891

Wuhan, China, 7th - 11th Apr 2025

Agenda Item:	14.3

Source:	ZTE Corporation

Title:	(TP to BL CR 38.413) Further discussion on support of regenerative payload

Document for:	Discussion and Approval

1  Introduction

In previous meetings, the discussion on support of regenerative payload was discussed, and the following agreements were reached[1].

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection? Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue: RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked. The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. Supported TAI list of the on-board gNB: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

In this contribution, we provide our further views on the support of regenerative payload for NTN in Rel-19.

2  Discussion

2.1  NG connection management

In RAN3#125 meeting, it has been agreed that the NG Removal/Setup mechanism can be used for NG interface management, and another option, NG Suspend/Resume mechanism can be further discussed.

For the NG Suspend/Resume mechanism, when the gNB decides to leave the service area of AMF based on the ephemeris information, it shall trigger the existing NGAP procedure (e.g. RAN Configuration Update procedure) to the AMF to suspend the NG interface instead of removing it. In this case, the gNB and the AMF shall store the UE context and the configuration information related to the NG interface. When the gNB re-enters the service area of AMF, the stored UE context and configuration information could be reused, and the NG interface shall be resumed.

Considering the NGAP signalling, two NGAP procedures need to be handled for NG Setup/Removal mechanism, i.e., the NG Setup procedure and NG Removal procedure, and the NG Removal procedure is a totally new NGAP procedure. While, for NG Suspend/Resume procedure, only one NGAP procedure need to be handled, e.g, the existing NGAP RAN Configuration Update procedure. Therefore, from the prospective of standard impact on NG interface, no new procedure needs to be introduced for the NG Suspend/Resume procedure.

Considering the energy consumption, for the NG Suspend/Resume procedure, since the gNB and AMF shall store the UE context and configuration information, it shall consume some energy for storage. While, with the development of storage technology, after 3 to 5 years, when the Regenerative payload is officially deployed, the storage energy cost shall be much less, and the storage is not a big issue possibly.

In addition, with the NG Suspend/Resume procedure, the NG interface can be timely resumed and the UE can attach the network quickly, which can bring the better user’s experience. In some specific circumstances, for the satellite whose moving orbit is almost vertical to equator, the satellite will fly back to the AMF easily, then the storage time will be not much.

Proposal 1: In addition to the NG Setup/Removal mechanism, the NG Suspend/Resume mechanism should be applied for the management of NG connection.

2.2  Support of RRC_INACTIVE

In RAN3#126 meeting, the support of RRC Inactive UE in NTN has been discussed, and some potential options to solve this issue are given as below.

Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

In [2], the detailed solution and possible impact/issues of the three options have been summarized in the following table.

| Options | Detail solution | Possible Impacts or issues |
| --- | --- | --- |
| Option1, UE context is forwarded to the next satellites | Page and relocate UE context to next the satellite | Stage2 impacts, rely on the ISL availability, may have signalling storm. |
|  | Pro-actively move UE context to next satellite (without notifying UE) | Xn impacts, may need to check SA3 with the security issue. |
| Option2, UE context is kept on a fixed node | relocate the UE context to the AMF on ground. | NG impacts |
|  | relocate the UE context to the gNB on ground. | Xn impacts |
| Option3, UE context is kept in the last serving gNB | NG-based UE context retrieve procedure is used if ISL is not available | NG impacts, may not work if NG is removed. |

- For Option 1, if the UE context is pro-actively moved to next satellite, there may be some security issues on integrity protection. While, if the new serving gNB is paged and the UE context is relocated to the new serving gNB before the ISL is to be disconnected, although it may bring some signalling overhead, it is workable. However, if we go with this option, the limited storage capability of the new serving gNB on board should be considered. And how to control the reception of the UE context is an important issue.

For Option 2, if the UE context is relocated to AMF on ground, there seems to be no corresponding procedure for the gNB to retrieve the UE context from the AMF. In this case, the UE Context Retrieve procedure over NG should be introduced. In addition, the new serving gNB may not be able to connect to the specific AMF on ground. If the UE context is relocated to the gNB on ground, since the TN-NTN Xn interface has not specified in current release, it is impossible for the new serving gNB to retrieve the UE context from the gNB on ground. Therefore, Option 2 should not be considered in this release.

For Option 3, the NG interface may not always exist, and a totally new procedure over NG should be introduced to support this option. Since the time in Rel-19 is limited. It may be difficult to introduce the new procedure over NG in this release.

Since there are some issues to be clarified for every option, and the time in Rel-19 is limited, we could try to solve the support of RRC_Inactive UE in next release.

Proposal 2: Since the remaining time in Rel-19 is limited, the support of RRC Inactive UE for NTN could be considered in next release.

3  Conclusion

Proposal 1: In addition to the NG Setup/Removal mechanism, the NG Suspend/Resume mechanism should be applied for the management of NG connection.

Proposal 2: Since the remaining time in Rel-19 is limited, the support of RRC Inactive UE for NTN could be considered in next release.

The TP to BL CR for 38.413 is given in the Annex.

References

- RAN3#127 Chairman Notes

- R3-247316, Support of Inactive UE mobility in NTN, Xiaomi, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, China Telecom, Lenovo

Annex

TP to BL CR for TS38.413 on NG Suspend/Resume mechanism for regenerative payload

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8.7.2	RAN Configuration Update

8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the RAN CONFIGURATION UPDATE message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the Global RAN Node ID IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall associate the TNLA to the NG-C interface instance using the Global RAN Node ID.

If the RAN CONFIGURATION UPDATE message includes the NG-RAN TNL Association to Remove List IE, the AMF shall, if supported, initiate removal of the TNL association(s) indicated by NG-RAN TNL endpoint(s) and AMF TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is present, or the TNL association(s) indicated by NG-RAN TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the NG-RAN TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN TNL endpoints correspond to all NG-RAN TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-    if the received TNL Association Transport Layer Address at AMF IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the RAN CONFIGURATION UPDATE message includes the RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node. If the RAN CONFIGURATION UPDATE message includes the Extended RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite any previously stored NB-IoT default paging DRX value for the NG-RAN node.

If the RAT Information IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the NTN Suspend Indication IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, suspend the NG interface.

If the NTN Resume Indication IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, resume the NG interface.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| NTN Suspend Indication | O |  | ENUMERATED (true, ...) | Indicates the NG interface is suspended. This IE only applies to NTN. | YES | ignore |
| NTN Resume Indication | O |  | ENUMERATED (true, ...) | Indicates the NG interface is resumed. This IE only applies to NTN. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

<<<<<<<<<<<<<<<<<<<<End of Changes >>>>>>>>>>>>>>>>>>>>

ASN.1 part will be added later