3GPP TSG-RAN WG3 Meeting #127-bis	R3-251896

Wuhan, China, 7-11 April, 2025

Title: 	Discussion on the emergency services and eDRX

Source: 	Huawei

Agenda item:	8.1

Document Type:	 Discussion

# 1. Introduction

The previous RAN3-127 meeting discussed the emergency services and eDRX, based on the LS in [1] from the last RAN2-128 meeting, but without no conclusion. The SA2#167 meeting provides the following replies from SA2 perspective in [2].

1. Overall Description: SA2 thanks RAN2 for the LS on emergency services and eDRX (S2-2500044/R2-2411041). The following question is raised by RAN2 in that LS: In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be. #### 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE ##### 5.31.7.2.1 Overview ##### // skipped irrelevant part When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. // skipped irrelevant part. Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN. RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this. RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed? SA2 analysed the identified issue above and would like to provide the following feedback: - Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node. - In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10. Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN. SA2 would like to ask RAN2 to provide feedback. 2. Actions: To RAN2 group: ACTION: SA2 kindly requests RAN2 to take the above information into account

Though RAN3 is in CC, this contribution provides our views on the received LS, if the online time allows.

# 2. Discussion

Though the LS from RAN2 mentioned that the NG-RAN may not be aware of the emergency PDU session, SA2 confirmed that the existing SA2 specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node, so the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node. Also, when referring to the TS 38.300, there are some procedure texts on the RAN awareness of the emergency services.

- Observation1: SA2 confirms that the NG-RAN could identify the emergency PDU session and TS 38.300 already had such descriptions.

The SA2 reply LS does not provide clear answers for the eDRX configuration for the RRC inactive UE. There are the following possible solutions for different cases as follows.

- Case 1: When the emergency PDU session does not exist at the NG-RAN node (i.e., the emergency PDU session is deactivated at the CN)

  - Option 1a: The AMF can update the RRC Inactive Assistance Information to exclude the extended idle mode eDRX information. Then the NG-RAN would not configure the RAN eDRX for the inactive UE.

- Case 2: When the emergency PDU session exists at the NG-RAN node (i.e., the emergency PDU session is not deactivated at the CN):

  - Option 2a: The NG-RAN would not turn the connected UE to be inactive.

  - Option 2b: The NG-RAN would not configure the RAN eDRX for the inactive UE

It can be observed that the key point is whether the emergency PDU session can be deactivated at the CN.  RAN3 can initially discuss these the above options.

RAN3 to initially discuss the solutions for the two different cases below:

  - Case 1: When the emergency PDU session does not exist at the NG-RAN node (i.e., the emergency PDU session is deactivated at the CN)

    - Option 1a: The AMF can update the RRC Inactive Assistance Information to exclude the extended idle mode eDRX information. Then the NG-RAN would not configure the RAN eDRX for the inactive UE.

  - Case 2: When the emergency PDU session exists at the NG-RAN node (i.e., the emergency PDU session is not deactivated at the CN):

    - Option 2a: The NG-RAN would not turn the connected UE to be inactive.

    - Option 2b: The NG-RAN would not configure the RAN eDRX for the inactive UE

Second, as indicated in the SA2 reply LS, it requests RAN2 to provide further feedback. From RAN3 perspective, we see many benefits that the RAN eDRX should be treated the same as idle eDRX, i.e., both configured, or neither configured in case of emergency PDU session. This allows the UE behaviour is pretty clear for the emergency PDU session case.

RAN3 to reply that from RAN3 perspective, it is preferred that the RAN eDRX should be treated the same as idle eDRX, i.e., both configured, or neither configured in case of emergency PDU session.

In addition, SA2#167 meeting also discussed this issue not only for eDRX, but also for PEI, LP-WUS. It was agreed in [3] that the unnecessary restrictions for emergency PDU sessions for PEI and LP-WUS was removed. The main point is that PSAP callback is considered as normal call in IMS PDU session, instead of the emergency PDU session.

Considering the eDRX could be coupled with the PEI, and LP-WUS, we consider all these features should be treated in a unified way for the emergency PDU session. And for PEI/LP-WUS, also the UE ID based subgrouping should be considered in a unified way as CN based subgrouping. This should be conveyed to other WGs for further consideration.

RAN3 to reply that a unified solution for eDRX, PEI, LP-WUS is preferred for emergency PDU session. Then it is up to SA2 to decide the final solution, e.g., whether the emergency PDU session could be deactivated or not.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Observation1: SA2 confirms that the NG-RAN could identify the emergency PDU session and TS 38.300 already had such descriptions.

- RAN3 to initially discuss the solutions for the two different cases below:

  - Case 1: When the emergency PDU session does not exist at the NG-RAN node (i.e., the emergency PDU session is deactivated at the CN)

    - Option 1a: The AMF can update the RRC Inactive Assistance Information to exclude the extended idle mode eDRX information. Then the NG-RAN would not configure the RAN eDRX for the inactive UE.

  - Case 2: When the emergency PDU session exists at the NG-RAN node (i.e., the emergency PDU session is not deactivated at the CN):

    - Option 2a: The NG-RAN would not turn the connected UE to be inactive.

    - Option 2b: The NG-RAN would not configure the RAN eDRX for the inactive UE

RAN3 to reply that from RAN3 perspective, it is preferred that the RAN eDRX should be treated the same as idle eDRX, i.e., both configured, or neither configured in case of emergency PDU session.

RAN3 to reply that a unified solution for eDRX, PEI, LP-WUS is preferred for emergency PDU session. Then it is up to SA2 to decide the final solution, e.g., whether the emergency PDU session could be deactivated or not.

The draft CRs to reflect the above two options (option1a and 2b) in proposal 2 could be found in [4-5], just for reference.  And the draft reply LS can be found in [6].

# 4. Reference

- R2-2411041 LS on emergency services and eDRX, RAN2

- S2-2502427 Reply LS on emergency call back and paging, SA2

- S2-2502428  Removing unnecessary restrictions for emergency PDU sessions, Qualcomm

- R3-251897	emergency services and eDRX for RRC inactive UE, Huawei

- R3-251898	emergency services and eDRX for RRC inactive UE, Huawei

- R3-251899	[Draft] Reply LS on emergency call back and paging, Huawei