3GPP TSG-RAN WG3 Meeting #127-bis	R3-251919

Wuhan, China, 7–17 April 2025

Title:	AI/ML support for NR-DC

Source:	Ericsson, Jio Platforms, InterDigital

Agenda item:	11.4

Document Type:	Discussion and agreement

# Introduction

At the RAN3#126 meeting, the following agreements and “to-be-continued” were captured for AI/ML assisted mobility optimization for NR-DC in the meeting minutes:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

Other information, e.g., predicted SCG bearer?

At RAN3-127, little progress was achieved towards any of the open points above.

In this contribution, we discuss the consequences of the above agreements and other aspects related to the enhancements needed in NG-RAN interfaces to support this use case.

# AI/ML support for mobility optimization for NR-DC

## General

Mobility in NR-DC can be optimized by means of AI/ML.

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only and Conditional Dual Connectivity procedures are out of scope.

The following is captured in the TR 38.743 in relation to AI/ML-based mobility optimization for NR-DC (clause 5.1.1), where we added the yellow highlight for clarity:

The text clearly states that AI/ML techniques can be used to optimize mobility in NR-DC; furthermore, any inference related to this optimization is located at the MN only. The question is thus, how can AI/ML techniques be used at the MN?

Given the text above and the conclusions captured in the TR 38.743, we would like to focus the discussion on two scenarios:

- a UE is in single connectivity, and the serving gNB infers the decision to initiate SN Addition,

- a UE is in NR-DC, and the MN infers the decision to initiate SN Change.

The following types of enhancements are needed to support the scenarios above:

- Enhancements related to AI/ML-based decisions to select dual connectivity for improved performance

- Enhancements related to data collection to assist the AI/ML-based decisions at the MN

We discuss both types of enhancements below.

## Enhancements for configuring dual connectivity

It is reasonable to think that the MN has inferred the action of SN Addition (or SN Change) assuming that adding (or changing) the SN would bring some benefits to the UE performance. We think that it is good if the MN informs the SN about some of the assumptions made, so that the SN can try to provide the best possible service to the UE. We would like to emphasize that a good implementation would not incur in the cost of using NR-DC if this is not benefitial to the UE in some way.

Observation 1:  An MN infers the action to add or change SN based on an assumption that this action will bring some benefits to the UE performance.

The benefits to UE performance brought by dual connectivity mainly stem from the more network resources available to the UE. However, these benefits can only be realized if the additional network resources from the SN are used. In other words, the SN needs to serve some part of the UE traffic for NR-DC to improve the UE performance. If the SN does not serve any traffic, there would be no reason to configure the UE in dual connectivity.

Observation 2:  An MN infers the action to add or change SN based on an assumption that this action will bring some benefits to the UE performance due to the (new) SN serving a certain amount of UE traffic.

In response to the above question of how AI/ML techniques can be applied to optimize mobility in NR-DC, one possible answer is that these techniques can be leveraged to predict UE performance under various configurations. Consequently, a UE would be configured in dual connectivity when this is forseen to yield (reasonable) improvements in its performance. Some companies have argued that signaling the PSCell the MN assumes to be used at the SN is a sufficient enhacement for mobility optimization in NR-DC. However, as previously noted, the effectiveness of moving a UE to dual connectivity is questionable if the SN does not serve any traffic. Signalling from the MN to the SN the PSCell the MN assumes to be used does not help achieving an optimal DC configuration because e.g. the SN may serve very little (or even no) traffic, therefore not producing the performance improvement that an AI/ML based NR-DC action is supposed to bring. Therefore, if the SN does not know from the MN what traffic it should serve (according to the MN predictions made via AI/ML), the chosen dual connectivity configuration might not result in improved UE performance.

Observation 3:  Merely signaling the PSCell that the MN assumes to be used at the SN does not provide sufficient information for the SN to select the best dual connectivity configuration that will indeed enhance UE performance.

The argument to only signal the PSCell seems to follow the work in mobility optimization for single connectivity done during Rel-18, which resulted in the introduction of predicted UE trajectory in the handover procedure. The key difference in dual connectivity is that, unlike single connectivity where data traffic terminates at a single cell/NG-RAN node, traffic is distributed between the MN and the SN in a flexible way. In dual connectivity, given that the UE maintains a connection with the MN, it is unclear how much UE traffic should be served by the SN once the AI/ML predicted DC configuration for the UE is activated, therefore there is a need to explicitly signal this information. Therefore, we see a need that both the assumed PSCell and the UE traffic that needs to be served by the SN are made available to the SN as assistance information.

Observation 4:  The assumptions made by the MN when deciding to configure the UE in dual connectivity (i.e., the assumed PSCell and the UE traffic that needs to be served by the SN) need to be available at the SN for the SN to select the best dual connectivity configuration that will enhance UE performance.

For example, when a (future) MN carries out an AI/ML-based SN Addition, it may request the establishment of SN-terminated bearers from the (future) SN. This decision can be based on the assumption that certain QoS flows will be served by the SN (as SCG). This is not possible in the current framework, where an SN might opt to have all QoS flows, for which the MN requested SN-terminated bearers, to be served by MCGs instead. Therefore, when the MN derives an AI/ML-based request for resources for SN-terminated bearers, it should be able to inform the SN that this request assumes that the SN will handle some or all of the QoS flows as SCGs. Upon receiving this information, the SN is left free to decide whether to serve the QoS flows as either MCG or SCG. However, it is now able to potentially align its decisions with the MN's assumptions, if feasible.

Therefore, we propose that, during SN Addition, the MN could inform the SN that it assumed that a specific cell, such as CellA, is selected as PSCell. While the SN is still able to take its own decisions, it is now possible to consider selecting CellA as the PSCell if feasible. Together with this, we propose that the MN should inform the SN of its assumptions regarding which QoS flows the SN is expected to serve as SCG. It is crucial to emphasize that both pieces of information—the chosen cell to select as PSCell and the expected traffic in terms of QoS flows within the SCG—are vital and should be considered together. This combined assistance information will enable more informed and effective decision-making at the SN.

Proposal 1:  Enhance dual connectivity procedures (e.g., SN addition, MN-initiated SN change) to indicate MN assumptions to be considered by the SN, namely the cell the MN assumed as PSCell, the QoS flows the MN assumed to be served as SCGs.

## Enhancements for data collection in dual connectivity

To assist inference at the MN (both if it is a “future MN” or a “current MN”), it is possible to mostly reuse the Rel-18 signaling design established to support AI/ML-assisted use cases. As already agreed in the RAN3#126 meeting, we can reuse the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to allow an MN (future or current) to request and obtain from a neighbor gNB (acting as SN) the reporting of UE performance. This is achieved by including the Data Collection ID IE in the XnAP S-NODE ADDITION REQUEST message, which signals to the (new or target) SN that it should start collecting the UE performance previously requested by the MN

Another part of the Rel-18 solution for collection of UE performance after a handover concerns the conditions that the target gNB needs to monitor to stop the collection of UE performance requested by the source gNB. For NR-DC, similar conditions can be defined, so that the SN takes them into account to terminate the collection of UE performance requested by the MN. In our view, at least the following conditions should be considered:

- Expiration of a timer (e.g., reporting time interval), after successful SN Addition;

- The SN is removed from NR-DC (MN-initiated or SN-initiated);

- The PSCell is changed.

Proposal 2:  For AI/ML-assisted mobility optimization for NR-DC, define the following conditions for the SN to stop collection of UE performance requested by the MN: 1) expiration of a timer, after successful SN Addition; 2) SN removal (MN initiated or SN initiated); 3) change of the PSCell.

Furthermore, in the Rel-18 solution, a condition is specified according to which the target gNB should stop collecting UE performance when the “UE is handed over to another cell”. UE performance is currently reported from target gNB to source gNB without an explicit indication of the cell they refer to. This is good enough for the Handover case, since the target cell is explicitly signalled in the HANDOVER REQUEST message, and the target gNB knows exactly for which cell it should report the UE performance. Also, since the data collection should stop when the UE is handed over to another cell, there is only one cell the UE performance can refer to. With some enhancements, the existing signaling design can be reused for NR-DC.

Essentially, the reported UE performance should be extended to include the PSCell used by the UE, so that it is clear to the MN as to which cell the UE performance refers. The MN can use this information, e.g., to understand whether its assumptions to select the SN were optimal or not.

Proposal 3:  For AI/ML-assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

# Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1:  An MN infers the action to add or change SN based on an assumption that this action will bring some benefits to the UE performance.

Observation 2:  An MN infers the action to add or change SN based on an assumption that this action will bring some benefits to the UE performance due to the (new) SN serving a certain amount of UE traffic.

Observation 3:  Merely signaling the PSCell that the MN assumes to be used at the SN does not provide sufficient information for the SN to select the best dual connectivity configuration that will indeed enhance UE performance.

Observation 4:  The assumptions made by the MN when deciding to configure the UE in dual connectivity (i.e., the assumed PSCell and the UE traffic that needs to be served by the SN) need to be available at the SN for the SN to select the best dual connectivity configuration that will enhance UE performance.

Proposal 1:  Enhance dual connectivity procedures (e.g., SN addition, MN-initiated SN change) to indicate MN assumptions to be considered by the SN, namely the cell the MN assumed as PSCell, the QoS flows the MN assumed to be served as SCGs.

Proposal 2:  For AI/ML-assisted mobility optimization for NR-DC, define the following conditions for the SN to stop collection of UE performance requested by the MN: 1) expiration of a timer, after successful SN Addition; 2) SN removal (MN initiated or SN initiated); 3) change of the PSCell.

Proposal 3:  For AI/ML-assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

A TP mirroring the above proposals is presented in R3-251920.