3GPP TSG-RAN WG3 #127-bis	R3-251953

Wuhan, China, 7th – 11th Mar 2025

Agenda Item:	20.2

Source:	Lenovo

Title: 		AIML for gNB assisted positioning

Document for:	Discussion and Approval

# Introduction

In this paper, we further discuss issues related to AIML for gNB assisted positioning case 3a and 3b.

# Discussion

## Case 3a: AIML for NG-RAN node assisted positioning with gNB-side model

Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement.

For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

In the previous RAN3 meeting, “opportunistic” and “proactive” approaches have been identified for gNB to collect the training data Part B from LMF. In our understanding, both “opportunistic” and “proactive” approaches can be supported for different purposes:

- “opportunistic” approach can work for e.g. AIML monitoring during an on going AIML based positioning session.

- “proactive” approach can work for the initial offline training with data collected from good amount of UEs.

RAN3 confirms that both “opportunistic” and “proactive” approaches are supported for gNB to collect the part B data from LMF.

In legacy UL based positioning, the NRPPa measurement information transfer procedure is used by the LMF to request and receive the SRS measurement from a gNB. The concerned NRPPa messages are contained in UL/DL Non UE Associated NRPPa Transport messages in NGAP interface. The gNB, especially non-serving gNB, performs SRS measurements and reports the measurement result to LMF without knowing the associated UE context.

TS 38.455 8.5 Measurement Information Transfer 8.5.1 Measurement 8.5.1.1 General The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB. 8.5.1.2 Successful Operation Figure 8.5.1.2.1: Measurement procedure. Successful operation. If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission. If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time. If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message. If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements. If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements. If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance. If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, measure the UL SRS resources from the UE within the indicated time window(s). TS 38.413 8.10 NRPPa Transport Procedures 8.10.1 General The purpose of the NRPPa Transport procedures is to carry NRPPa signalling (defined in TS 38.455 [19]) between the NG-RAN node and the LMF over the NG interface. The Downlink UE Associated NRPPa Transport procedure and the Uplink UE Associated NRPPa Transport procedure use UE-associated signalling. The UE-associated signalling is used to support E-CID Location Information Transfer, Positioning Information Transfer, Measurement Preconfiguration Information Transfer, and Reporting of General Error Situations due to reception of an NRPPa message that utilized UE-associated signalling. The Downlink Non UE Associated NRPPa Transport procedure and the Uplink Non UE Associated NRPPa Transport procedure use non-UE associated signalling. The non-UE associated signalling is used to support OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer, PRS Information Transfer, and Reporting of General Error Situations due to reception of an NRPPa message that utilized non-UE associated signalling.

In the legacy UL based positioning, LMF requests and receives SRS configuration from gNB in a UE agnostic manner.

Following similar principle, for AI training purpose, it would be enough for serving/non-serving gNB to know the association between an SRS measurement and the corresponding Part B label, without further linkage to a UE ID.

When LMF provides the Part B label information to serving/non-serving gNB, the Part B label information is associated with an SRS resource set that was previously configured to the gNB, e.g., in MEASUREMENT REQUEST message.

RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For activating a gNB-sided model, one needs to know in which scenario the trained AI model will be applicable. This is by nature known by gNB itself, but difficult to transfer the meta data about the gNB-sided AI model to LMF. Besides, since last meeting, RAN3 agreed to only support RAN based monitoring for case 3a, then it should be upon gNB’s own decision if an AIML model will be activated/deactivated and used to improve the positioning accuracy for case 3a.

FFS if LMF should be aware that the reported measurements are AI/ML generated.

Since there is no mean for LMF to understand if the measurements reported by gNB based on AIML is good or not, there seems no need for LMF to be aware if the measurements is generated by AIML neither.

For Case 3a, it is upon gNB’s decision if an AIML based method will be used to improve the positioning accuracy.

LMF does not need to explicitly know if the measurements reported by gNB are AIML generated or not.

## Case 3b: AIML for NG-RAN node assisted positioning with LMF-side model

RAN1 agreements relevant to Case 3b: Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

SA2 has sent an LS to RAN3 in R3-250074 asking RAN3’s feedback related to

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

According to RAN1 progress so far, for case 3b, the only RAN3 impact would be any enhancement to support new measurement exchange between gNB and LMF. For example,

For sample-based measurement (if supported), LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

When gNB reports the channel measurement result to LMF, gNB may also report timing information and paired timing information and power information.

For Case 3b, based on RAN1 discussion, RAN3 supports exchanging the following information between gNB and LMF over NRPPa:

  - From LMF to gNB, if sample-based measurement is supported, LMF can signal parameter values of Nt, Nt', k to gNB

  - From gNB to LMF, gNB will report (mandatory) channel measurement, (optional) quality of the channel measurement, and (mandatory) time stamp of the channel measurement.

In legacy, NRPPa procedures such as Positioning Information Exchange, TRP Information Exchange, Measurement procedures were specified for gNB to perform UL SRS based measurement according to LMF’s demand and send the result to LMF for UL based positioning (UL-TDOA, UL-AoA). Those existing NRPPa procedures can be used with possible enhancement to provide input for the LMF-sided model in case 3b.

For Case 3b, existing NRPPa procedures such as Positioning Information Exchange, TRP Information Exchange, Measurement procedures can be used with possible enhancement to provide input for the LMF-sided model.

From RAN load perspective, there seems no fundamental difference compared to when gNB is involved in legacy UL based positioning (UL-TDOA, UL-AoA). Therefore, no specific enhancement is foreseen.

For Case 3b, RAN3 assumes the load experienced by RAN is similar as when RAN is involved in legacy UL based positioning.

# Conclusion

Based on the discussion above, we observe:

Observation 1	In the legacy UL based positioning, LMF requests and receives SRS configuration from gNB in a UE agnostic manner.

Based on the discussion above, we propose:

Proposal 1	RAN3 confirms that both “opportunistic” and “proactive” approaches are supported for gNB to collect the part B data from LMF.

Proposal 2	When LMF provides the Part B label information to serving/non-serving gNB, the Part B label information is associated with an SRS resource set that was previously configured to the gNB, e.g., in MEASUREMENT REQUEST message.

Proposal 3	For Case 3a, it is upon gNB’s decision if an AIML based method will be used to improve the positioning accuracy.

Proposal 4	LMF does not need to explicitly know if the measurements reported by gNB are AIML generated or not.

Proposal 5	For Case 3b, based on RAN1 discussion, RAN3 supports exchanging the following information between gNB and LMF over NRPPa:

a.	From LMF to gNB, if sample-based measurement is supported, LMF can signal parameter values of Nt, Nt', k to gNB

b.	From gNB to LMF, gNB will report (mandatory) channel measurement, (optional) quality of the channel measurement, and (mandatory) time stamp of the channel measurement.

Proposal 6	For Case 3b, existing NRPPa procedures such as Positioning Information Exchange, TRP Information Exchange, Measurement procedures can be used with possible enhancement to provide input for the LMF-sided model.

Proposal 7	For Case 3b, RAN3 assumes the load experienced by RAN is similar as when RAN is involved in legacy UL based positioning.