3GPP TSG-RAN WG3 Meeting #127bis	R3-251997

Wuhan, China, 7th – 11th April, 2025

Title: 	(TP for AI/ML BLCR to TS 38.423) Discussion on AI/ML-based Mobility Optimization in NR-DC

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to the mobility optimization for NR-DC.

This contribution analyzes the technical enablers for specifying support over the Xn interface for mobility optimization in NR-DC, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

## 2.1 Assistance information for SN configuration

In the last RAN3 meeting the following FFS was captured in the Chair’s meeting minutes, but there was no progress due to lack of time [4]:

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

The above follows the discussion already occurred in RAN3#126 meeting, with following FFSs being captured in the meeting minutes, which are related to the specification of assistance information that could be sent over Xn from Master Node (MN) to Secondary Node (SN) for subsequent SN configuration [5]:

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information? Other information, e.g., predicted SCG bearer?

There were some contributions submitted in the last meeting, e.g. [6][7], proposing to indicate the PSCell ID predicted (or assumed, as labelled in [7]) by the MN to help the SN in choosing and configuring the Secondary Cell Group (SCG). First of all, it should be recalled that RAN3 had a discussion on providing predicted information from the MN to the SN during the SI, and discussed in particular the predicted UE trajectory in the SN (across PSCells). Back then, there was no consensus in RAN3 on transferring such prediction over the Xn interface as captured in RAN3#125 meeting [8]:

There is no consensus to transfer the predicted/measured UE trajectory over NR-DC. Check online whether to support it during normative phase.

and in our view the predicted PSCell ID is a particular case where the predicted UE trajectory in the SN is only limited to a single PSCell. Moreover, in our understanding, in the legacy NR-DC the MN does not provide any information to the SN to help it in selecting the PSCell of the SCG, and we see no technical reasons to deviate from this behaviour, also considering that among proposing companies there is no clear view on whether the PSCell ID information to be transferred to the SN is either a prediction or an assumption, and what is the difference between the two.

Our understanding is that the predicted PSCell ID (as proposed in [6]) is an output of the AI/ML model used in the MN – which, by the way, has not been agreed to be included in TR 38.743 [3] in clause 5.1.2 for the potential standard impact – and hence this will require RAN3 to also discuss the feedback information which is needed to validate such prediction. Proponents of [7], instead, refers to the PSCell ID to be transferred to the SN as an assumption made by the MN. Still based on our understanding, this information should be derived according to actual PSCell serving the UE that contribution [7] proposes in Proposal 2 to be transferred to the MN within the existing Rel-18 UE Performance IE. Hence, if a UE was served well by an SN on a certain PSCell in the past, the AI/ML model would likely predict that specific SN as the best option for the UE. It is however not clear how the MN would use past information for the inference process in order to predict the best SN to setup NR-DC with. And RAN3 had no discussions about this aspect during the SI.

Proposal 1: RAN3 to agree not to transfer predicted/assumed PSCell ID from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

In a similar way, [7] also proposes to transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers. This is not the current NR-DC behaviour, where an SN may decide that all the QoS flows for which the MN requests SN-terminated bearers will be served via the Master Cell Group (MCG) as MCG bearers, and informs the MN about this decision – refer to QoS Flows Not Admitted List IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message in TS 38.423 [9]. Differently from the assumed PSCell ID (where the PSCell actually selected by the SN after the Addition procedure is not known), the MN may have historical information from the previous SN Addition procedures for the UE from which it may deduce whether a certain SN was a good choice or not by checking the above mentioned QoS Flows Not Admitted List IE. And this information may be used during the inference process in the MN to determine the best SN to add as an input information (if available) locally available. In this way the MN may already be able to predict the best SN to add by “simply” discarding those SNs which, in the past, had less chances to serve SN-terminated bearers via the SCG. Therefore, we think there is no need for the MN to send to the SN the list of QoS flows that are assumed to be served by the SN as SCG bearers.

Proposal 2: RAN3 to agree not to transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

## 2.2 Scenarios for UE performance collection and reporting in NR-DC

During the Rel-19 SI, for mobility optimization in NR-DC, the use case description and standard impacts have been investigated and reported in TR 38.743 [3] clause 5.1, where the standard impacts are mainly on enabling the collection and reporting of UE performance from the SN to the MN over the Xn. In RAN3#126 meeting it was agreed to enhance the existing SN ADDITION REQUEST message in XnAP specification by introducing the Data Collection ID IE identifying a Data Collection Reporting context, that is, a Data Collection Reporting Initiation procedure to configure the collection of the UE performance at the SN side. The exact agreements are as follows [5]:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side. Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

The above agreements follow the outcome of RAN3#125bis meeting, during which RAN3 discussed about the scenarios to be considered for the collection and reporting of UE performance and reached the following agreement [10]:

As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance.

Based on the above agreement, the “basic” scenarios agreed to be considered for triggering the collection of UE performance from SN to MN are related to the successful completion of SN Addition and MN-initiated SN Change. Moreover, it is also part of the agreement that the involved gNBs (that is, MN and SN) need to have an existing Data Collection Reporting Initiation procedure already in place between the two, which is needed for configuring the collection of the UE performance by means of the UE Performance Collection Configuration IE within the DATA COLLECTION REQUEST message. To “link” the SN Addition and MN-initiated SN Change procedures with the Data Collection Reporting Initiation procedure, the Data Collection ID IE is used.

The actual reporting of UE performance from the SN to the MN can be performed via the Data Collection Reporting procedure (namely, within the DATA COLLECTION UPDATE message) already in place between MN and SN, based on the configuration from the MN included in the Data Collection Reporting Initiation procedure (identified by the Data Collection ID IE).

Proposal 3: RAN3 to agree that the reporting of UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message) already in place between MN and SN. No specification impact is foreseen.

In RAN3#125bis meeting the following open issue was captured as an aspect that needs further discussion [10]:

Whether to use the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side or whether to reuse existing DC procedure to configure the collection of UE performance at SN side and a new procedure for the reporting of UE performance needs further discussion.

Even if it is not explicitly mentioned, the above open issue is related to a scenario being different from the ones addressed before (i.e., SN Addition and MN-initiated SN Change) and covered by the agreement reached in RAN3#125bis meeting. This additional scenario has been described in the contribution [11] submitted in RAN3#125bis meeting, where a UE initially single-connected to a source gNB (say gNB1) is handed over to a target gNB (say gNB2) which, in turn, decides to set-up a NR-DC with a third gNB (say gNB3). In such scenario, the target gNB (gNB2) becomes the target MN while the third gNB (gNB3) becomes the target SN, and UE performance collection is configured from the source gNB (gNB1) to be reported by the target MN (gNB2) by means of the Data Collection Reporting Initiation procedure existing between gNB1 and gNB2. There is however the need to discuss which is the impact of the target SN (gNB3) on the UE performance reported from the target MN to the source gNB, as well as how to configure the collection of UE performance at the target SN side and reporting of such UE performance back to the target MN. The target MN should report to the source gNB – via the Data Collection Reporting procedure – either the aggregated UE performance collected at both target MN and target SN or UE performance measured at MN and SN separately. Moreover, it is our understanding that UE performance are typically collected immediately after the handover (source gNB gNB1 to a target gNB gNB2 in the example above) and for a short time window (configured by the source gNB), so it is unclear whether there will be enough time for the target gNB to establish NR-DC with the third gNB (gNB3). Therefore, considering that Rel-19 is the first release to discuss AI/ML-based mobility optimization in NR-DC, new scenarios other than the “basic” SN Addition and MN-initiated SN Change should be deprioritized and, if time allows, their actual feasibility can be discussed already in Rel-19 or in a future release.

Proposal 4: RAN3 to agree that scenarios other than “basic” SN Addition and MN-initiated SN Change are deprioritized. Their actual feasibility could be discussed in Rel-19 only if time allows.

## 2.3 On UE performance metrics in NR-DC

The UE performance metrics to be reported as part of the feedback information after an AI/ML-based action as well as the UE performance collection configuration have been specified in Rel-18 for XnAP [9] clause 9.2.3.179 and clause 9.2.3.186, respectively. Still in Rel-18, UE performance is only for the basic single connectivity case, where the collection and reporting is done at the target node side towards the source node on a per UE granularity. For the case of NR-DC, RAN3 needs to investigate which UE performance metrics are triggered to be collected by the SN after successful SN Addition / Change (MN-initiated) for the concerned UE as a function of the bearer type. Even though most of the metrics in TS 28.558 [12] are specified to be measured in the gNB-DU, in the last meeting and as part of the discussion on split architecture support for Rel-18 use cases RAN3 eventually considered the “gNB-CU-UP-centric” approach, based on which UE performance measurements are performed (and aggregated, for the case of packet delay) at PDCP level (and hence in the gNB-CU-UP) and reported to the gNB-CU-CP over the E1 interface. More specifically, for the packet delay, RAN3 agreements from [4] are as follows:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture. Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture. Stage 3 details of the solution are FFS.

As discussed in our paper on split architecture support [13], the above agreements which are valid for the packet delay suggest that the “gNB-CU-UP-centric” approach should be considered also for the remaining UE performance metrics, that is, Average UE Throughput UL/DL and Average Packet Loss UL/DL. This however requires some kind of SA5 involvement in the definition of measurements that are performed at PDCP level, and we proposed to send a LS to SA5 – a draft LS can be found in the Annex of our contribution in [13]. In addition to the need to define a measurement framework which is valid for all UE performance metrics, it was highlighted in contribution [14] that, in case of NR-DC with split bearers, measurements performed in the gNB-DU (at RLC level) are per leg and hence they cannot be aggregated to derive per DRB measurements (even though it is possible to collect the individual MCG and SCG leg level measurements), due to the fact that there is no common RLC SDU point for MCG and SCG legs and neither MN nor SN knows the transmission/buffer status of data in the other leg. The issue may be solved by relying on PDCP level measurements performed by the node hosting the PDCP entity (either MN or SN), which allow to collect per DRB information in all NR-DC scenarios independently from the bearer type (MCG, SCG, or split). As a general principle, the MN requests the SN to report UE performance only if there is at least a SN-terminated bearer configured at SN Addition or MN-initiated SN Change.

Proposal 5: RAN3 to agree the general principle that MN requests the SN to report UE performance only if there is at least a SN-terminated bearer configured for the UE at SN Addition or MN-initiated SN Change.

Based on the above, we suggest waiting for the outcome of the discussion on UE performance measurements in split architecture – with related SA5 feedback on feasibility of measurements at PDCP – and strive to find commonalities with the corresponding discussions in case of AI/ML-based mobility optimization in NR-DC.

Proposal 6: RAN3 to wait for the outcome of the discussion on UE performance measurements in split architecture and strive to find commonalities with the corresponding discussions in case of NR-DC.

An important issue to discuss is the granularity of the UE performance metrics that the SN needs to report. Our understanding is that UE performance metrics in TS 28.558 [12] are measured on a per DRB per UE granularity. However, the UE performance metrics in clause 9.2.3.179 of TS 38.423 [9] are intended to be reported on a per UE granularity. Thus, we suggest RAN3 further discussing this issue and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios.

Proposal 7: RAN3 to further discuss and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios (e.g., per UE as in Rel-18 or a finer granularity).

As a starting point, we can consider UE performance to be reported on a per UE granularity as done in Rel-18. This because the intended use of such UE performance should be to evaluate whether the AI/ML model decision to setup NR-DC with a certain SN for the UE was beneficial with respect to keeping the concerned UE in single connectivity. And single connectivity is addressed in Rel-18 where only per UE UE performance are reported. To derive per UE UE performance metrics starting from the corresponding per DRB per UE measurements the RAN3 common understanding from RAN3#124 meeting can be considered, that is, per UE UE performance is derived as the average value of per DRB measurements of the UE performance metrics [15]:

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

Proposal 8: RAN3 to agree that, as a starting point, UE performance is reported from the SN to the MN on a per UE granularity.

## 2.4 Conditions governing collection of UE performance at SN side

Conditions to start the UE performance collection in NR-DC

In the last meeting, contribution [16] proposed to capture an additional condition, i.e., SCG Activation, to be evaluated by the SN – along with the condition related to the successful S-NG-RAN Node Addition, as per the latest version of the XnAP BLCR – in order to understand when to start the collection of UE performance to be then reported to the MN. More in detail, the current version of the XnAP BLCR captures the following text [17]:

### 8.3.1 S-NG-RAN node Addition Preparation #### 8.3.1.1 General The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same. *** skip non-relevant text *** Interaction with the Activity Notification procedure Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level. Interaction with the Data Collection Reporting Initiation and the Data Collection Reporting procedures: If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful S-NG-RAN node addition, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

As per TS 37.340 [18] clause 7.13, an activation/deactivation mechanism of SCG is supported to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. Moreover, the MN can configure the SCG as (de-)activated upon PSCell addition (among other procedures such as, PSCell change, RRC Resume or handover), and it is also allowed that the SN independently (de-)activates the SCG. While the SCG is deactivated, there is no transmission via SCG RLC bearers, meaning that UP data cannot be transferred over the SCG and corresponding UE Performance measurements cannot be collected. As a consequence, RAN3 should discuss whether UE Performance measurements for SCG bearers can be collected only after SCG resources are activated.

Proposal 9: RAN3 to agree that UE performance at the SN side can be collected after successful S-NG-RAN node addition and SCG activation.

Conditions to stop the UE performance collection in NR-DC

Current TS 38.423 clause 8.4.13.2 captures, for the case of UE in single connectivity, the exit conditions to be considered by the target node in order to evaluate when to stop the collection of UE performance [9]:

### 8.4.13 Data Collection Reporting Initiation ### **** Skip non-relevant text **** #### 8.4.13.2 Successful Operation **** Skip non-relevant text **** If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell. The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message. **** Skip non-relevant text ****

As already specified in Rel-18 for the AI/ML mobility optimization, RAN3 should also discuss about the exit conditions to be evaluated by the SN in order to stop the collection of UE performance at the SN side. In light of what we proposed in Section 2.1, for the AI/ML-based NR-DC we think that at least the following conditions should be introduced so that the SN can understand when to stop the collection of the UE performance:

- the time since NR-DC was successfully setup for the UE with activated SCG is equal to the value configured to the SN as part of the preceding Data Collection Reporting Initiation procedure between MN and SN;

- the NR-DC for the concerned UE is released (in both cases of SN-initiated and MN-initiated);

- the SN Change procedure is performed.

In Rel-18 and for the single connectivity case, UE performance can be collected and reported periodically during the collection time duration (the latter identified via the Collection Time Duration for UE Performance IE in XnAP) which start at successful handover. Based on this, for the AI/ML-based mobility in NR-DC, it should be discussed what is the SN behaviour in case the SCG is deactivated during the periodic collection and reporting of UE performance within the configured collection time duration. For the UE performance collection, since Rel-18, there is the common understanding in RAN3 that UE performance metrics are collected and reported within a time interval which is typically very short, therefore, in case the SCG is deactivated during this short collection time duration, we believe that the SN should simply stop the collection of UE performance and report the collected measurements to the MN at the first available DATA COLLECTION UPDATE message, without the need to resume the collection process in case the SCG is re-activated during the residual collection time duration which is supposed to be extremely short and close to its expiration, hence without adding further complexity to the UE performance collection and reporting process.

Proposal 10: RAN3 to agree that exit conditions to be evaluated by the SN to stop the collection of UE performance at the SN side need to be specified.

Proposal 11: RAN3 to agree the following list of exit conditions to stop the collection of UE performance at the SN side:

- the time since NR-DC was successfully setup for the UE with activated SCG is equal to the value configured to the SN as part of the preceding Data Collection Reporting Initiation procedure between MN and SN;

- the NR-DC for the concerned UE is released (in both cases of SN-initiated and MN-initiated);

- the SN Change procedure is performed;

- the SCG is deactivated during the collection time duration.

## 2.4 UE trajectory feedback in NR-DC

In the Rel-18 mobility optimization use case, it is the source gNB to predict the cell-based UE trajectory and, based on that information, to choose the next hop target gNB. And as in the current Rel-18 TS 38.423 [9], the source gNB can configure the target gNB to collect and report the measured UE trajectory via the Data Collection Reporting (Initiation) procedures, to help the source gNB evaluating the AI/ML-driven mobility decision. The source gNB configures the collection time duration and number of visited cells as triggers to stop the collection of measured UE trajectory information. And the target gNB will feedback the measured UE trajectory, including the list of cells that the UE visited and the corresponding time spent in each cell. In the NR-DC, the MN may predict the SN for DC, but the PSCell and corresponding change strategies are decided by the SN. This means that the MN is not aware of which PSCell the SN will choose for the UE as well as possible PSCell changes. With the measured UE trajectory among PSCells, the MN may evaluate whether the previous decision on the SN selection and addition via AI/ML was accurate or AI/ML model’s optimization is needed.

In our opinion, the current measured UE trajectory collection configuration related signalling can be the baseline for further enhancements aimed at including the configuration for reporting information about the measured UE trajectory in PSCells.

Proposal 12: RAN3 to agree that the current measured UE trajectory collection configuration related signalling is further enhanced to include the configuration for reporting information about UE trajectory in PSCells.

If Proposal 12 above is agreed, then the same considerations as in Section 2.4 for starting and terminating the collection and reporting of the measured UE trajectory across PSCells could be considered.

The TP for the XnAP BLCR for AI/ML for NG-RAN in the Annex reflects the above proposals.

# 3. Conclusion

Based on the above discussion, we have the following proposals:

Assistance information for SN configuration

Proposal 1: RAN3 to agree not to transfer predicted/assumed PSCell ID from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

Proposal 2: RAN3 to agree not to transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

Scenarios for UE performance collection and reporting in NR-DC

Proposal 3: RAN3 to agree that the reporting of UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message) already in place between MN and SN. No specification impact is foreseen.

Proposal 4: RAN3 to agree that scenarios other than “basic” SN Addition and MN-initiated SN Change are deprioritized. Their actual feasibility could be discussed in Rel-19 only if time allows.

On UE performance metrics in NR-DC

Proposal 5: RAN3 to agree the general principle that MN requests the SN to report UE performance only if there is at least a SN-terminated bearer configured for the UE at SN Addition or MN-initiated SN Change.

Proposal 6: RAN3 to wait for the outcome of the discussion on UE performance measurements in split architecture and strive to find commonalities with the corresponding discussions in case of NR-DC.

Proposal 7: RAN3 to further discuss and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios (e.g., per UE as in Rel-18 or a finer granularity).

Proposal 8: RAN3 to agree that, as a starting point, UE performance is reported from the SN to the MN on a per UE granularity.

Conditions governing collection of UE performance at SN side

Proposal 9: RAN3 to agree that UE performance at the SN side can be collected after successful S-NG-RAN node addition and SCG activation.

Proposal 10: RAN3 to agree that exit conditions to be evaluated by the SN to stop the collection of UE performance at the SN side need to be specified.

Proposal 11: RAN3 to agree the following list of exit conditions to stop the collection of UE performance at the SN side:

- the time since NR-DC was successfully setup for the UE with activated SCG is equal to the value configured to the SN as part of the preceding Data Collection Reporting Initiation procedure between MN and SN;

- the NR-DC for the concerned UE is released (in both cases of SN-initiated and MN-initiated);

- the SN Change procedure is performed;

- the SCG is deactivated during the collection time duration.

Other aspects

Proposal 12: RAN3 to agree that the current measured UE trajectory collection configuration related signalling is further enhanced to include the configuration for reporting information about UE trajectory in PSCells.

# 4. Reference

- RP-250326, Revised New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- R3-250664, Discussion on AIML based Mobility Optimization for NR-DC, CMCC, CATT, ZTE Corporation, Lenovo

- R3-250459, AI/ML support for NR-DC, Ericsson, Jio Platforms (JPL), InterDigital, FiberCop

- 3GPP RAN3#125 Chairman’s notes (RAN3_125_agenda_20240823EOM1.doc)

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), V18.4.0 (2024-12)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-245608, Procedures for AI/ML Mobility Optimization in NR-DC, Nokia, Jio

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, V19.2.0 (2025-01)

- R3-251998, (Draft LS to SA5 on UE performance) Discussion on split architecture support for Rel-18 “AI/ML for NG-RAN” use cases, Huawei

- R3-250575, Discussion on Rel-18 UE Performance measurements , Nokia

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)

- R3-250573, Procedures for AI/ML Mobility Optimization in NR-DC, Nokia

- R3-251544, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange

- 3GPP TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage-2, V18.4.0 (2024-12)

# 5. Annex – TP for BLCR to TS 38.423 for AI/ML NR-DC (based on R3-251544)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

Interaction with the Data Collection Reporting Initiation and the Data Collection Reporting procedures:

If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful S-NG-RAN node addition and SCG activation, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the slice-specific information reporting is requested, the Slice To Report for Data Collection List IE shall be included.

*** skip non-relevant text ***

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting, or for the collection and reporting of the UE trajectory across PSCells in case NG-RAN node2 acts as S-NG-RAN node for the UE. NG-RAN node2 shall report the UE trajectory, or the UE trajectory across PSCells in case of NR-DC, only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2;

-	the time since NR-DC was successfully setup with the NG-RAN node2 (acting as S-NG-RAN node) for the UE with activated SCG is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the NR-DC with the NG-RAN node2 (acting as S-NG-RAN node) for the UE is released;

-	the SCG of the NG-RAN node2 (acting as S-NG-RAN node) is deactivated during the Collection Time Duration for UE Trajectory IE.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	the time since NR-DC was successfully setup with the NG-RAN node2 (acting as S-NG-RAN node) for the UE with activated SCG is equal to the value of the Collection Time Duration for UE Performance IE;

-	the NR-DC with the NG-RAN node2 (acting as S-NG-RAN node) for the UE is released;

-	the SCG of the NG-RAN node2 (acting as S-NG-RAN node) is deactivated during the Collection Time Duration for UE Performance IE.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE.

*** skip non-relevant text ***

-	the Predicted Slice Available Capacity IE, if the tenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if

- the Slice To Report List for Data Collection IE is included for the cell, and

- the measurement object is admitted by NG-RAN node2.

- the Measured UE Trajectory in SN IE, if the eleventh bit, "Measured UE Trajectory in SN" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity, Eleventh Bit = Measured UE Trajectory in SN. Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory, Seventh Bit = Measured UE Trajectory in SN. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >>Slice Measurement Initiation Result | O |  | 9.2.3.x1 | List of measurement objects that failed to be initiated per slice. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally includes the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | Slice Available Capacity 9.2.2.55 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| >>Measured UE Trajectory in SN | O |  | Measured UE Trajectory 9.2.3.182 | It contains information about PSCells that the UE has connected to when operating in NR-DC. |  |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.182	Measured UE Trajectory

The Measured UE Trajectory IE contains information related to NG-RAN cells where the UE connected after being handed over to the target NG-RAN node, or information related to PSCells where the UE connected to when operating in NR-DC.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measured UE Trajectory |  | 1 |  | List of cells where the UE connected, in chronological order. Most recent information is added to the top of this list. |
| >Measured UE Trajectory Item |  | 1..<maxnoofCellsTrajectory> |  |  |
| >>Measured Trajectory Cell Information | M |  | 9.2.3.183 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTrajectory | Maximum number of cells that can be reported for UE trajectory. Value is 16. |

    - 9.2.3.183	Measured Trajectory Cell Information

The Measured Trajectory Cell Information contains information related to the NG-RAN cells where a UE connected after being handed over to the target NG-RAN node, or information related to PSCells where the UE connected to when operating in NR-DC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Measured Trajectory Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 | Indicates an NR Cell Identity. |
| >>Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell If the duration is more than 4095s, this IE is set to 4095. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.185	UE Trajectory Collection Configuration

This IE contains configurations for UE trajectory collection after successful handover, or configurations for the collection of UE trajectory across PSCells when the UE is in NR-DC.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Trajectory | M |  | INTEGER (1..4096, …) | Time duration starting at successful handover within which UE Trajectory Information is collected. It also indicates the time duration starting at successful SN Addition and SCG activation within which the UE Trajectory Information across PSCells is collected when the UE operates in NR-DC. Unit: second |
| Number of Visited Cells | O |  | INTEGER (1..16, …) | Maximum number of intra-node visited cells. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-NRPPaPositioningInformation,

id-SliceToReportForDataCollection-List,

id-PredictedSliceAvailableCapacity,

id-SliceMeasurementInitiationResult,

id-measuredUETrajectorySN,

maxEARFCN,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofRSPPQoSFlows,

maxnoofFailedSliceMeasObjects

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- U

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						NG-RANnodeUEXnAPID,

uEPerformance								UEPerformance			OPTIONAL,

measuredUETrajectory						MeasuredUETrajectory	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-measuredUETrajectorySN				CRITICALITY ignore	EXTENSION MeasuredUETrajectory				PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

*** Unmodified Text Omitted ***

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofFailedSliceMeasObjects				INTEGER ::= FFS

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

*** skip non-related text ***

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-Future-Coverage-Modification-List																ProtocolIE-ID ::= xx1

id-SliceToReportForDataCollection-List																ProtocolIE-ID ::= xx2

id-PredictedSliceAvailableCapacity																	ProtocolIE-ID ::= xx3

id-SliceMeasurementInitiationResult																	ProtocolIE-ID ::= xx4

id-measuredUETrajectorySN																			ProtocolIE-ID ::= xx5

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>