3GPP TSG-RAN WG3 Meeting #127bis	R3-252031

Wuhan, China, 07 - 11 April, 2025

Agenda Item:	21.3

Source:	Ericsson

Title:	Discussion on XR Bitrate control over F1

Document for:	Approval

# 1	Introduction

The WID of R19 XR has been updated in [1] to add the following objective:

- Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

This paper provides our view on the above objective.

# 2	Background

Network exposure of per QoS flow/per DRB congestion information was discussed in RAN2 [2]. It was agreed that​

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

RAN2 has informed RAN3 of its agreement on REL-19 XR UL bit rate control and requests feedback from RAN3 in the liaison copied below [3]:

1. Overall Description: RAN2 has made the following agreement regarding a solution on UL bit rate control Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. 2. Actions: To SA4, RAN3: ACTION: RAN2 kindly asks SA4 and RAN3 to take above into consideration and provide feedback on the agreement provided, if any.

RAN3 has replied to RAN2 in [4] agreeing that both options have an impact on the F1 interface. RAN3 will further discuss whether gNB-DU can support per QoS flow data bit rate control when TU is available.

Observation 1: A discussion confirming that gNB-DU can support per QoS Flow data bit rate control has still not taken place and any agreement towards such approach has not been confirmed

Furthermore, RAN2 has made the following agreements and a LS was sent to SA2 and RAN3 in [5]:

Assistance information - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. - We may revisit UAI option based on SA2/RAN3 reply

## 2.1	NGAP and XnAP Impact

First of all, we note that RAN2 have recently agreed that the CN can provide information on XR rate control over NGAP, indicating which QoS flow within a DRB can be rate adaptive (e.g., recommended to be reduced or throttled) during the setup of the PDU session.

It should be noted that such indication does not imply that rate control can be achieved with a per QoS flow granularity. Indeed, the CN is not aware of how the RAN will form DRBs (i.e. which QoS Flows the RAN will group within each RB), hence the only way the CN has to communicate a specific per service treatment, such as rate control, is by including per QoS Flow information. This is similar to mechanisms specified for e.g. ECN marking and QoS Monitoring, where the CN indicates to the RAN e.g. for which QoS Flow UL/DL Congestion Information shall be reported, or for which QoS Flow RAN delay measurements shall be reported. In both the examples mentioned, the reporting of information is however done per DRB.

To support this in our RAN3 specification, the indication of which QFI can be subject to rate control, and for which associated direction (i.e., UL or DL) can be indicated as part of the QoS Flow QoS Parameters IE. See example below. Note that XnAP may be affected as well, for handover scenarios,

#### 9.3.1.12 QoS Flow Level QoS Parameters This IE defines the QoS parameters to be applied to a QoS flow. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. CHOICE QoS Characteristics; M; -. //skipped rows//. PDU Set QoS Parameters; 0..1; YES; ignore. >UL PDU Set QoS Information; O; PDU Set QoS Information 9.3.1.264; -. >DL PDU Set QoS Information; O; PDU Set QoS Information 9.3.1.264; -. Indication of Bitrate Adaptation; O; ENUMERATED (UL, DL, Both, …); YES; ignore.

Observation 2: the NGAP protocol needs to be enhanced to indicate which QoS flow can be rate adaptative for a specific direction (UL/DL, both). XnAP may be affected similarly.

The above change is feasible, and similar to the QoS Monitoring Request IE added within the QoS Flow Level QoS Parameters IE or similar to the ECN Marking or Congestion Information Reporting Request IE added per QoS Flow. We propose to agree on the above changes with FFS pending SA2’s update to their specification. LS to RAN2 can be sent.

Proposal 1: RAN3 to add indication of Bit rate Adaptation in the QoS Flow Level QoS Parameters in NGAP and XnAP specs.

## 2.2	F1AP enhancements

Once the NG-RAN decides on which QoS flows it can enable bitrate control recommendation , based on the assistance information received from CN described in previous section and gNB implementation, it informs the UE, so that UE performs the rate adaptation. Based on RAN2 discussion, two options are available:

- QoS flows are indicated by MAC CE;

- Or QoS Flows are indicated by RRC while MAC CE is per DRB.

Based on discussions from last meeting, both options require F1 enhancements, as there is no dedicated F1AP IE for the purpose of recommended rate control indication, either on a per QoS Flow or per DRB basis. Such indication is motivated so that the gNB-DU, which is the entity that handles radio resource management, can make decisions to assist the UE’s application to adapt the bitrate by sending the indication (per QoS flow or per DRB) to UE via MAC-CE.

Before discussing which option to choose, the problem highlighted by RAN3 in the LS in [4] shall be tackled, namely can the gNB-DU support per QoS flow data bit rate control?

The short answer to this question is “no”. The reasons are explained below.

When a UE context is created in a gNB, the gNB-CU signals to the gNB-DU a F1: UE context Setup Request in which the gNB-CU states which DRBs should be created for the UE. This is described in the F1 DRB to Be Setup List IE, where for each DRB the gNB-DU assigns a DRB QoS, which is the set of QoS rules according to which traffic for this DRB will be handled. Traffic for all the QoS Flows that are multiplexed to a DRB will enter the PDCP layer and from this point on they will be numbered and handled as if they belonged to a single traffic stream. Namely, the gNB-DU receiving PDCP traffic will have no knowledge of whether a PDU within a DRB traffic stream belongs to one or another QoS Flow.
It is true that in Rel15, for each DRB described in the DRB to Be Setup List IE a Flows Mapped to DRB List was added. However, this list is provided to the gNB-DU only for information, to enable the gNB-DU to derive the best RRM handling for the DRB traffic, based on the QoS requirements of each QoS Flow.

Observation 3: A gNB-DU is not able to distinguish between different QoS flow traffic for QoS Flows that are mapped to the same DRB. The gNB-DU is only able to recognize and manage DRB traffic, i.e. traffic resulting from multiplexing multiple QoS Flows added to the DRB.

Of course, a per QoS Flow traffic handling at the gNB-DU could be achieved if a single QoS Flow was mapped to the DRB. In this case, the gNB-DU would manage the DRB traffic, which in turn would mean to manage the single QoS Flow traffic.

Observation 4: A gNB-DU can manage and influence single QoS Flow traffic only if there is a one to one mapping between the QoS Flow and a DRB.

The above can be proven when looking at features like L4S, where the following happens:

- The gNB-CU receives from the CN a per QoS Flow indication of ECN Marking or Congestion Information Reporting Request, tasking the gNB to monitor traffic for the QoS Flow and report to the CN ECN marking or congestion information

- The gNB-CU decides how to multiplex QoS Flows into DRBs for a given UE and it signals to the gNB-DU an ECN Marking or Congestion Information Reporting Request per DRB. TS38.473 procedure text stated the following:
“If the ECN Marking or Congestion Information Reporting Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it accordingly for the specific DRB.”
Hence, if the ECN Marking or Congestion Information Reporting Request was included over the NG from the CN to the gNB for a QoS Flows, and if the QoS Flow is multiplexed in a DRB, the gNB-CU includes the ECN Marking or Congestion Information Reporting Request for that DRB towards the gNB-DU

- In reply to the received request per DRB, the gNB-DU reports over the F1-U a per DRB UL/DL Congestion Information to the gNB-CU-UP. Indeed, the details of this can be found in TS38.425, where the following is specified for example in the case of UL congestion information reporting:

#### 5.5.3.61 UL Congestion Information Description: For the cases of ECN marking Request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a DRB.

In conclusion, any monitoring and control of user plane traffic at the gNB-DU can be done on a per DRB level.

Let´s now address the question of how to indicate rate control, if per QoS Flow or per DRB.

For the case of QoS Flow indicated per RRC and DRB via MAC CE, a per DRB ID indication needs to be indicated in the UE CONTEXT SETUP/MODIFICATION REQUEST message from gNB-CU to gNB-DU to indicate that the DRB can be rate controlled. This approach is correct and in line with the ability of a gNB-DU to rate control only at a per DRB level.

For the case of QoS Flow indication via MAC-CE, a per QFI indication would need to be indicated in the UE CONTEXT SETUP/MODIFICATION REQUEST message from gNB-CU to gNB-DU to indicate that the QoS Flow can be rate controlled. However, as it was explained such approach is not feasible because the gNB-DU cannot distinguish nor rate control traffic on a per QoS Flow basis, unless there is a one to one mapping between QoS Flow and DRB.

Observation 5: The gNB-DU is not able to monitor and control traffic on a per-QoS flow level for QoS flows multiplexed in a DRB to provide bit rate recommendation. Any monitoring and control of user plane traffic at the gNB-DU can be done on a per DRB level.

Observation 6: The option of per QoS Flow rate indication via MAC-CE can only work if there is one-to-one mapping between the DRB and the QoS Flow. However, this case can be covered with a per DRB bit rate recommendation.

Secondly, it is unclear from the RAN2 LS and agreements, whether the option of performing bit rate control indication per QoS Flow at the gNB-DU should be for GBR or non GRB QoS Flows. As argued above, this question would translate to whether rate adaptation and rate recommendation is done on a per GBR DRB or if it also covers non GBR DRBs. 
If the intention is to rate adapt non GBR DRBs, it is questionable if any rate recommendation can make sense, given that there are no real rate targets to follow for non GBR services. Therefore, the option of per DRB adaptation would only make sense for GBR DRBs.

Observation 7: It is still unclear if the option of per DRB rate adaption should be for GBR or non GBR DRBs. RAN3 to consider that only GBR DRBs are in scope and should confirm this with RAN2.

From the above, it can be concluded that the only feasible option is to support the solution of per RRC QoS flow indication and DRB via MAC CE.  This option can also cover the case of per QoS Flow indication, in case there is a one to one mapping between QoS Flow and DRB.

Proposal 2: RAN3 t agree and reply to RAN2 that the only feasible option is to support the solution of per RRC QoS flow indication and DRB via MAC CE.  This option can also cover the case of per QoS Flow indication, in case there is a one to one mapping between QoS Flow and DRB

A reply LS is available in annex.

# 3	Conclusion

In this contribution we discuss the questions to RAN3 from the RAN2 LS [2].

Observations and proposals are listed below:

Observation 1: A discussion confirming that gNB-DU can support per QoS Flow data bit rate control has still not taken place and any agreement towards such approach has not been confirmed.

Observation 2: the NGAP protocol needs to be enhanced to indicate which QoS flow can be rate adaptative for a specific direction (UL/DL, both). XnAP may be affected similarly.

Proposal 1: RAN3 to add indication of Bit rate Adaptation in the QoS Flow Level QoS Parameters in NGAP and XnAP specs.

Observation 3: A gNB-DU is not able to distinguish between different QoS flow traffic for QoS Flows that are mapped to the same DRB. The gNB-DU is only able to recognize and manage DRB traffic, i.e. traffic resulting from multiplexing multiple QoS Flows added to the DRB.

Observation 4: A gNB-DU can manage and influence single QoS Flow traffic only if there is a one to one mapping between the QoS Flow and a DRB.

Observation 5: The gNB-DU is not able to monitor and control traffic on a per-QoS flow level for QoS flows multiplexed in a DRB to provide bit rate recommendation. Any monitoring and control of user plane traffic at the gNB-DU can be done on a per DRB level.

Observation 6: The option of per QoS Flow rate indication via MAC-CE can only work if there is one-to-one mapping between the DRB and the QoS Flow. However, this case can be covered with a per DRB bit rate recommendation.

Observation 7: It is still unclear if the option of per DRB rate adaption should be for GBR or non GBR DRBs. RAN3 to consider that only GBR DRBs are in scope and should confirm this with RAN2.

Proposal 2: RAN3 t agree and reply to RAN2 that the only feasible option is to support the solution of per RRC QoS flow indication and DRB via MAC CE.  This option can also cover the case of per QoS Flow indication, in case there is a one to one mapping between QoS Flow and DRB

# 4	References

[1] RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), 3GPP TSG RAN Meeting #107.

[2] R2-2213226, Reply LS to SA2 on XR, RAN2

[3] R3-250012, LS on XR UL bit rate control, RAN2

[4] R3-250805, Reply LS on XR UL Rate Control, RAN3

[5] R3-251513/R2-2501347 LS on uplink rate control, Nokia

# 5	LS to RAN2

3GPP TSG-RAN WG3 Meeting #127bis	R3-25xxxx

Wuhan, China, 7 – 11 April 2025

# Title:	[Draft] RAN3 feedback on support of XR UL bit rate control

# Response to:	Reply LS on XR UL bit rate control (R3-251513/R2-2501347)

# Release:	Release 19

# Work Item:	NR_XR_Ph3-Core

Source:	[Ericsson] to be RAN3

To:	RAN2, SA4

Cc:	SA2

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 has discussed the support of XR UL bit rate control would like to provide the following feedback to RAN2 and SA2:

- It is technically feasible to enhance the NGAP protocol to indicate which QoS flow(s) can be rate adaptative for a specific direction (UL/DL/Both) as part of the QoS Flow QoS Parameters. The XnAP specification may also be affected similarly.

- A gNB/gNB-DU is not able to monitor on a per QoS flow basis, if more than one QoS flow is multiplexed in a DRB, for rate indication. The option of per QoS Flow rate indication via MAC-CE can only work if there is one-to-one mapping between the DRB and QoS Flow.

- Rate indication from gNB to the UE on a per QoS flow by RRC while MAC CE is per DRB is the only feasible option covering both the case of one to many QoS Flow to DRB mapping (where per DRB rate adaptation is possible) and one to one QoS Flow to DRB mapping (where per DRB rate adaptation coincides with per QoS Flow rate adaptation)

- It is still unclear to RAN3 if the option of per DRB rate adaption should be for GBR or non GBR DRBs. A rate adaptation indication can only be derived for GBR DRBs.

2. Actions:

To RAN2, SA2:

ACTION: 	RAN3 kindly asks RAN2 and SA2 to take the above feedback into consideration.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting #128		May 19-23, 2025			Malta, MT

TSG-RAN3 Meeting #129		August 25-28, 2025			Bangalore, India