3GPP TSG-RAN WG3 Meeting #127 Bis	             R3-252035

Wuhan, China, 7 – 11 April 2025

Agenda Item:	20.2

Source:	CMCC

Title:	Support Assisted AI/ML Positioning

Document for:	Discussions & Approval

# Introduction

In the RAN3 127 meeting, the following agreement for Case 3a has been made [1]:

[Agreement] Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. To be continued: Stage 2 (TS 38.305) text proposals for case 3a data collection. Companies are encouraged to consolidate solutions, if possible.

# Discussions on Assisted AI/ML Positioning

AI/ML-assisted positioning utilizes AI/ML models to produce intermediate measurement statistics that play a critical role in determining location. These statistics can include metrics such as the probability of a signal being Line-of-Sight (LOS) or Non-Line-of-Sight (NLOS), the angle of arrival (AoA) or departure (AoD) of signals, and time-of-arrival data (ToA). The NG-RAN Node requires data for AI/ML assisted positioning model training, testing, validation and monitoring. Such data samples includes input and output pairs, where the input data is the raw measurements or channel information between UE and NG-RAN Node (also called as Part A), and the output data is the intermediate measurement statistics for positioning (also called as Part B).  Note, in the document gNB and NG-RAN Node are used interchangeablely.

For Case 3a, the input data is the raw measurements or channel information between UE and NG-RAN Node (also called as Part A), and the output data is the intermediate measurement statistics for positioning (also called as Part B), which could be derived from the UE location by LMF or NG-RAN Node.

### Data Collection for gNB

#### Opportunistic Data Collection

A UE could be positioned in the legacy positioning scenarios as following:

- A Network Induced Location Request (NI-LR) is initiated by a serving AMF to locate a UE. This request is typically made for regulatory purposes, such as verifying the UE's location during an emergency call, confirming the UE's location within a specific country or international area especially in NR satellite access scenarios.

- A Mobile Terminated Location Request (MT-LR) occurs when a LCS client or an Application Function (AF), either external or internal to the serving Public Land Mobile Network (PLMN), sends a request for the location of a target UE. It facilitates the applications involving tracking, location-based services, or regulatory purposes, which needs the UE’s location as external services.

- A Mobile Originated Location Request (MO-LR) occurs when a UE sends a request to its serving PLMN for location-related information. This request is initiated by the UE itself, typically for purposes such as providing its location to a service or application, or for regulatory compliance.

Figure 1 Unlink positioning procedure (Section 8.14 in TS 38.305 [2])

The serving and assisting gNBs could acquire part A opportunistically during the unlink positioning procedure as shown in the Step 7 in Figure 1. However, in order to acquire part B, the gNB needs to inform LMF that part B is needed. Part B of the data sample could be the UE’s location, or intermediate measurements that are derived by LMF.

In the Step 8, NRPPa Measurement Response, an indicator could be included by the serving or assisting gNB, which lets the LMF be aware that the part B needs to be sent to the gNB following the message in the Step 8.

During legacy uplink UE positioning, the serving and assisting gNB can add a Part B Transfer indicator in the NRPPa MEASUREMENT Response.

Upon receiving a Part B Transfer indicator in the NRPPa MEASUREMENT Response, the LMF sends a class 2 NRPPa Data Transfer message to the corresponding gNB, which includes the Part B of the data sample that the gNB could obtain opportunistically.

Figure 2 Part B transfer in opportunistic data collection

#### Proactive Data Collection

The proactive data collection happens when the gNB needs more data samples for the model training, testing and validation, etc., which means that the data samples that are obtained opportunistically cannot satisfy the gNB’s need.

Question 1: Which node should trigger the proactive data collection?

gNB is the entity which needs data samples and could realize the data samples obtained opportunistically are not enough. Therefore, it would be the gNB triggers the proactive data collection.

Question 2: Which node should provide the additional Part B data samples other than the ones that are obtained during the legacy uplink UE positioning?

The LMF is the entity that provides the additional Part B data samples to the gNB. It is ideal for the gNB to interact with LMF directly.

##### Solution I

For Solution I, NRPPa procedures could be considered with less impact on the core network functions. The gNB shall inform the LMF that the data samples are needed. Such request from a gNB to the LMF shall follow the principle that a NRPPa transaction is always initiated by the LMF.

Solution I includes a class 1 NRPPa Data Collection Solicitation Request/Response message, optionally followed by a class 2 NRPPa Data Collection Solicitation Update message, that are used for the NG-RAN Node to inform the LMF its data collection intention. On the other hand, a class 2 NRPPa Data Collection Report message is used by LMF to send relevant data to the requesting NG-RAN Node. The procedures are shown in Figure 4:

- Step 1: The LMF could let the gNB be aware that it can provide data samples to the gNB in the NRPPa Data Collection Solicitation Request Message.

- Step 2: The gNB sends the data collection intention in the NRPPa Data Collection Solicitation Response Message, which could include the number of data samples needed, whether it is a periodic request or not, the type of UEs for data collection, etc. The data collection intention could be updated in the NRPPa Data Collection Report Message as a class 2 message.

- Step 3: LMF side UE selection: based the data collection intention sent from the gNB or maybe other neighbouring gNBs, the LMF could figure out that proactive UE positioning needs to be triggered to collect more data samples. The LMF retrieves from the AMF regarding the UEs served by the AMF and located in “Area Of Interest”. For each returned UE, the LMF checks with its associated UDM and determine whether the UE has privacy restriction of location positioning. It uses the same mechanism as the LMF side initiated data collection for model training at the LMF as the steps 1—5 shown in Figure 3.

Figure 3 Data collection by LMF (Section 6.22.3 in TS 23.273 [3])

- Step 4: UE positioning could be initiated from the LMF.

- Step 5: LMF obtains the SRS configuration from the serving NG-RAN Node. If the uplink positioning is used in Step 4, then this step can be merged with Step 4.

- Step 6: LMF sends a class 2 NRPPa Data Collection Report to the NG-RAN Node. There are two types of NG-RAN Nodes, one is serving and assisting NG-RAN Node if in Step 4 uplink positioning is used, another is neighboring NG-RAN Nodes which are reachable by UE’s SRS. Since the LMF knows about the Step 4, whether uplink positioning is used, and whether the NG-RAN Node is involved in the uplink positioning as serving or assisting NG-RAN Node, the LMF can include the SRS configuration, measurement ID and timestamp in the Step 4 uplink positioning, and ground truth label. The following rules could be applied:

  - Ground truth label is always included for all scenarios.

  - If the uplink positioning is used in Step 4, the NG-RAN Node is serving or assisting NG-RAN Node, then SRS configuration does not need to be included. Measurement ID and timestamp need to be included to match ground truth label (part B) to the part A data.

  - If the uplink positioning is used in Step 4, the NG-RAN Node is not serving or assisting NG-RAN Node, then SRS configuration needs to be included. Measurement ID and timestamp do not need to be included.

  - If other positioning methods are used in Step 4, SRS configuration needs to be included.

- Step 7: UL SRS measurement is conducted to obtain the part A data if it was not conducted before in Step 4. The Step 3 to Step 7 could be carried out periodically dependent on what is included in the NRPPa Data Collection Solicitation Response message.

- Step 8: A class 2 NRPPa Data Collection Solicitation Update message could be sent from the NG-RAN Node to LMF to update its data collection intention, could be an update to the current request, or a cancellation of the current request.

Figure 4 Solution I

﻿A Class 1 NRPPa Data Collection Solicitation Request message could be initiated by the LMF to solicitate the NG-RAN Node’s data collection need.  The NG-RAN Node responds NRPPa Data Collection Solicitation Response to notify the LMF its data collection need, which could be a one-time request or periodic requests.

NG-RAN Node could suggest the interested UE, or type of UEs for data collection to the LMF in the Data Collection Solicitation Response.

LMF is the entity to perform the final UE selection by using the same mechanism as LMF-side model data collection.

LMF initiates a class 2 NRPPa Data Collection Report message to the NG-RAN Node, which must include the ground truth label, optionally SRS configuration, Measurement ID, timestamp dependent of the role of the NG-RAN Node.

##### Solution II

Solution II also uses NRPPa procedures, similar to Solution I. Solution II includes a class 2 NRPPA Data Collection Supported message initiated from the LMF, which is used to notify the NG-RAN Node that data collection is supported by the LMF. The NG-RAN Node could sit on the information and send a class 1 NRPPa Data Collection Request or NRPPa Data Collection Subscription message, the latter could be used as a long-term periodic data collection request. The class 1 NRPPa Data Collection Request is followed by a or multiple NRPPa Data Collection Response(s) that include the relevant data that was discussed previously. The procedures are shown in Figure 5.

Figure 5 Solution II

A Class 2 NRPPa Data Collection Supported message could be initiated by the LMF to inform the NG-RAN Node that data collection is supported at the LMF.

A class 1 NRPPa Data Collection Request or NRPPa Data Collection Subscription message is sent from the NG-RAN Node, the latter could be used as a long-term periodic data collection request.

The class 1 NRPPa Data Collection Request is followed by one or multiple NRPPa Data Collection Response(s) which must include the ground truth label, optionally SRS configuration, Measurement ID, timestamp dependent of the role of the NG-RAN Node.

##### Solution III

In Solution III, NG-RAN Node interacts with AMF to initiate the data collection request. NG-RAN Node could provide a list of UEs to the AMF that it wants data collection in the NGAP Data Collection Request Message. It could also be a longer-term periodic data collection request, which is expressed in a NGAP Data Collection Subscription message. Additionally, AMF could selection UEs in the area of interest. Based on the selected UE(s), the AMF selects the LMF and start Nlmf_Location_DetermineLocation request to the selected LMF in the Step 4. Step 5 and Step 6 are similar to Solution II. In the Nlmf_Location_DetermineLocation response in Step 7, the UE’s location and other information that correlates part A and part B data shall also be returned. In the Step 8, NGAP Data Collection Response Message or NGAP Data Collection Notification Message that include the relevant data that was discussed previously.

A Class 1 NGAP Data Collection Request message could be initiated by the NG-RAN Node to the AMF that data collection is needed, which could include a list of target UEs. AMF selects UEs to collect data from.

For long-term periodic data collection request, a NGAP Data Collection Subscription message could be initiated by the NG-RAN Node to the AMF, which could include notification criteria.

The class 1 NGAP Data Collection Request is followed by one or multiple NRPPa Data Collection Response(s) which must include the ground truth label, optionally SRS configuration, Measurement ID, timestamp dependent of the role of the NG-RAN Node.

##### Comparison of Three Solutions

| Solution | I | II | III |
| --- | --- | --- | --- |
| Opportunistic | It can support, if Step 1 and Step 2 are completed before any uplink positioning happens. | It can support, if Step 1--3 are completed before any uplink positioning happens. | It can support, if Step 1 is completed before any uplink positioning happens. |
| Proactive | YES | YES | YES |
| Support data from served UE | YES | YES | YES |
| Support data from non-served UE | YES | YES | YES |
| New Message(s) | Class 1 NRPPA Data Collection Solicitation Request Procedure Class 2 NRPPA Data Collection Report | Class 2 NRPPA Data Collection Supported Class 1 NRPPA Data Collection Request/Subscription | Class 1 NGAP Data Collection Request/Subscription |
| CN Impact | NO | NO | YES, AMF’s functionality needs to be updated. Nlmf_Location_DetermineLocation response needs to include the relevant data. |

##### Data Collection in CU-DU split architecture

In the CU-DU split architecture, the gNB-CU is responsible for higher-layer functions, such as RRC and PDCP. These layers are involved in signaling procedures, including positioning-related messages.  When UE positioning is required, the gNB-CU handles communication with the LMF via control plane signaling. This typically includes exchanging messages to initiate positioning procedures or forward measurement results. The gNB-DU is typically responsible for lower-layer functions like MAC, RLC, and PHY. It performs tasks such receiving uplink signals like SRS for positioning. The gNB-DU forwards measurements to the gNB-CU, which in turn communicates with the LMF.

In Section 6.3.1 of  [11], it says: “﻿In case of a split gNB architecture, the NRPPa protocol is terminated at the gNB-CU.” It is our view, that AI/ML model training can be hosted within the gNB-CU, aligning with the principles outlined in the AI/ML for NG-RAN framework. It also avoids frequent F1 communication for data sample forwarding, such as from LMF to gNB-CU to gNB-DU if the model training is hosted at gNB-DU.

For the CU-DU split architecture, AI/ML model training is hosted within the gNB-CU for Case 3a. gNB-CU involves in the above proposed procedures.

gNB-DU collects raw measurements, channel information (Part A), and is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

### Model Inference

It was agreed in RAN1 118 meeting [5], that the existing procedures can be reused in terms of SRS configuration for model training, monitoring and inference. Regarding the model inference output, RAN1 116 meeting [4] agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting from the gNB to the LMF. If reporting the LOS/NLOS indicator, it can be provided as a soft or hard indicator, as specified in TS 38.214 [7]. If reporting timing information, it includes at minimum the UL RTOA or the gNB Rx-Tx time difference, as specified in TS 38.215 [8]. However, the details of the reporting are pending further discussions in RAN1.

Agreement (RAN1#116) Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

Agreement (RAN1#118 Bis) [6] Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

It has agreed from the RAN3 126 meeting, the model inference at the NG-RAN side in Case 3a can reuse the current NRPPa measurement procedures [10]. Legacy measurement IEs can still be used as part of traditional positioning methods, while additional AI/ML-based information elements may be introduced. The LMF can then calculate the UE's position using either legacy measurements or AI/ML-based measurements, incorporating both traditional and new IEs as needed.

To track model accuracy in the model monitoring function, in our view, the LMF could provide some assistance information to the NG-RAN Node, such as the precision level or accuracy level of the final positioning based on the intermediate measurement statistics inferred from the AI/ML assisted positioning model hosted at the NG-RAN Node. Thus, it is important for the LMF be aware of that those intermediate measurement statistics actually inferred from the AI/ML assisted positioning model hosted at the NG-RAN Node. In other words, AI/ML inferred measurements are differentiated from the measurements generated in a traditional manner.

AI/ML inferred measurements are differentiated from the measurements generated in the traditional manner. They need to be marked by the NG-RAN Node as different types of measurements.

For CU/DU Split architecture, since the model training is performed at the gNB-CU, it is reasonable to think that the gNB-CU also performs the model inference. The input data for the model inference comes from TRPs, which are located in the gNB-DU. According to TS 38.470 [9], the positioning function of F1 interface enables the gNB-CU to request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

For CU-DU split architecture, AI/ML model inference is hosted within the gNB-CU for Case 3a. The input data for the AI/ML model inference is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

### Model Monitoring

As agreed in the RAN3 125 126 meeting, we turned the WA to agreement: RAN3 will proceed with Option A: NG-RAN Node performs monitoring metric calculation for its own model. However, the LMF is the entity that gathers data from multiple NG-RAN nodes and calculates the UE’s position in the final step. Therefore, it may be advantageous for the LMF to provide assistance to the NG-RAN Node in calculating the monitoring metric or making the monitoring decision. What assistance information could be provided from the LMF to the NG-RAN node is for further discussion.

For Case 3a, the LMF can provide assistance information to the NG-RAN Node for the monitoring metric calculation and monitoring decision making.

# Conclusions

In this submission, we discussed the direct AI/ML positioning cases and potential RAN 3 impact. We presented the following proposals:

- For Case 3a, the input data is the raw measurements or channel information between UE and NG-RAN Node (also called as Part A), and the output data is the intermediate measurement statistics for positioning (also called as Part B), which could be derived from the UE location by LMF or NG-RAN Node.

- During legacy uplink UE positioning, the serving and assisting gNB can add a Part B Transfer indicator in the NRPPa MEASUREMENT Response.

- Upon receiving a Part B Transfer indicator in the NRPPa MEASUREMENT Response, the LMF sends a class 2 NRPPa Data Transfer message to the corresponding gNB, which includes the Part B of the data sample that the gNB could obtain opportunistically.

- A Class 1 NRPPa Data Collection Solicitation Request message could be initiated by the LMF to solicitate the NG-RAN Node’s data collection need.  The NG-RAN Node responds NRPPa Data Collection Solicitation Response to notify the LMF its data collection need, which could be a one-time request or periodic requests.

- NG-RAN Node could suggest the interested UE, or type of UEs for data collection to the LMF in the Data Collection Solicitation Response.

- LMF is the entity to perform the final UE selection by using the same mechanism as LMF-side model data collection.

- LMF initiates a class 2 NRPPa Data Collection Report message to the NG-RAN Node, which must include the ground truth label, optionally SRS configuration, Measurement ID, timestamp dependent of the role of the NG-RAN Node.

- A Class 2 NRPPa Data Collection Supported message could be initiated by the LMF to inform the NG-RAN Node that data collection is supported at the LMF.

- A class 1 NRPPa Data Collection Request or NRPPa Data Collection Subscription message is sent from the NG-RAN Node, the latter could be used as a long-term periodic data collection request.

- The class 1 NRPPa Data Collection Request is followed by one or multiple NRPPa Data Collection Response(s) which must include the ground truth label, optionally SRS configuration, Measurement ID, timestamp dependent of the role of the NG-RAN Node.

- A Class 1 NGAP Data Collection Request message could be initiated by the NG-RAN Node to the AMF that data collection is needed, which could include a list of target UEs. AMF selects UEs to collect data from.

- For long-term periodic data collection request, a NGAP Data Collection Subscription message could be initiated by the NG-RAN Node to the AMF, which could include notification criteria.

- The class 1 NGAP Data Collection Request is followed by one or multiple NRPPa Data Collection Response(s) which must include the ground truth label, optionally SRS configuration, Measurement ID, timestamp dependent of the role of the NG-RAN Node.

- For the CU-DU split architecture, AI/ML model training is hosted within the gNB-CU for Case 3a. gNB-CU involves in the above proposed procedures.

- gNB-DU collects raw measurements, channel information (Part A), and is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

- AI/ML inferred measurements are differentiated from the measurements generated in the traditional manner. They need to be marked by the NG-RAN Node as different types of measurements.

- For CU-DU split architecture, AI/ML model inference is hosted within the gNB-CU for Case 3a. The input data for the AI/ML model inference is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

- For Case 3a, the LMF can provide assistance information to the NG-RAN Node for the monitoring metric calculation and monitoring decision making.

# References

draft_MeetingReport_RAN3#127_Feb28

3GPP TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 18)

3GPP TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2 (version 19.1.0 Release 19)

Final_Minutes_report_RAN1#116_v100

Final_Minutes_report_RAN1#118_v100

Draft_Minutes_report_RAN1#118b_v010

﻿3GPP TS 38.214, “Physical layer procedures for data,” (version 18.4.0 Release 18)

﻿3GPP TS 38.215, “Physical layer measurements,” (version 18.3.0 Release 18)

﻿3GPP TS 38.470, “F1 general aspects and principles,” (version 18.3.0 Release 18)

3GPP TS 38.455, “NR Positioning Protocol A (NRPPa),” V18.3.0 (2024-09)

﻿“Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN ,” 3GPP, Tech. Rep. 3GPP TS 38.305 version 18.3.0 Release 18, 2024.