3GPP TSG-RAN WG3 Meeting #127 Bis	             R3-252041

Wuhan, China, 7 – 11 April 2025

Agenda Item:	17.2

Source:	CMCC

Title:	Discussion on On-Demand SSB SCell Operation

Document for:	Discussion

# Introduction

In the RAN3 126 meeting, regarding on-demand SSB SCell operation, the following agreement was made [1]:

[Agreement] For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

In the RAN3 127 meeting, the following discussion results were reached:

For the allowing On-demand SSB SCell, we discussed the two way-forward as follows: - WF1: No new List from the gNB-CU to the gNB-DU for “allowing OD SSB SCell” is needed, instead we can reuse, if needed, the Rel-18 existing “Cells Allowed to be Deactivated List” (CU to DU). Potential rewording of the procedure may be needed to make the “list” more general. - WF2: It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. But the above is not agreed, there are two concerns raised during the discussion: - One concern is that gNB-DU does not always need the “CU to DU” assistance in order to go to the OD-SSB SCell operation. - Another concern is how and whether to handle the transition from PCell normal operation to OD-SSB SCell only operation.

In this contribution, we discuss on the support of on-demand SSB SCell operation further, following the agreement that has been made in RAN3.

# Discussions

## Discussions on On-Demand SSB SCell Operation

RAN1 has outlined two cases for on-demand SSB: one where no always-on SSB is present on the cell, and another where always-on SSB is periodically transmitted on the cell with configurable periodicity. Additionally, four scenarios have been identified, primarily focusing on the timing related to the UE receiving the SCell activation.

In the case of the split architecture, RAN3 discusses whether the gNB-CU or gNB-DU is responsible for deciding on activation/deactivation of on-demand SSB on cells.

Defined in the current specification TS 38.473 [8], the gNB-CU could indicate the list of cells allowed to be deactivated through the Cells Allowed to be Deactivated List IE contained in the GNB-CU CONFIGURATION UPDATE message, after receiving this, the gNB-DU considers that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

The gNB-CU has comprehensive knowledge of the status of all cells connected to the gNB-DUs it controls, as well as information about neighbouring cells similar to its indication of the list of cells allowed to be deactivated. As a result, the gNB-CU could indicate the list of cells allowed to be on-demand SSB activated.  After a gNB-DU receives this through the GNB-CU CONFIGURATION UPDATE message, it considers that is allowed to enter on-demand SSB operation mode for network energy saving purpose. Furthermore, the gNB-CU is responsible for configuring the mode (on-demand, periodic) and parameters associated with SSB transmission.

The gNB-DU is responsible for both controlling and performing the SSB transmission. Consequently, it is the gNB-DU that consumes energy for these transmissions. Because the gNB-DU handles the transmission, it can directly monitor and be aware of the energy used for SSB transmissions. Most of the energy required for baseband processing in the network is also consumed by the gNB-DU, which means that the gNB-DU is well-positioned to evaluate the energy efficiency of a cell. The gNB-DU can monitor its status, such as energy usage efficiency of a cell and report to the gNB-CU for it to make decisions on the cells allowed to be on-demand SSB mode. If the energy efficiency is low (perhaps due to low user data rates or because only a few UEs are connected to the cell) the gNB-DU can take action to improve efficiency. How energy efficiency is defined and when to trigger the gNB-DU to take action are upto implementation and out of scope of this contribution. However, we view that the gNB-DU has the knowledge of cells’ status in terms of energy efficiency. The gNB-DU could send the energy efficiency to the gNB-CU in the ﻿RESOURCE STATUS UPDATE.

Based on the updated resource status of the gNB-DU, one approach is that the gNB-CU might determine to update the list of cells allowed to be on-demand SSB activated. This could involve deactivating some cells that are currently in the on-demand SSB activated state and adding new cells to the list of those allowed for on-demand SSB activation. For cells that are currently in the on-demand SSB activated state but need to be deactivated, the GNB-CU CONFIGURATION UPDATE and Cells to be Activated List Item IE can be reused to allow the gNB-CU to request, via the F1 interface, that the gNB-DU switch on certain cells.

In the last RAN1 120 meeting, the following agreement was reached:

Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) FFS: Which of the above parameters are a list

The gNB-CU is responsible for configuring the mode (on-demand, periodic) and parameters associated with SSB transmission, which include what are agreed in RAN1.

The gNB-CU could indicate the list of cells allowed to be on-demand SSB to gNB-DU.

It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”.

The gNB-DU could update the gNB-CU its cells status, such as energy efficiency, such that the gNB-CU could adjust the list of cells permitted to be on-demand SSB activated.

In the last RAN3 127 meeting, we have reached a conclusion that the existing the Cells Allowed to be Deactivated List IE can be used to carry the list of cells allowed to be on-demand SSB activated, and associated SSB transmission mode and parameter(s). However, the Cells Allowed to be Deactivated List might include the list of cells that are beyond the list of cells allowed to be on-demand SSB activated. The difference between "cells allowed to be on-demand SSB Activated" and "Cells Allowed to be Deactivated" lies in their purposes and functionalities:

- Cells Allowed to be On-Demand SSB Activated: These are cells that are dynamically activated to transmit SSBs when requested.

- Cells Allowed to be Deactivated: This is a list of cells that can be temporarily turned off (deactivated) to conserve energy or optimize resource usage without compromising essential network performance.

The existing the Cells Allowed to be Deactivated List IE could be reused to include cells allowed to be on-demand SSB activated, and associated SSB transmission mode and parameter(s).

After SCells are established for a UE through the F1 UE context setup or modification procedure, the gNB-DU is responsible for deciding whether to activate or deactivate these SCells for the UE. It is also logical for the gNB-DU to decide when to start and stop on-demand SSB transmissions for these cells. As agreed in the RAN1 118 meeting [5]:

For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

As agreed in RAN2 127 Bis meeting [7]:

Agreements on OD-SSB SCell - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

It is also agreed in the RAN1 118 Bis meeting [6], for a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported.

RAN1 has agreed on network-triggered on-demand SSB operations to support energy-saving efforts:

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A

    - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command.

    - Scenario #2A: SCell is configured to a UE when a UE receives SCell activation command

  - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

As agreed from the RAN3 126 meeting, For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

In the RAN1 119 meeting, the following agreement was reached [9]:

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

It can be interpreted as: in order to deactivate on-demand SSB transmission from a UE perspective, two options could be used:

- First option is to use a MAC-CE to explicitly signal the deactivation of SSB transmission in a scenario where SSBs are transmitted on-demand. It is up to RAN2 to decide whether RRC could be used for explicit deactivation.

- Second option is to configure or indicate the number N of on-demand SSB bursts to be transmitted, which specifies how many consecutive SSB bursts should be sent after an on-demand SSB activation is triggered. After the SSB bursts are sent, the on-demand SSB transmission is stopped, which implicitly deactivate the SSB transmission.

On-demand SSB transmission activation and deactivation are configured/indicated by gNB-DU to the UEs by MAC-CE, optionally by RRC (up on RAN2’s decision).

gNB-DU configures/indicates the number N of on-demand SSB bursts to be transmitted for implicit deactivation of SSB transmission.

For MAC CE based signaling, the gNB-DU can send the MAC CE to the UE. When the SCell’s status changes due to whether DU decides to start or stop the SSB broadcast, RAN3 needs to discuss if the change should be notified from the gNB-DU to the gNB-CU. If the notification of the status change that is decided by the gNB-DU is sent to the gNB-CU, the gNB-CU will have a real-time knowledge of the all-cells’ status, which helps it decide/update the list of cells allowed to be on-demand SSB activated as discussed earlier.

gNB-CU should be notified, when the DU decides to activate broadcasting SSB with SSB periodicity, and deactivate the SSB broadcast explicitly through MAC-CE (optionally RRC) or through configuration/indication the number N of on-demand SSB bursts to be transmitted.

# Conclusions

In this submission, we discuss the support of on-demand SSB SCell operation. The following observations and proposals are made:

- The gNB-CU is responsible for configuring the mode (on-demand, periodic) and parameters associated with SSB transmission, which include what are agreed in RAN1.

The gNB-CU could indicate the list of cells allowed to be on-demand SSB to gNB-DU.

It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”.

The gNB-DU could update the gNB-CU its cells status, such as energy efficiency, such that the gNB-CU could adjust the list of cells permitted to be on-demand SSB activated.

The existing the Cells Allowed to be Deactivated List IE could be reused to include cells allowed to be on-demand SSB activated, and associated SSB transmission mode and parameter(s).

RAN1 has agreed on network-triggered on-demand SSB operations to support energy-saving efforts:

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A

    - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command.

    - Scenario #2A: SCell is configured to a UE when a UE receives SCell activation command

  - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

On-demand SSB transmission activation and deactivation are configured/indicated by gNB-DU to the UEs by MAC-CE, optionally by RRC (up on RAN2’s decision).

gNB-DU configures/indicates the number N of on-demand SSB bursts to be transmitted for implicit deactivation of SSB transmission.

gNB-CU should be notified, when the DU decides to activate broadcasting SSB with SSB periodicity, and deactivate the SSB broadcast explicitly through MAC-CE (optionally RRC) or through configuration/indication the number N of on-demand SSB bursts to be transmitted.

# References

- draft_MeetingReport_RAN3#126_vDec05

- Final Report of 3GPP TSG RAN WG1 #116 v1.0.0 	(Athens, Greece, February 26th - March 1st, 2024)

- Final Report of 3GPP TSG RAN WG1 #116b v1.0.0 (Changsha, China, April 15th – 19th, 2024)

- Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0 (Fukuoka, Japan, May 20th – 24th, 2024)

- Draft Report of 3GPP TSG RAN WG1 #118 v0.1.0	(Maastricht, The Netherlands, August 19th – 23rd, 2024)

- Draft_Minutes_report_RAN1#118b_v010 (Hefei, China, October 14th – 18th, 2024)

- Draft_RAN2_127b_Meeting_Report_v1 (Hefei, China, October 14th – 18th, 2024)

- TS 38.473 V18.2.0

- Draft_Minutes_report_RAN1#119_v030