3GPP TSG-RAN WG3 #127-bis                                           R3-252153

Wuhan, China, 07-11 April, 2025

Agenda Item:	13.2

Source:	Samsung

Title:	(TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM

Document for:	Approval

Introduction

In this contribution, we propose TP for XnAP/F1AP on Inter-CU LTM based on the discussion in R3-252152.

TP to TS 38.423

## 8.x	Procedures for L1/L2 Triggered Mobility

### 8.x.1	LTM Configuration Update

Editor’s note: Details need to be continued.

#### 8.x.1.1	General

The purpose of the LTM Configuration Update procedure is to update LTM configuration data, or to inform the change of serving NG-RAN node and setup the UE associated signalling between the new serving NG-RAN node and all other candidate NG-RAN nodes. The procedure uses UE associated signalling.

- 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: LTM Configuration Update procedure. Successful operation

The NG-RAN node1 initiates the procedure by sending the LTM CONFIGURATION UPDATE message to NG-RAN node2.

The LTM CONFIGURATION UPDATE message may contain:

-	the LTM Information Request IE;

-	the Early Sync Information Request IE;

-	the Early Sync Information IE;

-	the Early Sync Information for SUL IE;

-  the Data Forwarding Info IE.

If the LTM Information Request IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider that the NG-RAN node1 has requested the LTM information update for the UE, include the LTM Information Response IE in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message and act as defined in TS 38.300 [9].

If the Reference Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, take it into account for generating the LTM configuration.

If the Request for CSI Resource Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, include the CSI Resource Configuration IE in the LTM Information Response IE in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM-CandidateConfig IE for the serving cell.

If the LTM Configuration ID Mapping List IE is contained in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider that the NG-RAN node1 has requested the early synchronization information for the UE in the NG-RAN node2.

If the Early Sync Information Response IE is contained in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall, if supported, store it and use it as defined in TS 38.300 [9].

If the Early Sync Information IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider it as the consolidated early sync information to use for subsequent LTM.

If the Early Sync Information for SUL IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider it as the consolidated early sync information for SUL to use for subsequent LTM.

If the Data Forwarding Info List IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, use the information for forwarding data traffic after UE accesses into this NG-RAN node2 via LTM execution.

If the UE Context Reference at the Last Source NG-RAN node IE are included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall consider it to identify UE and setup the UE associated Xn signalling connection.

If the LTM Candidate Cells To Be Cancelled List IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall consider that only the resources reserved for the cells identified by the included NG-RAN CGI are about to be released.

If the LTM Candidate Cells To Be Cancelled List IE is included in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall consider that only the resources reserved for the cells identified by the included NG-RAN CGI are about to be released.

### 8.y.1	Cell Switch Notification

Editor’s note: Details need to be continued.

#### 8.y.1.1	General

The purpose of the Cell Switch Notification procedure is to enable the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the source NG-RAN node to the target NG-RAN node. The procedure uses UE-associated signalling.

#### 8.y.1.2	Successful Operation

Figure 8.y.1.2-1: Cell Switch Notification procedure. Successful operation.

The source NG-RAN node initiates the procedure by sending a CELL SWITCH NOTIFICATION message.

Upon reception of the CELL SWITCH NOTIFICATION message, the target NG-RAN node shall, if supported, consider that a cell switch command was sent to the UE where one cell of the target NG-RAN node is indicated by the included Target Cell Global ID IE.

If the LTM Cell Switch Information IE is included in the CELL SWITCH NOTIFICATION message, the target NG-RAN node shall, if supported, use it as specified in TS 38.300 [9].

If the TA Information List IE is included in the CELL SWITCH NOTIFICATION message, the target NG-RAN node shall, if supported, use it as specified in TS 38.300 [9].

### 8.z.1	TA Information Transfer

Editor’s note: Details need to be continued.

#### 8.z.1.1	General

The purpose of the TA Information Transfer procedure is to enable the NG-RAN node1 to send the TA related information to the NG-RAN node2. The procedure uses non UE-associated signalling.

- 8.z.1.2	Successful Operation

Figure 8.z.1.2-1: TA Information Transfer procedure. Successful operation.

The NG-RAN node1 initiates the procedure by sending a TA INFORMATION TRANSFER message.

If the Early RACH Resources Requester ID IE is included in the TA INFORMATION TRANSFER message, the NG-RAN node2 shall, if supported, use it to forward related TA information to the entity indicated by the Early RACH Resources Requester ID IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item |  | 1 .. <maxnoofTAList> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.x2		TA INFORMATION TRANSFER

This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information.

Editor’s note: Details on IEs need to be continued.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| TA Information List |  | 1 |  |  | YES | ignore |
| >TA Information Item |  | 1 .. <maxnoofTAList> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |
| >>Early RACH Resources Requester ID | O |  | 9.2.1.xx9 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |
| Early Sync Information | O |  | 9.2.1.xx6 |  | YES | ignore |
| Early Sync Information for SUL | O |  | 9.2.1.xx6 |  | YES | ignore |
| Data Forwarding Info List | O |  | 9.2.1.x12 |  | YES | ignore |
| UE Context Reference at the Last Source NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| LTM Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofLTMCells> |  |  | YES | ignore |
| >Target Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

#### 9.1.1.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| LTM Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofLTMCells> |  |  | YES | ignore |
| >Target Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.xx3	Early Sync Information Request

This IE contains a request for Early synchronization information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Early Sync Configuration Request | FM |  | ENUMERATED (true, …) |  |
| Early RACH Resources List |  | 0..1 |  |  |
| >Early RACH Resources Item |  | 1..< maxnoofEarlyRACHResourcesID> |  |  |
| >>Global gNB ID | M |  | 9.2.2.1 |  |
| >>Early RACH Resources Requester ID | O |  | 9.2.1.xx9 | Identifies the entity requesting Early RACH resources. |

| Range bound | Explanation |
| --- | --- |
| maxnoofEarlyRACHResourcesID | Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8. |

#### 9.2.1.xx4	Early Sync Information Response

This IE contains Early synchronization related information to be provided in response to information provided in the Early Sync Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. |
| Early UL Sync Configuration | O |  | 9.2.1.xx6 | Early UL sync configurations for the UE. |
| Early UL Sync Configuration for SUL | O |  | 9.2.1.xx6 | Early UL sync configurations for the UE for SUL carrier. |

#### 9.2.1.xx6	Early UL Sync Configuration

This IE indicates the Early UL sync configurations for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RACH Configuration | M |  | OCTET STRING | Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [10]. |
| Early RACH Resources List |  | 0..1 |  |  |
| >Early RACH Resources Item IEs |  | 1..< maxnoofEarlyRACHResourcesID> |  |  |
| >>Global gNB ID | M |  | 9.2.2.1 |  |
| >>Early RACH Resources Requester ID | O |  | 9.2.1.xx9 | Identifies the entity requesting Early RACH resources. |
| >>Preamble Index List | O |  | 9.2.1.xx7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEarlyRACHResourcesID | Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8. |

#### 9.2.1.xx7	Preamble Index List

This IE indicates the list of preamble indexes to be used for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Preamble Index Item IEs |  | 1..< maxnoofLTMCells> |  |  |
| >Preamble Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

#### 9.2.1.xx9	Early RACH Resources Requester ID (naming FFS)

This IE indicates the identity of the Early RACH resources requester.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Early RACH Resources Requester ID (naming FFS) | M |  | INTEGER (0 .. 236-1) | Identifies requester of the Early RACH resources. |

#### 9.2.1.x12	Data Forwarding Info List

This IE contains different data forwarding addresses information per-PDU session for different candidate LTM cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Data Forwarding Info List |  | 1 |  |  | YES | ignore |
| >Data Forwarding Info Item |  | 1.. <maxnoof LTMCells> |  |  | – |  |
| >>LTM Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>PDU Session Resources List |  | 1 |  |  | – |  |
| >>>PDU Session Resources Item |  | 1. .<maxnoofPDU Sessions> |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>>Data Forwarding Info from target NG-RAN node | M |  | 9.2.1.16 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSsessions | Maximum no. of PDU sessions. Value is 256 |

TP to TS 38.473

/*********************Start of changes***********************/

- 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM Nodes List |  | 0..1 |  | This IE contains the IDs of the source node and candidate node(s). | YES | reject |
| >>LTM Nodes Item IEs |  | 1..< maxnoofLTMNodes> |  |  |  |  |
| >>>LTM gNB-DU ID | O |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 | This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE. | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMNodes | Maximum no. of Nodes allowed to be configured with LTM towards one UE, the maximum value is 8. |

/*********************Next change***********************/

- 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM Nodes ID List |  | 1 |  | This IE contains the IDs of the source node and candidate node(s). | YES | reject |
| >>LTM Nodes Item IEs |  | 1..< maxnoofLTMNodes> |  |  | - |  |
| >>>LTM gNB-DU ID | O |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMNodes | Maximum no. of Nodes allowed to be configured with LTM towards one UE, the maximum value is 8. |

/*********************Next change***********************/

#### 9.2.1.24	DU-CU TA INFORMATION TRANSFER

This message is sent by the gNB-DU to inform the gNB-CU about TA information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DU to CU TA Information List |  | 0..1 |  |  | YES | ignore |
| >DU to CU TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |
| >>Source gNB-DU ID | O |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | - |  |
| >>Source gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |

- 9.3.1.328	Early UL Sync Configuration

This IE indicates the early UL sync configurations for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RACH Configuration | M |  | OCTET STRING | Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8]. |
| LTM Nodes List |  | 0..1 |  | This IE contains the IDs of the source node and candidate node(s). |
| >LTM Nodes Item IEs |  | 1..< maxnoofLTMNodes> |  |  |
| >>LTM gNB-DU ID | O |  | gNB-DU ID 9.3.1.9 |  |
| >>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  |
| >>Preamble Index List | O |  | 9.3.1.329 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMNodes | Maximum no. of Nodes allowed to be configured with LTM towards one UE, the maximum value is 8. |

/*********************End of changes***********************/