3GPP TSG-RAN WG3 Meeting #127bis	R3-252167

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	10.3.1

Source:	CMCC, Huawei

Title:	Discussion on SON/MDT enhancement for NTN

Document for:	Discussion

# 1 Introduction

In RAN3#127 meeting, there was discussion on SON/MDT enhancement for Intra-NTN mobility. Some agreements were achieved and some issues were captured [1] and need to be continued in next meeting:

RAN3#127:

Time based CHO:

Existing timers can be reused to identify when RLF happens with 100ms accuracy.

FFS whether more accuracy is needed.

No need to identify whether RLF happens before or after T1 for time-based CHO.

Location based CHO:

Revert the WA: There is no need to report any other info in the RLF report for the location based CHO.

For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged.

- Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report.

- Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred).

FFS on option 1 or option 2.

MRO mechanism for intra-NTN mobility shall be enhanced to enable handling of local and temporary propagation changes (e.g. due to clouds, rain etc.)?

In this contribution, we would discuss on the issues of MRO for failure scenario for intra-NTN and our initial consideration on MRO for near failure scenario.

# 2 Discussion

2.1 MRO for failure scenarios

Location based CHO:

Revert the WA: There is no need to report any other info in the RLF report for the location based CHO.

For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged.

Based on the discussion about location based CHO in previous meetings, we believe there is already a consensus on the importance of the distance information between the UE and the moving reference location in both the serving cell and the candidate cell at the time of failure. The distance information could help to optimize whether the configured distance thresholds may contribute to the failure.

Observation 1: For location based trigger condition, the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs is helpful to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Proposal 1: The objective of MRO for location-based CHO is to optimize configuration issues of the distance thresholds in trigger condition.

As per the agreements of last meeting, we could see that RAN3 has acknowledged the accuracy and complexity issue of the network-based solution that the network derives the distance between UE and the referenceLocation in serving cell and in candidate cell based on UE location information. Therefore, some necessary information reported from UE should be considered in the RLF report for the location based CHO. And the following potential solutions were captured in last meeting:

- Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report.

- Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred).

For Option 1, the absolute UTC time of when RLF/HOF happens was originally proposed as a potential solution to replace the relative time (e.g., timeSinceCHO-Reconfig) for the Time based CHO. And we think the proposal was not agreed in the Time based CHO scenario for the following reasons:

- The existing timers (e.g., timeSinceCHO-Reconfig) with 100ms accuracy are sufficient to identify the time of when RLF happens;

- The IE size of the absolute UTC time is big, and reporting absolute UTC time when RLF occurred in the RLF report would cause UE complexity[2].

We observe that a few information related to the absolute UTC time (e.g., timeInfoUTC in SIB9 or t1-Threshold for CondEventT1 in ReportConfigNR) are specified in TS 38.331. As per the definition of t1-Threshold shown below, we can see that 39 bits may be required to represent this parameter. Therefore, the information related to the absolute UTC time are only provided from Network to UE by now. From our point of view, we should avoid to directly introduce the absolute UTC time in RLF report from UE which would cause UE complexity.

Observation 2: The information related to the absolute UTC time which may be represented by 39 bits are only provided from Network to UE by now.

Observation 3: Reporting absolute UTC time when RLF occurred in the RLF report would cause UE complexity. It should avoid to directly introduce the absolute UTC time in RLF report from UE.

Furthermore, for location-based CHO, we need to get the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell to optimize configuration issues of the distance thresholds in trigger condition. Option 1 is only used to get the time of when RLF/HOF happens. Network still has to perform the additional calculation to derive the distance between UE and the referenceLocation in serving cell and in candidate cell when RLF occurred based on UE location information.

Observation 4: Option 1 is only used to get the time of when RLF/HOF happens. The additional calculation is still needed for Network to derive the distance between UE and the referenceLocation in serving cell and in candidate cell when RLF occurred based on UE location information.

For Option 2, it is a straightforward solution to get the distance information for location-based CHO. There is no need for Network to perform any additional calculation for distance information. Actually, UE also needs to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively to evaluate whether execution condition is satisfied for location-based CHO. Therefore, there is no additional complexity for UE to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell when RLF occurred.

Moreover, as per the definition of distanceThreshFromReference1 and distanceThreshFromReference2 in Event D2, we could roughly get to know that the size of measured distance from UE to moving reference location of serving cell and candidate cell would be much less than the size of the absolute UTC time.

Based on the above analysis, we think Option 2 is a more reasonable solution than Option 1 to get the distance information for location-based CHO with the following reasons:

- Option 2 is a straightforward solution to get the distance information for location-based CHO. There is no need for Network to perform any additional calculation for distance information.

- There is no additional complexity for UE to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell when RLF occurred. Anyway, UE needs to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively to evaluate whether location-based CHO execution condition is satisfied.

- The IE size of measured distance from UE to moving reference location of serving cell and candidate cell would be much less than the IE size of the absolute UTC time.

Observation 5: Option 2 is a more reasonable solution to get the distance information for location-based CHO with the following reasons:

- Option 2 is a straightforward solution to get the distance information for location-based CHO. There is no need for Network to perform any additional calculation for distance information.

- There is no additional complexity for UE to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell when RLF occurred. Anyway,UE needs to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively to evaluate whether location-based CHO execution condition is satisfied.

- The  IE size of measured distance from UE to moving reference location of serving cell and candidate cell would be much less than the IE size of the absolute UTC time.

Based on the above comparation, the pros and cons of Option 1 and Option 2 are summarized as below:

|  | Option 1 | Option 2 |
| --- | --- | --- |
| The IE size of reporting information | Big | Small |
| Additional calculation for Network | Yes | No |
| Additional UE complexity | Yes | No |

Proposal 2: RAN3 to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in RLF report to get the distance information for location-based CHO.

2.2 MRO for near failure scenarios

Based on the agreements achieved in RAN3#123bis meeting [3], the work on MRO for near failure scenarios could start after failure scenarios. Considering the progress of normative work on MRO for failure scenarios, we propose to start the work on optimization for near failure scenarios.

MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later.

For near failure scenarios, Successful Handover Report (SHR) could be enhanced to support successful Conditional handovers in NTN to detect a sub-optimal successful CHO event. Considering the time-based and location-based trigger condition for CHO are supported in NTN, a new time-based SHR trigger and a location-based SHR trigger configured by network could be considered accordingly to trigger the SHR report when UE execute CHO successfully to a candidate cell with high risk of failure. Specifically,

- A new time-based SHR trigger for NTN could be the threshold(s) of time (e.g., in terms of percentage values) used to trigger the SHR report. For example, UE collects SHR if the ratio between the value of elapsed time since t1(t1 is configured by NW in CondEventT1) until UE execute CHO and the value of Duration (Duration is configured by NW in CondEventT1) is greater than the threshold, and makes the SHR available to the network.

- A new location-based SHR trigger for NTN could be the threshold(s) of distance used to trigger the SHR report. For example, in the case that the threshold of distance is in terms of percentage values, UE collects SHR if the ratio between the value of distance between UE and the referenceLocation/moving referenceLocation in serving cell when UE execute CHO, and the threshold configured by NW is greater than the location-based SHR trigger, and makes the SHR available to the network.

Proposal 3: Successful Handover Report (SHR) could be enhanced to support MRO for near failure scenarios in NTN.

Proposal 4: The introduction of a new time-based SHR trigger and a location-based SHR trigger could be considered for optimization of near failure scenarios in NTN.

Based on initial analysis, we think that the following information maybe helpful for optimization of CHO trigger conditions in NTN and could be logged by UE in the enhanced SHR.

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network; this information helps to identify whether too late CHO may occur and optimize T1 threshold or Duration in the time-based trigger condition or distance thresholds/timeToTrigger in location-based trigger condition or the Thresholds in CHO events A3/A4/A5.

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network; this information helps to identify whether the second CHO trigger condition satisfied late that may cause too late CHO and optimize Duration in the time-based trigger condition or distance thresholds/timeToTrigger in location-based trigger condition or the Thresholds in CHO events A3/A4/A5.

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled; the information could be used for location-based trigger condition to identify whether too late CHO or too early CHO may occur and optimize distance thresholds or timeToTrigger in location-based trigger condition, for the earth moving case.

- The cause value that triggers the SHR; this information helps to know which configurations of CHO trigger condition have potential problems.

Proposal 5: RAN3 to consider SHR enhancement to support optimization of near failure scenarios in NTN, e.g.,

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network;

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network;

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled, for the earth moving case;

- The cause value that triggers the SHR, e.g., the threshold(s) of time in time-based SHR trigger and the threshold(s) of distance in location-based SHR trigger.

# 3 Conclusion

In this paper, we discuss on MRO mechanisms for intra-NTN mobility, we have the following observations and proposals:

MRO for failure scenarios

For location based CHO:

Observation 1: For location based trigger condition, the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs is helpful to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Proposal 1: The objective of MRO for location-based CHO is to optimize configuration issues of the distance thresholds in trigger condition.

Observation 2: The information related to the absolute UTC time which may be represented by 39 bits are only provided from Network to UE by now.

Observation 3: Reporting absolute UTC time when RLF occurred in the RLF report would cause UE complexity. It should avoid to directly introduce the absolute UTC time in RLF report from UE.

Observation 4: Option 1 is only used to get the time of when RLF/HOF happens. The additional calculation is still needed for Network to derive the distance between UE and the referenceLocation in serving cell and in candidate cell when RLF occurred based on UE location information.

Observation 5: Option 2 is a more reasonable solution to get the distance information for location-based CHO with the following reasons:

- Option 2 is a straightforward solution to get the distance information for location-based CHO. There is no need for Network to perform any additional calculation for distance information.

- There is no additional complexity for UE to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell when RLF occurred. Anyway,UE needs to measure the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively to evaluate whether location-based CHO execution condition is satisfied.

- The  IE size of measured distance from UE to moving reference location of serving cell and candidate cell would be much less than the IE size of the absolute UTC time.

Proposal 2: RAN3 to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in RLF report to get the distance information for location-based CHO.

MRO for near failure scenarios

Proposal 3: Successful Handover Report (SHR) could be enhanced to support MRO for near failure scenarios in NTN.

Proposal 4: The introduction of a new time-based SHR trigger and a location-based SHR trigger could be considered for optimization of near failure scenarios in NTN.

Proposal 5: RAN3 to consider SHR enhancement to support optimization of near failure scenarios in NTN, e.g.,

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network;

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network;

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled, for the earth moving case;

- The cause value that triggers the SHR, e.g., the threshold(s) of time in time-based SHR trigger and the threshold(s) of distance in location-based SHR trigger.

# Reference

- RAN3_127_agenda_20250221_1300_EOM, Feb 2025.

- R3-250271, Discussion on MRO for intra-NTN mobility, Lenovo, Feb 2025.

- RAN3_123bis_agenda_20240419_EOM, Apr 2024.