3GPP TSG-RAN WG3 Meeting #127bis														R3-252174

Wuhan, China, Apr 7th - Apr 11th, 2025

Agenda item:			11.3

Source:					CMCC

Title:						Discussion on AI/ML-based CCO

Document Type:		Other

Introduction

During RAN3#127, the following agreements have been captured for AI/ML enabled Coverage and Capacity Optimization.

Each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE has a Future Coverage Modification Cause IE associated to it from DU to CU.

The Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only.

Value ‘0’ for the Future Cell Coverage State IE is needed and has the same meaning as cell inactive in legacy CCO.

Whether to exchange time for predicted CCO issue over XnAP needs further discussion.

The maximum value of time for predicted CCO issue and time for future CCO state needs further discussion.

UE performance feedback for CCO? Legacy UE performance feedback is sufficient or other finer granularity, e.g., cell-/SSB-level UE performance feedback?

Whether gNB-CU provides to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation?

In this paper, we provide our consideration of AI/ML based Coverage and Capacity Optimization.

Discussion

Time information over XnAP

Whether to exchange time for predicted CCO issue over XnAP needs further discussion.

In RAN3#127, RAN3 has discussed whether to exchange time information of predicted CCO issue over XnAP. From our understanding, whether the time information of the coverage modification cause needs to be transferred over Xn depends on whether this time information is needed in the target node. For the target node, what it needs to do is to modify its own coverage at the time the coverage of source node changes. So, the target node needs to know when to make what kind of adjustment. Since the target node has received the future CCO state and the corresponding time information, it can be used by the target node to determine when to make the corresponding adjustments. Therefore, the time information of the predicted CCO issue seems to be less essential.

Proposal 1: No need to send time information related to coverage modification cause over Xn.

The maximum value of time for predicted CCO issue and time for future CCO state needs further discussion.

Then, for the Time for Future Coverage Modification and Predicted CCO issue, RAN3 has discussed whether 60s is sufficient or not. Since there is no criteria to evaluate whether 60s is enough, we propose to start from 60s, and expanding with sufficient motivation if needed.

Proposal 2: Time interval for predicted CCO issue and future CCO state propose to agree with 60s and expands if needed.

Suggested CCO state

Whether gNB-CU provides to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation?

During the RAN3#127 online discussion, whether CU needs to provide suggested CCO state to DU was not discussed. From our understanding, in legacy CCO mechanism, gNB-CU does not provide the suggested CCO state to the gNB-DU, the gNB-CU detects the CCO issue and sends it to the gNB-DU, and gNB-DU provides the CCO state to gNB-CU. It seems that gNB-DU has the ability to derive CCO state and gNB-CU does not need to provide suggested CCO state to gNB-DU as assistance information. As the gNB-DU has the ability to derive CCO state in legacy, we also think that gNB-DU should also be able to derive future CCO state in AI/ML-based CCO. Also, the CCO state information is only configured to gNB-DU by OAM. If gNB-CU provides the suggested CCO state to gNB-DU, the OAM should also provide the CCO state configuration to gNB-CU, which may have impact of OAM. Therefore, we think it’s no need to provide suggested CCO state from gNB-CU to gNB-DU.

Proposal 3: No need to provide suggested future CCO state from CU to DU as recommendation.

Feedback information

UE performance feedback for CCO? Legacy UE performance feedback is sufficient or other finer granularity, e.g., cell-/SSB-level UE performance feedback?

In RAN3#127, the above FFS for UE performance feedback was captured in the Chair notes. As discussed in the SI phase, the legacy UE performance feedback for those UEs handover from the source gNB was captured as feedback information in TR 38.743.

- 4.2.2.4 Feedback of AI/ML based CCO To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs: - Measured radio resource status - Legacy UE performance feedback for those UEs handed over from the source gNB SON Reports (e.g., RLF, CEF, RA)

The handover UEs of CCO may including two cases, the UE(s) mobility from source to the target gNB and the UE(s) serving gNB is changed after CCO due to the coverage change.

The UE performance feedback for both cases can be collected via legacy procedure.

However, for the latter one, as the UE(s) is mainly affected by CCO action, this kind of UE(s) performance needs to be specified and evaluated whether the generated future CCO state is appropriate. For example, the stationary UE(s) may locate in the gNB1 before coverage configuration changes, and locate in gNB2 after coverage configuration changes, if the UE(s) experiences severe performance degradation after coverage modification in gNB2, it seems that the generated future CCO state is not very reasonable. The UE performance feedback needs to be specified for these UE(s) as assistance information for source gNB-DU optimization.

Observation 1: The legacy UE performance for those UEs handover from the source gNB including the following two cases:

Case 1: The UE(s) mobility from source to the target gNB.

Case 2: The UE(s) serving gNB is changed after CCO due to the coverage change. These UE(s) mainly affected by AI/ML-based CCO action and the UE performance needs to be specified and evaluated whether the generated future CCO state is appropriate.

Although both cases need to collect the same legacy UE performance, it’s better to distinguish the handover caused by the AI/ML based CCO action from the mobility handover. So, it is proposed to indicate in the handover request message that this handover is affected by AI/ML enabled CCO action.

Proposal 4: The legacy UE performance feedback procedure can be reused for AI/ML enabled CCO.

Proposal 5: Distinguish the handover caused by the AI/ML based CCO action from the mobility handover by introducing an indication in handover request message.

For the finer granularity UE performance feedback, we think no need to collect the cell/SSB level UE performance, but the cell/node level gNB performance needs to be feedback from the target nodes.

When the coverage configuration of local node changes, the coverage of neighbor nodes also need to be changed. Whether the coverage configuration modification caused by the local node results in severe performance degradation of neighbor node needs to be considered. For example, after the coverage configuration modification, the number of RRC connections of neighbor node increases, which may cause the neighbor node overload, or the PRB usage decrease, which waste the neighbor node resources. The neighbor nodes need to inform the local node these information and the local node can update the AI/ML model with these information.

Proposal 6: No need to collect the cell/SSB level UE performance feedback.

Proposal 7: It’s proposed to collect the cell/node level performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage.

Conclusion

In this contribution, we propose the following proposals:

Proposal 1: No need to send time information related to coverage modification cause over Xn.

Proposal 2: Time interval for predicted CCO issue and future CCO state propose to agree with 60s and expands if needed.

Proposal 3: No need to provide suggested future CCO state from CU to DU as recommendation.

Observation 1: The legacy UE performance for those UEs handover from the source gNB including the following two cases:

Case 1: The UE(s) mobility from source to the target gNB.

Case 2: The UE(s) serving gNB is changed after CCO due to the coverage change. These UE(s) mainly affected by AI/ML-based CCO action and the UE performance needs to be specified and evaluated whether the generated future CCO state is appropriate.

Proposal 4: The legacy UE performance feedback procedure can be reused for AI/ML enabled CCO.

Proposal 5: Distinguish the handover caused by the AI/ML based CCO action from the mobility handover by introducing an indication in handover request message.

Proposal 6: No need to collect the cell/SSB level UE performance feedback.

Proposal 7: It’s proposed to collect the cell/node level performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

- Annex: TP for 38.473

    - 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

    - 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

- Annex: TP for 38.401

  - 7.9	Support for CCO

    - 7.9.1	General

The NR Capacity and Coverage Optimization (CCO) Function in non-split gNB case is specified in TS 38.300 [2]. The objective of this function is to detect and mitigate coverage and cell edge interference issues.

    - 7.9.2	OAM requirements

Each gNB-DU may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

    - 7.9.3	Dynamic coverage configuration changes

In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.

    - 7.9.X	AI/ML enabled CCO

In case of split gNB architecture, CCO issue is predicted at the gNB-CU. The gNB-CU sends to the gNB-DU the predicted CCO issue with the predicted affected cells and beams and the time for predicted CCO issue. The gNB-DU generates the future coverage state(s) and the time when the future coverage state(s) will be applied based on the predicted CCO issue and other local information. The gNB-DU may also take into account information received for other cells when generates the future CCO configuration. The gNB-DU informs the gNB-CU of the future coverage state and the time when the future coverage state will be applied.