3GPP TSG-RAN WG3 Meeting #127bis	R3-252192

# Wuhan, CN, 7th April– 11st April 2025

# Agenda item:	10.2

# Source:	ZTE Corporation

# Title:	(TP for SON BLCR 38.300 and38.473)MRO for LTM- invalid TA

# Document for:	Discussion

1	Introduction

RAN3#127：

MRO for LTM:

For RACH-less optimization:

-RAN3 to optimize the “Invalid TA” in outdate case?

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM?

Scenario needs to be further clarified (e.g. outdated TA, re-establishment in target cell, etc…).

Network based or UE based solution for LTM candidate cell list?

Network based solution is used for LTM candidate cell list.

Network based or UE based solution for TA acquisition type?

Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

The agreements achieved in RAN3 #126 [1]: For TP to BL CR to TS 38.473: - In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI. For TP to BL CR to TS 38.401: - For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM. - For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell. Whether network based solution is needed? What’s the UE context to be stored in RAN side? UHI and ping-pong: - Which node is responsible for avoiding LTM cell switch ping-pong (CU or DU)? - CU sends UHI to DUs? The agreements achieved in RAN3 #125bis [2]: - For TP to BL CR to TS 38.300: - Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions. - For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM. - Change“successful handover" definition, as proposed in R3-245230. - For TP to BL CR to TS 38.401: - Capture wrong LTM prepared cell selection. - Add failure type to F1AP ACCESS AND MOBILITY INDICATION message. - CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell. The agreements achieved in RAN2 #128 [3]: - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF. The agreements achieved in RAN2 #127bis [4]: - The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. - If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation. - Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell. - UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

In  this contribution, we will discuss how to optimize the “Invalid TA” in out date case.

2 Discussion

2.1 Invalid TA Scenario

Figure 1: Intra-CU LTM procedures (extract from TS38.401 Figure 8.2.1.5-1: Inter-gNB-DU LTM)

According to current procedure, for a UE configured with intra-CU LTM, if LTM cell switch is determined by the source DU, the source DU will send CELL SWITCH COMMAND MAC CE to UE and subsequently send DU-CU CELL SWITCH NOTIFICATION to the CU. If the source DU consider the UE  has a valid TA value for the target cell, it would include the TA value in the  CELL SWITCH COMMAND MAC CE. Then the UE will perform RACH-less LTM to the target cell. However, if the provided TA value is invalid (e,g, it is out date), the RACH-less LTM cell switch may fail since the target cell can not decode the UE's uplink messages which arrives outside the target cell's reception window caused by the incorrect TA.

Observation 1: if the provided TA value is invalid (e,g, it is out date), the RACH-less LTM cell switch may fail since the target cell can not decode the UE's uplink messages which arrives outside the target cell's reception window.

Therefore, the source DU should  find out whether the provided the TA value is valid or not.

Proposal 1: the source DU should find out whether the provided the TA value in the CELL SWITCH COMMAND MAC CE to the UE is valid or not.

As we know, in case the RACH-less LTM cell switch fails, the UE will try to find a suitable NR cell to access, and following cases may happen for the UE:

|  | - User case description | - Reason for failure |
| --- | --- | --- |
| - Case 1 | - After radio failure happen, the re-selected NR cell is inside the target cell indicated the CELL SWITCH COMMAND MAC CE, and the UE perform RACH-based LTM recovery to the target cell successfully; | - No Cell selection/configuration issue! But root cause may be the following: - 1:TA is not valid - Or 2: Wrong beam information |
| - Case 2 | - After radio failure happen, the re-selected NR cell is not the target cell indicated the CELL SWITCH COMMAND MAC CE but is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config, and the UE perform RACH-based LTM recovery to the selected cell successfully; | - Wrong cell issue! - Already covered by MRO for LTM cases(wrong cell ,too early etc) |
| - Case 3 | - After radio failure happen, the re-selected cell is not one of the LTM candidate cells, the UE perform RRC-reestablishment successfully. | - Wrong cell issue! - Already covered by MRO for LTM cases(wrong cell) |

More clarify of case 1:

How to understand LTM recovery and RRC-reestablishment in RAN3?

- TS 38.331 - 5.3.5.18.6 LTM cell switch execution - Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall: - .. - 1> if the LTM cell switch is triggered by an indication from lower layers: - 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3; - 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running): - 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3; - 5.3.7 RRC connection re-establishment - 5.3.7.3 Actions following cell selection while T311 is running - Upon selecting a suitable NR cell, the UE shall: - ... - 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and - 1> if attemptLTM-Switch is configured; and - 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: - 2> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

According to current TS38.331 spec as above, when the  LTM cell switch  fails, the UE will select a suitable NR cell, including LTM recovery or RRC-reestablishment:

-  if the selected cell is one of the LTM candidate cells and attemptLTM-Switch is configured, the UE can perform the LTM cell switch procedure for the selected LTM candidate cell, this procedure can be called as LTM recovery.

- Otherwise, the UE can only perform RRC-reestablishment as a normal UE without LTM configuration.

Compared with RRC-reestablishment, LTM recovery is better since it can reduce control plane signalling and interruption delay. According to understanding, if the configured LTM candidate cells are appropriate, the UE should perform LTM recovery rather than RRC-reestablishment in case of link failure.

- As we know, the RACH-less LTM cell switch command may include following four kinds of indication based on TS38.321:

- target cell id(Target config ID);

- TA value(Timing advance command);

- beam information (TCI state ID and UL TCI state ID);

- RACH resource.

If the Case 1 happens, it means the selected target cell and the switch time is appropriate ( the target cell selected by the source DU and the RACH-based LTM recovery is successfully), no cell the failure, so all the failure reasons of too early, too late and to wrong cell  can be excluded. In addition RACH resource issue will be record in RA report so not mentioned by here.

Therefore, the root cause should only be the invalid TA valued or wrong beam information  .

More clarify of case 2:

If the case 2 happens, it means the candidate LTM cells are appropriate but the selected target cell for LTM cell switch by the source DU is inappropriate. 	This issue is already covered by MRO for LTM cases(wrong cell ,too early etc), in this paper we only focus on TA value issue (only case 1).

More clarify of case 3:

If the case 3  happens, based on definition of 38.331,the re-selected cell is not one of the LTM candidate cells.It is wrong cell issue (RRC-reestablishment), not relate to wrong TA. The root cause may be the candidate LTM configuration is inappropriate.  This issue is already covered by MRO for LTM cases(wrong cell ,too early etc), in this paper we only focus on TA value issue (only case 1).

Observation 2: if UE fails to perform RACH-less LTM cell switch,  and followed by a successfully RACH-based  LTM recovery to the same target cell indicated in the CELL SWITCH COMMAND MAC CE, the failure cause may be the invalid TA value or wrong beam information.

Proposal 2: In addition to too late, too early, and wrong cell,  RAN3 to add a new MRO user case of invalid TA or wrong beam for detect connection failures.

How to differentiate root cause of invalid TA value from wrong beam failure

As previously explained, when Case 1 occurs, the root cause is limited to either an invalid Tracking Area (TA) value or a wrong beam selection. Therefore, the question arises: how can the NG-RAN node definitively identify if the issue is attributable to the TA value?

To accurately determine whether the failure is due to an invalid TA or a wrong beam, the network requires, at a minimum, the following information:

TA value 1: The TA value originally configured by the source Distributed Unit (DU) in the Layer 2 Measurement (LTM) Cell Switch Command Media Access Control Control Element (MAC CE), if applicable.

TA value 2: The TA value utilized during the successful Random Access Channel (RACH)-based LTM recovery, regardless of whether it was calculated or configured by the User Equipment (UE).

If these two values are different or do not align (TA1 - TA2 exceeds a predefined threshold), the network can conclusively determine that the TA value provided by the source DU was invalid.

Observation 3: in order to identify whether the problem of failure is invalid TA value or not, the source DU needs to acquire the TA value used for successful RACH-based accessing to the target cell after RACH-Less LTM cell switch fails.

Is it necessary to resolve invalid TA value issues caused by UE-based calculations?

According to the current specification, the Tracking Area (TA) value utilized for a Random Access Channel (RACH)-Less Layer 2 Measurement (LTM) cell switch can be provided by either the source Distributed Unit (DU) or measured by the User Equipment (UE) itself. If the TA value is provided by the source DU and is subsequently determined to be invalid, the source DU must perform TA optimization. Conversely, if the TA value is measured by the UE and is found to be invalid, the network may consider restricting the UE from performing RACH-less LTM based on UE-measured TA in subsequent attempts.

Observation 4: The source DU is required to perform TA optimization only when the TA value is provided by the network for a RACH-Less LTM cell switch.

2.2 candidate solution

To help the source DU acquire above information, following two options can be considered:

- UE-based solution

For a UE-based solution, when the UE detects RACH-less LTM cell switch failure, and followed by a successfully RACH-based LTM recovery to the same target cell indicated in the CELL SWITCH COMMAND MAC CE, it can store the TA value used for successful RACH-based LTM recovery in the RLF report.

In addition, since the TA valued used for RACH-Less LTM cell switch can either be provided by the source DU or be measured by the UE itself, and the network only consider to do TA optimization for the case that invalid TA is provided by the source DU. During last meeting, there is a FFS issue as below:

Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

- According to current specification, after DU trigger LTM cell switch for the UE, it will send  DU-CU CELL SWITCH NOTIFICATION message to the CU, in the DU-CU CELL SWITCH NOTIFICATION message, the TA Information List only include the TA value of the candidate cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

- It seems not clear whether the target cell id is also included in the TA Information List. Furthermore, we check the current TS 38.401 specification, during the step 19 of the Inter-gNB-DU LTM procedures (Figure 8.2.1.5-1) as below:

- 19. The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch Command to the UE, including the target cell ID and the TCI state ID(s). The TA value(s) related information applicable for subsequent LTM may also be included.

- We find that the source DU may only send the TA value(s) related information applicable for subsequent LTM to the CU, which may not include the TA value(s) related the target cell, since when the UE switch to the target cell, the target cell will know the more accurate TA of UE soon.

- Base on above analysis, we cannot ensure that the CU can know whether the TA value is provided to the UE by the source DU or not.

- Moreover, it is not clear whether the DU still keep the TA information of the UE when it receive the RLF report from the UE.

Observation 5: according to current specification, the CU may not know whether TA is provided to the UE by the source DU or not. And  it is not clear whether the DU still keep the TA information of the UE when it receive the RLF report from the UE.

In order to make this solution reliable, the UE has to provide more information in the RLF report besides the TA value used for successful RACH-based LTM recovery, such as indication of whether the TA used in RACH-less LTM cell switch is provided by the source DU,  and  the TA value used for failure RACH-less  LTM cell switch.

Proposal 3: if  UE-based solution is adopted, the UE should provide following information in the RLF report:

-  the TA value used for successful RACH-based LTM recovery.

-  an indication of whether the TA used in RACH-less LTM cell switch is provided by the source DU.

-  the TA value used for failure RACH-less LTM cell switch.

- Network solution

- Network seems to have everything as we mentioned above to determine invalid TA value.

- The network should aware the failure is happen (the trigger) :

- Upon receiving a RACH-less Layer 2 Measurement (LTM) cell switch command from the source cell, the User Equipment (UE) shall immediately execute RACH-less LTM to the target cell. From the target cell's perspective, upon receiving a Centralized Unit - Centralized Unit (CU-CU) CELL SWITCH NOTIFICATION from the source Distributed Unit (DU) via the source CU, it should anticipate the UE's imminent arrival and expect a RACH-less access. If the target cell observes the UE performing a RACH-based access, it may suspect a RACH-less LTM failure. Consequently, the target DU becomes aware of a potential anomaly.

- The network action more faster than UE based solution:

- Without waiting for the subsequently RLF report from the UE, the target cell can send the RACH-based related info ( e.g, the TA value used for successful RACH-based accessing) to the source CU in the ACCESS SUCCESS message, then the source CU can forward this info to the source DU to do analyse.

- If the source DU did not provide valid TA in the LTM cell switch command before, there is no problem in the network, it , the issue should be UE has calculated the wrong TA value, nothing need to be optimized in the network.

- If the source DU has provided valid TA in the LTM cell switch command to the UE, but the UE still perform RACH-based access mode to the target cell and the TA value used in RACH-based accessing is much different from the one it provides,  which means the value provide to UE is wrong, the source it should do some optimization for the UL early sync.

- The source DU has enough information to do the optimaization, it has not only the original TA value, but also has the actually used TA value in LTM recovery.

Proposal 4: if network-based solution is adopted, if the target cell find the UE perform RACH-based access mode,  it can send the RACH-based related info ( e.g, the TA value used for successful RACH-based accessing) to the source CU in the ACCESS SUCCESS message, then the source CU can forward this info to the source DU to do analyse.

Compared with UE based solution, the network based solution seems having following two advantages:

- Having no impacts on UE.

- Providing faster response for the issue, since the source DU can acquire the assistant info to determine whether UL early sync should be optimized upon the RACH-less LTM cell switch is executed, without waiting for the UE RLF report which may be received long time later.

Observation 6: Compared with UE based solution, the network based solution seems having following two advantages:

- Having no impacts on UE

- Providing faster response for the TA issue, since the source DU can acquire the assistant info to determine whether UL early sync should be optimized upon the RACH-less LTM cell switch is executed, without waiting for the UE RLF report which may be received long time later.

Proposal 5: RAN3 to adopt network based solution for TA invalid issue.

3 Conclusion

Observation 1: if the provided TA value is invalid (e,g, it is out date), the RACH-less LTM cell switch may fail since the target cell can not decode the UE's uplink messages which arrives outside the target cell's reception window.

Observation 2: if UE fails to perform RACH-less LTM cell switch,  and followed by a successfully RACH-based  LTM recovery to the same target cell indicated in the CELL SWITCH COMMAND MAC CE, the failure cause may be the invalid TA value or wrong beam information.

Observation 3: in order to identify whether the problem of failure is invalid TA value or not, the source DU needs to acquire the TA value used for successful RACH-based accessing to the target cell after RACH-Less LTM cell switch fails.

Observation 4: source DU may need to do TA optimization only when the TA value is provided by the network for RACH-Less LTM cell switch.

Observation 5: according to current specification, the CU may not know whether TA is provided to the UE by the source DU or not. And  it is not clear whether the DU still keep the TA information of the UE when it receive the RLF report from the UE.

Observation 6: Compared with UE based solution, the network based solution seems having following two advantages:

- Having no impacts on UE;

- Providing faster response for the TA issue, since the source DU can acquire the assistant info to determine whether UL early sync should be optimized upon the RACH-less LTM cell switch is executed, without waiting for the UE RLF report which may be received long time later.

Proposal 1: the source DU should find out whether the provided the TA value in the CELL SWITCH COMMAND MAC CE to the UE is valid or not.

Proposal 2: In addition to too late, too early, and wrong cell,  RAN3 to add a new MRO user case of invalid TA or wrong beam for detect connection failures.

Proposal 3: if  UE-based solution is adopted, the UE should provide following information in the RLF report: the TA value used for successful RACH-based LTM recovery, an indication of whether the TA used in RACH-less LTM cell switch is provided by the source DU,  and the TA value used for failure RACH-less LTM cell switch.

Proposal 4: if network-based solution is adopted, if the target cell find the UE perform RACH-based access mode,  it can send the RACH-based related info ( e.g, the TA value used for successful RACH-based accessing) to the source CU in the ACCESS SUCCESS message, then the source CU can forward this info to the source DU to do analyse.

Proposal 5: RAN3 to adopt network based solution for TA invalid issue.

Annex A: TP to TS38.300

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

-	Intra-system invalid TA for RACH-less LTM: an a RACH-less LTM cell switch failure occurs during the LTM cell switch procedure, then the UE perform  RACH-based LTM cell switch to the same target cell successful.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover, handover to wrong cell and Intra-system invalid TA for RACH-less LTM are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM or CHO with candidate SCG(s) is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

-	Intra-system invalid TA for RACH-less LTM:  there is a recent RACH-less LTM cell switch for the UE prior to the connection failure e.g. the cell UE attempts LTM recovery is the target cell of the last LTM cell switch and the TA value provided by the target cell, if received,  is much different from the one indicated in the LTM cell switch command by the last serving cell.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform the failure event to the candidate NG-RAN node(s) controlling the candidate PSCell where the mobility configuration caused the failure.

Editor’s note: FFS on the definition of failure event.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

<< End of Change >>

Annex B: TP to TS38.473

9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover, conditional PSCell addition, conditional PSCell change, LTM, or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |
| TA event | O |  | FFS |  |  |  |

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Report Information List |  |  |  |  | YES | ignore |
| >LTM Report Information Item |  |  |  |  |  |  |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |  |  |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |  |  |
| >>TA value | M |  | FFS |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |