3GPP TSG RAN WG3 Meeting #127bis	R3-252216

Wuhan, China, 7 – 11 April, 2025

Agenda Item:	9.2

Source:		Huawei

Title:	Further discussions on QMC coordination for RRC segmentation in NR-DC

Document for:	Discussions & Approval

- Introduction

In the RAN2#129 meeting, RAN2 discussed the LS from RAN3 on the QMC coordination for RRC segmentation in NR-DC, and sent the reply LS [1] to RAN3. Based on the reply LS, we provide the following analysis.

- Discussion

For the Question 1&2 on whether UE can receive rrc-SegAllowedSRB5-r18 in MCG configuration or rrc-SegAllowedSRB4-r17 in SCG configuration, RAN2 has different understandings.

Based on it, there are two different understandings in RAN2: - Understanding 1: rrc-SegAllowedSRB4-r17 is only used for an MCG, and rrc-SegAllowedSRB5-r18 is only used for SCG. But there is no restriction that MN should not include rrc-SegAllowedSRB5-r18 in AppLayerMeasConfig-r17 or SN should not include rrc-SegAllowedSRB4-r17 in AppLayerMeasConfig-r17. In order words, UE can receive both rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 in a common AppLayerMeasConfig-r17 from any node. - Understanding 2: rrc-SegAllowedSRB4-r17 is only configured in an MCG configuration and rrc-SegAllowedSRB5-r18 is only configured in an SCG configuration.

For the Question 3&4 on how UE understand the need code Need R for the rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18, RAN2 also has different understandings.

- Understanding 1: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, UE considers the RRC segmentation function for SRB5 is disabled. If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, UE considers the RRC segmentation function for SRB4 is disabled. - Understanding 2: rrc-SegAllowedSRB4-r17 cannot be included in a configuration for an SCG and absence of the field in an SCG configuration means no action by the UE as the field cannot be present. Similarly, rrc-SegAllowedSRB5-r18 cannot be included in a configuration for an MCG and absence of the field in an MCG configuration means no action for the UE as the field cannot be present.

Therefore the current design on rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18 is not clear from RAN2 side. RAN2 also does not want to modify the specification due to the non-backward compatible changes. In the end, RAN2 reached the following agreements:

We do not correct this in RAN2 and let RAN3 make corrections

Observation 1: RAN2 has different understandings for the current description in the RAN2’s specification and does not want to solve the issue.

According to the reply LS, different UEs may have different understandings on the configuration and absence of rrc-SegAllowedSRB4-r17 and rrc-SegAllowedSRB5-r18. It seems it is better to solve this issue from the network side. In order to avoid the incorrect release of the SRB segmentation, network need to include the correct IEs in Uu if it wants to configure new QoE measurement or modifiy the QoE measurement, see below the ASN.1 coding from 38.311, from which the node configurating QMC seems to need the configuration info of SRB segmentation from the other node, due to the requirement of “Need R”.

Observation 2: In order to have correct QMC configuration towards UE, the node configurating QMC seems to need the configuration info of SRB segmentation from the other node.

But according to the current design, one node may configure or modify the QoE measurement without the need to coordinate with another node. For example, one node can send the release command to UE before informing another node, and the MN does not need to interacte with SN for the signaling based QoE measurement.

Observation 3: One node may send the QoE configuration/modifiy/release command to UE without coordination with another node.

Therefore we think one node need to know whether another node has configured the SRB segmentation. If node#1 (re)configures the SRB segmentation, it should inform node#2. In this case, node#2 can include the correct IE if it wants to send/modify the QoE measurement.

Proposal 1: If one node (re)configures the SRB segmentation, it should first inform another node.

In our understanding, RAN node does not know whether the segmentation is needed because RAN node does not know the size of the following QoE report, it may only depend on the RAN capability and UE capability. Therefore, the straightforward solution is the MN can decide whether to enable the SRB4 segmentation by itself, and the SN can also decide whether to enable the SRB5 segmentation by itself.

Proposal 2: MN decides whether to configure SRB4 segmentation and SN decides whether to configure SRB5 segmentation.

Based on the above proposals, we the following analysis and proposals for the detail coordination.

The node in charge of sending QMC seems to need the configuration information of SRB segmentation from the other node, due to the requirement of “Need R”. In addition, for the node which is not in charge of sending QMC configuration to UE, it may still need the segmentation information, since it may reconfigure other (non-QMC related or another ongoing QoE measurement at local side) information directly to UE while the SRB segmentation information of the other node has to be included, still due to “Need R”. In the followings, we give the detail coordination.

For the QoE measurement of MN, if MN wants to configure SRB4 segmentation for this QoE measurement, MN should inform SN of SRB4 segmentation information in MN to SN QMC Coordination Request List because SN may modify some configurations information for other (not the concerned one here) QoE measurement at local side in the furture.

For the QoE measurement of MN, if MN requests SRB5 for the reporting of this QoE measurement, SN should inform MN of SRB5 segmentation information in SN to MN QMC Coordination Response List because MN needs to include the SRB5 segmentation information in this QoE measurement configuration and may modify its QoE measurement configurations in the furture.

For the QoE measurement of SN, if SN wants to configure SRB5 segmentation for this QoE measurement, SN should inform MN of SRB5 segmentation information in SN to MN QMC Coordination Request List because MN may modify some configurations information for other (not the concerned one here) QoE measurement at local side in the furture.

For the QoE measurement of SN, if SN requests SRB4 for the reporting of this QoE measurement, MN should inform SN of SRB4 segmentation information in MN to SN QMC Coordination Response List because SN needs to include the SRB4 segmentation information in this QoE measurement configuration and may modify its QoE measurement configurations in the furture.

Proposal 3: Introduce the SRB4 segmentation indication in MN to SN QMC Coordination Request List and in MN to SN QMC Coordination Response List and introduce the SRB5 segmentation indication in SN to MN QMC Coordination Request List and in SN to MN QMC Coordination Response List.

The corresponding CR to 38.423 is referred in the Annex part.

- Conclusion

In this contribution, we discuss the QoE coordination for RRC segmentation in NR-DC and has the following observations and proposals:

Observation 1: RAN2 has different understandings for the current description in the RAN2’s specification and does not want to solve the issue.

Observation 2: In order to have correct QMC configuration towards UE, the node configurating QMC seems to need the configuration info of SRB segmentation from the other node.

Observation 3: One node may send the QoE configuration/modifiy/release command to UE without coordination with another node.

Proposal 1: If one node (re)configures the SRB segmentation, it should first inform another node.

Proposal 2: MN decides whether to configure SRB4 segmentation and SN decides whether to configure SRB5 segmentation.

Proposal 3: Introduce the SRB4 segmentation indication in MN to SN QMC Coordination Request List and in MN to SN QMC Coordination Response List and introduce the SRB5 segmentation indication in SN to MN QMC Coordination Request List and in SN to MN QMC Coordination Response List.

- References

[1] R2-2501396, Reply LS on QMC Coordination for RRC Segmentation in NR-DC, RAN2

5	Annex

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to QMC Coordination for RRC Segmentation in NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 received the reply LS on QMC Coordination for RRC Segmentation in NR-DC in R3-251516, from which some updates are needed on the exchange of RRC Segmentation between MN and SN during QMC coordination procedure, in order to indicate the correct configuration of rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 towards UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce two new IEs SRB4 Segmentation Indication and SRB4 Segmentation Indication in the IEs QMC Coordination Request and QMC Coordination Response respectively Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has isolated impact with the previous version of the specification (same release) on the QMC coordination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration info about rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 might be misaligned between network and UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.197, 9.2.3.198 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev -: submitted to RAN3#125bis Rev 1: resubmitted to RAN3#126 with updates |  |  |  |  |  |  |  |  |

=============================Start of change==============================

9.2.3.197	QMC Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node, or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  | – |  |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. | – |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE indicates the identity of the application layer measurement configuration, and corresponds to information provided in the MeasConfigAppLayerId IE as defined in TS 38.331 [10]. This IE is allocated by the M-NG-RAN node. | – |  |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. | – |  |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the QoE reports. | – |  |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. | – |  |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node to indicate whether the S-NG RAN node is interested in receiving further RVQoE reports. | – |  |
| >>Further RAN Visible QoE Reporting Path Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node the preferred SRB for receiving further RAN Visible QoE reports. | – |  |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the S-NG-RAN node. | – |  |
| >>Available RAN Visible QoE Metrics | O |  | 9.2.3.158 |  | – |  |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, …) | This IE indicates that the configuration has been released by the M-NG-RAN node. | – |  |
| SN to MN QMC Coordination Request List |  | 0..1 |  |  | – |  |
| >SN to MN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. | – |  |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. | – |  |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred SRB for receiving the QoE reports. | – |  |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. | – |  |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the S-NG-RAN node when the S-NG-RAN node is the RAN Visible QoE configuring node and the M-NG-RAN node provides the bearers for the application session, to request from the M-NG-RAN node to indicate whether the M-NG-RAN node is interested in receiving further RVQoE reports. | – |  |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node to indicate whether the S-NG RAN node is interested in receiving further RVQoE reports. | – |  |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration. | – |  |
| >>Available RAN Visible QoE Metrics |  |  | 9.2.3.158 |  | – |  |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, ...) | This IE indicates that the configuration has been released by the S-NG-RAN node. | – |  |
| SRB4 Segmentation Indication | O |  | ENUMERATED (true, false, ...) | This IE is ignored if the MN to SN QMC Coordination Request List IE is not included. | YES | ignore |
| SRB5 Segmentation Indication | O |  | ENUMERATED (true, false, ...) | This IE is ignored if the SN to MN QMC Coordination Request List IE is not included. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

9.2.3.198	QMC Coordination Response

This IE contains the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, or the information that the S-NG-RAN node needs to provide to the M-NG-RAN node in response to the QMC Coordination Request.

| IE/Group Name | Presence | Range | IE type and reference |  | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Response List |  | 0..1 |  |  |  | – |  |
| >MN to SN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |  |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. | – |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  | This IE indicates the identity of the application layer measurement configuration, and corresponds to information provided in the MeasConfigAppLayerId IE as defined in TS 38.331 [10]. | – |  |
| >>QoE Configuration Sending Path | O |  | ENUMERATED (mn, sn, …) |  |  | – |  |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |  | – |  |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |  | – |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) |  | This IE is to indicate whether the M-NG-RAN node is interested in receiving further RAN Visible QoE reports. | – |  |
| >>Further RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (srb4, srb5, ...) |  | This IE indicates the preferred path for further RAN Visible QoE reporting. | – |  |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 |  | The RAN Visible QoE Configuration preference of the M-NG-RAN node. | – |  |
| SN to MN QMC Coordination Response List |  | 0..1 |  |  |  | – |  |
| >SN to MN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |  |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. | – |  |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |  | – |  |
| >>RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |  | – |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) |  | This IE indicates whether the S-NG-RAN node is interested in receiving further RAN Visible QoE reports. | – |  |
| >>Further RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (srb4, srb5, ...) |  | This IE indicates the preferred path for further RAN Visible QoE reporting. | – |  |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 |  | The RAN Visible QoE Configuration preference of the S-NG-RAN node. |  |  |
| SRB4 Segmentation Indication | O |  |  | ENUMERATED (true, false, ...) | This IE is ignored if the MN to SN QMC Coordination Response List IE is not included. | YES | ignore |
| SRB5 Segmentation Indication | O |  |  | ENUMERATED (true, false, ...) | This IE is ignored if the SN to MN QMC Coordination ResponseList IE is not included. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

=============================Next change==============================

id-CHO-Maxnoof-CondReconfig,

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-TAISliceUnavailableCellList,

id-MobileIABCell,

id-XR-Bcast-Information,

id-MaximumDataBurstVolume,

id-CPAC-Preparation-Type,

id-MN-only-MDT-collection,

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-SRB4SegmentationIndication,

id-SRB5SegmentationIndication,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

=============================Next change==============================

QMCCoordinationRequest ::= SEQUENCE {

mN-to-SN-QMCCoordRequestList	MN-to-SN-QMCCoordRequestList	OPTIONAL,

sN-to-MN-QMCCoordRequestList	SN-to-MN-QMCCoordRequestList	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QMCCoordinationRequest-ExtIEs} }	OPTIONAL,

...

}

QMCCoordinationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

ID id-SRB4SegmentationIndication		CRITICALITY ignore	EXTENSION SRB4SegmentationIndication			PRESENCE optional},

ID id-SRB5SegmentationIndication		CRITICALITY ignore	EXTENSION SRB5SegmentationIndication			PRESENCE optional},

...

}

SRB4SegmentationIndication ::= ENUMERATED {true, false, ...}

SRB5SegmentationIndication ::= ENUMERATED {true, false, ...}

MN-to-SN-QMCCoordRequestList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF MN-to-SN-QMCCoordRequestList-Item

MN-to-SN-QMCCoordRequestList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qOEMeasConfigAppLayerID			QOEMeasConfAppLayerID						OPTIONAL,

measCollectionEntityIPAddress	MeasCollectionEntityIPAddress				OPTIONAL,

qoEReportingPathRequest			ENUMERATED{srb4, srb5, ...}					OPTIONAL,

rVQoEReportingPathRequest		ENUMERATED{srb4, srb5, ...}					OPTIONAL,

furtherRVQoEInterestInquiry		ENUMERATED{true, ...}						OPTIONAL,

furtherRVQoEReportingPathInquiry		ENUMERATED{true, ...}				OPTIONAL,

currentRVQoEConfig				RVQoEConfig									OPTIONAL,

availableRVQoEMetrics			AvailableRVQoEMetrics						OPTIONAL,

configReleaseIndication			ENUMERATED{rvqoe,qoe-and-rvqoe, ...}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MN-to-SN-QMCCoordRequestList-Item-ExtIEs} }	OPTIONAL,

...

}

MN-to-SN-QMCCoordRequestList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SN-to-MN-QMCCoordRequestList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SN-to-MN-QMCCoordRequestList-Item

SN-to-MN-QMCCoordRequestList-Item ::= SEQUENCE {

qOEReference					QOEReference,

measCollectionEntityIPAddress	MeasCollectionEntityIPAddress				OPTIONAL,

qoEReportingPathRequest			ENUMERATED{srb4, srb5, ...}					OPTIONAL,

rVQoEReportingPathRequest		ENUMERATED{srb4, srb5, ...}					OPTIONAL,

furtherRVQoEInterestInquiry		ENUMERATED{true, ...}						OPTIONAL,

furtherRVQoEReportingPathInquiry		ENUMERATED{true, ...}				OPTIONAL,

currentRVQoEConfig				RVQoEConfig									OPTIONAL,

availableRVQoEMetrics		AvailableRVQoEMetrics							OPTIONAL,

configReleaseIndication			ENUMERATED{rvqoe,qoe-and-rvqoe, ...}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SN-to-MN-QMCCoordRequestList-Item-ExtIEs} }	OPTIONAL,

...

}

SN-to-MN-QMCCoordRequestList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QMCCoordinationResponse ::= SEQUENCE {

mN-to-SN-QMCCoordResponseList	MN-to-SN-QMCCoordResponseList	OPTIONAL,

sN-to-MN-QMCCoordResponseList	SN-to-MN-QMCCoordResponseList	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QMCCoordinationResponse-ExtIEs} }	OPTIONAL,

...

}

QMCCoordinationResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

ID id-SRB4SegmentationIndication		CRITICALITY ignore	EXTENSION SRB4SegmentationIndication			PRESENCE optional},

ID id-SRB5SegmentationIndication		CRITICALITY ignore	EXTENSION SRB5SegmentationIndication			PRESENCE optional},

...

}

MN-to-SN-QMCCoordResponseList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF MN-to-SN-QMCCoordResponseList-Item

MN-to-SN-QMCCoordResponseList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qOEMeasConfigAppLayerID			QOEMeasConfAppLayerID						OPTIONAL,

qoEConfigSendingPath			ENUMERATED{mn, sn, ...}						OPTIONAL,

qoEReportingPathResponse		ENUMERATED{accepted, rejected, ...}			OPTIONAL,

rVQoEReportingPathResponse		ENUMERATED{accepted, rejected, ...} 			OPTIONAL,

furtherRVQoEInterestResponse	ENUMERATED{interested, not-interested, ...}	OPTIONAL,

furtherRVQoEReportingPathResponse		ENUMERATED{srb4, srb5, ...}			OPTIONAL,

preferredRVQoEConfig			RVQoEConfig									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MN-to-SN-QMCCoordResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MN-to-SN-QMCCoordResponseList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SN-to-MN-QMCCoordResponseList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SN-to-MN-QMCCoordResponseList-Item

SN-to-MN-QMCCoordResponseList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qoEReportingPathResponse		ENUMERATED{accepted, rejected, ...}										OPTIONAL,

rVQoEReportingPathResponse		ENUMERATED{accepted, rejected, ...} 										OPTIONAL,

furtherRVQoEInterestResponse	ENUMERATED{interested, not-interested, ...}								OPTIONAL,

furtherRVQoEReportingPathResponse		ENUMERATED{srb4, srb5, ...}										OPTIONAL,

preferredRVQoEConfig			RVQoEConfig																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SN-to-MN-QMCCoordResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

SN-to-MN-QMCCoordResponseList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

=============================Next change==============================

id-PDUSetQoSParameters														ProtocolIE-ID ::= 448

id-N6JitterInformation														ProtocolIE-ID ::= 449

id-ECNMarkingorCongestionInformationReportingRequest						ProtocolIE-ID ::= 450

id-PDUSetbasedHandlingIndicator												ProtocolIE-ID ::= 451

id-TAISliceUnavailableCellList												ProtocolIE-ID ::= 452

id-MobileIAB-AuthorizationStatus											ProtocolIE-ID ::= 453

id-MIAB-MT-BAP-Address														ProtocolIE-ID ::= 454

id-MobileIABCell															ProtocolIE-ID ::= 455

id-sk-Counter																ProtocolIE-ID ::= 456

id-Source-M-NG-RANnodeID													ProtocolIE-ID ::= 457

id-ProtocolIE-ID458-NotToBeUsed												ProtocolIE-ID ::= 458

id-SourceSN-to-TargetSN-QMCInfo												ProtocolIE-ID ::= 459

id-RegistrationRequestForDataCollection										ProtocolIE-ID ::= 460

id-ReportCharacteristicsForDataCollection									ProtocolIE-ID ::= 461

id-ReportingPeriodicityForDataCollection									ProtocolIE-ID ::= 462

id-NodeAssociatedInfoResult													ProtocolIE-ID ::= 463

id-SLPositioning-Ranging-Services-Info										ProtocolIE-ID ::= 464

id-XR-Bcast-Information					 									ProtocolIE-ID ::= 465

id-PDUSessionsListToBeReleased-UPError										ProtocolIE-ID ::= 466

id-MaximumDataBurstVolume													ProtocolIE-ID ::= 467

id-CPAC-Preparation-Type													ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication												ProtocolIE-ID ::= 469

id-MN-only-MDT-collection													ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo											ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric										ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest									ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation												ProtocolIE-ID ::= 474

id-SRB4SegmentationIndication												ProtocolIE-ID ::= XXX

id-SRB5SegmentationIndication												ProtocolIE-ID ::= YYY

END

-- ASN1STOP

=============================End of change==============================