3GPP TSG-RAN WG3 Meeting #127-bis	R3-252217

Wuhan, China, 7-11 April, 2025

Title:	Discussion on the Vendor Specific Trace Record

Agenda Item:	8.1

Source:	Huawei

Document for:	Discussion and decision

1	Introduction

This paper tries to discuss the potential RAN3 impacts resulted from the incoming LS from SA5 [1], with some proposals being suggested.

2	Discussion

In the incoming LS, SA5 indicated that they made some updates to the existing MDT configuration, the motivation are as follows:

There is support for a vendor to add their own content to an existing standardized record.  There is however limited support for the configuration of such vendor content.  For instance, a vendor can add content to existing trace messages such as M1, however there is limited support for how such vendor-specified content is configured for collection.

There is no standardized support to allow a vendor to define their own records. A vendor wishing to define their own “Mx”, where “x” is vendor defined, cannot use the existing solution to define, request, configure nor report such content.

It would be beneficial to enhance the current trace solution to better support vendor-defined content in a consistent manner.

Considering the existing support for the collection of Trace Records in the SA5 specifications a possible solution would be to enhance the MDT mechanism to support vendor specific content.

SA5 has discussed and agreed to update SA5 specifications as attached agreed CR.

While the attached SA5 CR to 32.422 shows the main changes to the table as below:

| Trace Depth | Meaning |
| --- | --- |
| Minimum | Recording of some IEs in the signalling messages plus any vendor specific extensions to this definition, in decoded format. |
| Medium | Recording of some IEs in the signalling messages together with the radio measurement IEs plus any vendor specific extensions to this definition, in decoded format. |
| Maximum | Recording entire signalling messages plus any vendor specific extensions to this definition, in encoded format. |
| MinimumWithoutVendorSpecificExtension | Recording of some IEs in the signalling messages in decoded format. |
| MediumWithoutVendorSpecificExtension | Recording of some IEs in the signalling messages together with the radio measurement IEs in decoded format. |
| MaximumWithoutVendorSpecificExtension | Recording entire signalling messages in encoded format. |
| MinimumOnlyVendorSpecificTraceRecord | Recording of vendor specific trace records only, at vendor defined minimum level |
| MediumOnlyVendorSpecificTraceRecord | Recording of vendor specific trace records only, at vendor defined medium level |
| MaximumOnlyVendorSpecificTraceRecord | Recording of vendor specific trace records only, at vendor defined maximum level |

And the following texts:

Trace depth shall be an enumerated parameter with the following possible values:

Minimum (0),

Medium (1),

Maximum (2),

MinimumWithoutVendorSpecificExtension (3),

MediumWithoutVendorSpecificExtension (4),

MaximumWithoutVendorSpecificExtension (5),

MinimumOnlyVendorSpecificTraceRecord (6),

MediumOnlyVendorSpecificTraceRecord (7),

MaximumOnlyVendorSpecificTraceRecord (8).

When checking the RAN3 specs, this Trace Depth IE is explitly defined in Trace related message, see below in 38.413 as example:

| Trace Depth | M |  | ENUMERATED (minimum, medium, maximum, minimumWithoutVendorSpecificExtension, mediumWithoutVendorSpecificExtension, maximumWithoutVendorSpecificExtension, …) | Defined in TS 32.422 [11]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

Comparing the specs between SA5 and RNA3, obvious some corresponding updates are needed for RAN3 specs, since this Trace Depth IE is defined in 38.413, 38.423, 37.483 and 38.473, thus we may need corrections to these three specs.

Proposal 1: Some updates are needed to RAN3 specs including 38.413, 38.423, 37.483 and 38.473, corresponding to the update to Trace Depth IE in SA5 spec.

From the LS, SA5 is trying to introduce vendor specific content Mx, however there is no definition nor any metioning of “Mx” in the attached CR, technically it seems there some still updates expected, or, SA5 would like to leave the remaining updates to pure implementation, if there is a replying LS is needed, i.e. to inform SA5 about corresponding updates in RAN3, and may further check with SA5 on potential further work on Mx, so that RAN3 could also expect some further updates to RAN3 specs resulted from further work in SA5.

Proposal 2: RAN3 to discuss the need of a reply LS informing SA5 about corresponding updates in RAN3, and in which whether there is a need to further check with SA5 on potential further work on Mx.

Another thing is, since 32.422 covers different RAT, including LTE and NR, while in the incoming LS, the WI is a R19 one which should target NR. In our understanding, considering the WI should be for NR, there is no need to update LTE specs on this point, RAN3 needs to discuss this and may also need to confirm the understanding with SA5.

Proposal 2bis: If a LS resply is to be sent, RAN3 should also confirm with SA5 if the update only applies to NR.

In our understanding, an LS might be needed and we should check with SA5 about their further work on Mx, corresponding draft CRs and draft LS could be seen in [2~6]

3	Conclusion

This paper tries to discuss the potential RAN3 impacts resulted from the incoming LS from SA5 [1], and suggest the following proposals.

Proposal 1: Some updates are needed to RAN3 specs including 38.413, 38.423, 37.483 and 38.473, corresponding to the update to Trace Depth IE in SA5 spec.

Proposal 2: RAN3 to discuss the need of a reply LS informing SA5 about corresponding updates in RAN3, and in which whether there is a need to further check with SA5 on potential further work on Mx.

Proposal 2bis: If a LS resply is to be sent, RAN3 should also confirm with SA5 if the update only applies to NR.

4	References

[1]	R3-251530, LS on Vendor Specific Trace Record, SA5

[2]	R3-2519, Correction to Trace Depth on Vendor Specific Trace Record, Huawei

[3]	R3-2520, Correction to Trace Depth on Vendor Specific Trace Record, Huawei

[4]	R3-2521, Correction to Trace Depth on Vendor Specific Trace Record, Huawei

[5]	R3-2522, Correction to Trace Depth on Vendor Specific Trace Record, Huawei

[6]	R3-2518, Draft Reply LS on Vendor Specific Trace Record, Huawei