3GPP TSG-RAN WG3 #127-bis					 				                                  R3-252242

Wuhan, China, 7-11 April 2025

Title: 	(TP for BL CR 38.413) New AIoT procedures

Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the RAN3 #127 meeting, we have achieved the following progress.

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1.

WA: Take Option1 (Including AIoTF information in NGAP).

WA: The AIoT RAN node may only provide the AIoT radio.

AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list.

The location of reader is FFS.

FFS on whether signaling is needed, further discussion on the detail information and necessity.

AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.

The behavior on the AIoT RAN on how to select the proper readers needs to be further discussed?

Architecture aspects:

- AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- AIOTF may be aware of the location of reader via OAM configuration.

- FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

- FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT?

- Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area.

- Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

- WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

- FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

About Inventory Procedures:

- WA: Introduce a NGAP Class 1 Inventory Request procedure.

- Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE.

- Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE.

- Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE.

- Introduce a NGAP Class 2 Inventory Report procedure.

- Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE.

- The above Transfer IEs are transparent to AMF in case of indirect communication.

- FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

About Command Procedure:

- WA: Introduce a NGAP Class 1 Command Request procedure

- Introduce COMMAND REQUEST message, which includes Command Request Transfer IE.

- Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE.

- Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE.

- The above Transfer IEs are transparent to AMF in case of indirect communication.

- FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

- FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

- FFS: Command request procedure is a per device procedure or to be used for multiple devices?

- FFS: The need for a Command Report procedure?

- FFS: Device association between gNB and AIOTF is used in Command procedure.

About Interface management procedures:

- FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF.

- FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF.

- FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Command Report?

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

In the RAN2 #129 meeting, the following progresses are achieved.

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Agreements

From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

This paper further discusses the new AIoT procedure based on the above progress.

- Discussion

  - AIoT service cancel procedure

RAN2 has agreed that a certain device can perform Inventory procedures with multiple readers.

5. RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

For example, if AIoT CN is not sure that a group of devices are associated with which reader, it can send multiple Inventory request messages to the several readers in parallel or in sequence. If AIoT CN receives Inventory report message including inventoried devices from one reader, it acknowledges that this group of devices are served by this reader, then it can cancel other ongoing Inventory procedures with other readers.

For the Inventory and command procedure, for the similar reason, if AIoT CN receives Inventory report message from one reader, it can cancel other Inventory and command procedure with other readers.

Proposal 1: AIoT CN can cancel ongoing AIoT service procedures.

For an ongoing Inventory procedure, if the reader receives Inventory cancel request message from AIoT CN and decides to cancel this Inventory procedure, meanwhile, it also receives inventory result from devices. In this case, the reader shall send Inventory report message including the inventory result to the AIoT CN.

Similarly, for an ongoing Inventory and command procedure, when the reader decides to cancel this procedure upon receiving the cancel request from AIoT CN, it if receiving inventory result or command result from devices, shall send the Inventory report message or Command response message to the AIoT CN.

Proposal 2: If the reader decides to cancel ongoing AIoT service procedure upon receiving request from AIoT CN, it shall send Inventory report message or Command response message to AIoT CN, if any.

Proposal 3: Introduce AIoT service cancel procedure.

  - AIoT reader Status Reporting procedure

SA2 spec has capture the following content.

The AIOTF selects AIoT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the AIoT RAN node(s) based on the internal area information.

The AIOTF may have AIoT RAN information and the optional RAN reader information. For direct path, the AIOTF obtains the information from AIoT RAN by the NGAP or OAM configuration.

It means that the AIOTF can provide reader list to the RAN node. So far, AIOTF provides reader list according to the stored association between reader and devices by receiving Inventory report message. Besides, AIOTF may acquire other assistant information from RAN node, for reader selection.

RAN node can provide reader status reporting to the AIOTF, e.g., workable or not workable, load status.

Proposal 4: AIoT CN can request RAN node to provide reader status information.

Proposal 5: Introduce AIoT reader Status Reporting procedure.

- Conclusion

Based on the above analysis, we provide the following proposals.

Proposal 1: AIoT CN can cancel ongoing AIoT service procedures.

Proposal 2: If the reader decides to cancel ongoing AIoT service procedure upon receiving request from AIoT CN, it shall send Inventory report message or Command response message to AIoT CN, if any.

Proposal 3: Introduce AIoT service cancel procedure.

Proposal 4: AIoT CN can request RAN node to provide reader status information.

Proposal 5: Introduce AIoT reader Status Reporting procedure.

- TP to TS38.413

-----------------Start of the Change-----------------

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| Inventory Request | INVENTORY REQUEST | INVENTORY RESPONSE | INVENTORY FAILURE |
| Command Request | COMMAND REQUEST | COMMAND RESPONSE | COMMAND FAILURE |
| AIoT reader Status | AIOT READER STATUS REQUEST | AIOT READER STATUS RESPONSE | AIOT READER STATUS FAILURE |
| AIoT Service Cancellation | AIOT SERVICE CANCEL | AIOT SERVICE CANCEL ACKNOWLEDGE |  |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| RAN Paging Request | RAN PAGING REQUEST |
| Inventory Report | INVENTORY REPORT |
| AIoT reader Status Report | AIOT READER STATUS REPORT |

-----------------Start of the Next  Change-----------------

8.xx.a	AIoT reader Status

8.xx.a.1	General

The purpose of the AIoT reader Status procedure is to enable the AIoT CN to request the NG-RAN node to start or stop reporting of AIoT reader status information as specified in TS 23.501 [9] and TS 23.502 [10]. The procedure uses non-UE associated signalling.

8.xx.a.2	Successful Operation

Figure 8.19.1.2-1: AIOT READER status procedure: successful operation

The AIoT CN initiates the procedure by sending an AIOT READER STATUS REQUES message to the NG-RAN node.

8.xx.a.3	Unsuccessful Operation

Figure 8.19.1.3-1: Timing synchronisation status procedure: unsuccessful operation

If the NG-RAN node is not able to report AIoT reader status, it shall consider the procedure as failed and reply with the AIOT READER STATUS FAILURE message.

8.xx.a.4	Abnormal Conditions

Void.

8.xx.b	AIoT reader Status Report

8.xx.b.1	General

The purpose of the AIoT reader Status Report procedure is to enable the NG-RAN node to provide AIoT reader status information to the AIoT CN as specified in TS 23.501 [9] and TS 23.502 [10]. The procedure uses non-UE associated signalling.

8.xx.b.2	Successful Operation

Figure 8.19.2.2-1: Timing synchronisation status report

The NG-RAN node initiates the procedure by sending an AIOT READER STATUS REPORT message to the AIoT CN.

8.xx.b.3	Abnormal Conditions

Void.

8.xx.c	AIoT Service Cancellation

8.xx.c.1	General

The purpose of the AIoT Service Cancellation procedure is to enable an AIoT CN to cancel an ongoing AIoT Service procedure. The procedure uses non-UE associated signalling.

8.xx.c.2	Successful Operation

Figure 8.4.5.2-1: Handover cancel: successful operation

The AIoT CN initiates the procedure by sending an AIOT SERVICE CANCEL message to the NG-RAN node.

8.xx.c.3	Unsuccessful Operation

Not applicable.

8.xx.c..4	Abnormal Conditions

Void.

-----------------Start of the Next  Change-----------------

9.2.xx.a	AIOT READER STATUS REQUEST

This message is sent by the AIoT CN to request the NG-RAN node to start or stop reporting of AIoT reader status information.

Direction: AIoT CN  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |

9.2.xx.b	AIOT READER STATUS RESPONSE

This message is sent by the NG-RAN node to confirm the request to start or stop reporting of AIoT reader status information.

Direction: NG-RAN node  AIoT CN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.xx.c	AIOT READER STATUS FAILURE

This message is sent by the NG-RAN node to indicate that reporting of AIoT reader status information cannot be initiated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.xx.d	AIOT READER STATUS REPORT

This message is sent by the NG-RAN node to report previously requested AIoT reader status information.

Direction: NG-RAN node  AIoT CN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |

9.2.xx.e	AIOT SERVICE CANCEL

This message is sent by the AIoT CN to the NG-RAN node to request the cancellation of an ongoing AIoT service procedure.

Direction: AIoT CN  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.xx.f	AIOT SERVICE CANCEL ACKNOWLEDGE

This message is sent by the NG-RAN node to the AIoT CN to confirm that the ongoing AIoT service procedure was cancelled.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------------END of the Change-----------------