3GPP TSG-RAN WG3 Meeting #127bis	R3-252248

Wuhan, China, 7 –11 April 2025

Agenda Item:	16.3

Source: 				LG Electronics Inc.

Title:			Discussion on support of location report

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 WID on Solutions for Ambient IoT (Internet of Things) in NR was agreed at RAN #106 [1]. One of the objectives in this WID is to specify necessary architectural aspects, and signaling and procedures including device location reporting at reader ID granularity as follows:

… - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2. …

In this contribution, we will discuss the issue on support of location report for the A-IoT device and provide our view on it.

2.	Discussion

In TR 38.769, the RAN3 discussion for locating A-IoT devices is captured. There is the NOTE on whether multiple reader ID(s) can be included in the Inventory Report/Command Response message needs further discussion.

TR 38.769: … ## 6.9 Locating ambient IoT devices ### 6.9.1 General A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the A-IoT service. Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported. NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future. ### 6.9.2 Topology 1 For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages. The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded. NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion. NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion. NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion. …

Regarding this issue, we need to check the other WG’s progress on A-IoT WI. It was agreed in RAN2 that the A-IoT device is expected to only perform one procedure at a time. Therefore, even though the service areas of one or more readers within the A-IoT RAN are overlapped and the A-IoT device is located in this overlapped area, this A-IoT device can respond to at most one reader.

However, it is also acknowledged by RAN2 that multi-reader scenario may exist. Note also that the AIOTF may send the same service request to multiple readers (e.g., for locating the A-IoT device). In this case, the A-IoT device needs to repeatedly respond to the paging message of each reader. That is, since the A-IoT RAN can coordinate the readers to avoid transmission of parallel service requests by multiple readers simultaneously based on the implementation, it is possible that the A-IoT device receives another same service request from the different reader within same A-IoT RAN. Specifically, if additional information (e.g., reader ID) is included in the Paging message, the A-IoT device is able to differentiate between two readers. Therefore, if the previous procedure with one reader is successfully terminated, the A-IoT device needs to respond to the Paging message from another reader. As a result, the A-IoT RAN could receive the response from the A-IoT device via multiple readers.

In addition, SA2 considers that during a certain period, the A-IoT RAN needs to collect the A-IoT device response before sending the Inventory Report/Command Response message to the AIOTF.

TR 38.769: … e. The AIOTF may provide the following assistance information to AIoT RAN/UE Reader: - AIoT service type (e.g. Inventory, Command); - approximate number of AIoT devices based on AF request; - approximate D2R message size based on AF request. NOTE 5: If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation. NOTE 6: The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3. NOTE 7: Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary. Editor's note: For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed. f. The AIOTF may provide the AIoT aggregation assistance information to the AIoT RAN. Aggregation assistance information is used to assist in the aggregation of AIoT responses received from multiple AIoT devices. The AIOTF may determine the aggregation assistance information based on operator policy, considering information provided from the AF, which consists of: - Time interval: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF. This may potentially happen multiple times until the AIoT RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to AIOT RAN that is equal or shorter than the time interval received from the AF. NOTE 8: Further details how to use the AIoT aggregation assistance information are subject to RAN3 decision. NOTE 9: The AIoT responses aggregated from the AIoT devices share the same correlation identifier. NOTE 10: AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID. NOTE 11: If the AIOTF does not provide aggregation assistance information, the aggregation process in the AIoT RAN is determined by implementation. As a result, the interpretation of aggregated data by the AIOTF or AF also depends on individual implementation choices. g. When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF. …

If two or more procedures for same service request are triggered during the time interval and successfully terminated with different readers, the A-IoT device responses received via multiple readers may be aggregated at the A-IoT RAN. Therefore, the A-IoT RAN can include the aggregated A-IoT responses together with the multiple reader IDs to AIOTF.

Based on the above observations, the following proposal is suggested to RAN3:

Proposal 1: Multiple reader ID(s) are included by the A-IoT RAN in the Inventory Report/Command Response message.

3.	Conclusion

In this contribution, we focused on the issue on support of location report for the A-IoT Device and provided our view on it. The following proposal is kindly suggested to RAN3:

Proposal 1: Multiple reader ID(s) are included by the A-IoT RAN in the Inventory Report/Command Response message.

4.	Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR,” RAN1 Vice-chair (Huawei), December, 2024.