3GPP TSG-RAN WG3 Meeting #127bis														R3-252250

Wuhan, China, 7th – 11th April 2025

Agenda item:	21.3

Source:	China Telecom

Title:	Discussion on Rate Control Singalling over F1 Interface

Document for:	Discussion and Decision

1	Introduction

The revised WI on XR for NR Phase 3 WID was agreed in RAN#107 meeting[1]. One of objectives is to specify MAC layer XR rate control for uplink congestion adaption as following.

-	Specify uplink congestion signalling [RAN2, RAN3]:

- Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion

From RAN3 perspetive, it is feasbile to specify associated F1 signalling to support MAC layer rate control. The intention of this contribution is to provide views on F1 signalling based on latest RAN2 progress.

2	Discussion

To support codec adaptation for MMTEL voice and MMTEL video, a RAN-assisted codec adaptation mechanism was introduced in Release 15 to support uplink and downlink bit rate adjustments. Upon receiving the recommended bit rate MAC CE, the UE may initiate an end-to-end bit rate adaptation with its peer. The existing recommended bit rate MAC CE is associated with the DRB/LCH level. Since congestion evaluation and rate adjustment can be completed within the gNB-DU, no RAN3 work is involved. During the RAN2#128 meeting, the following agreements on rate control and congestion information were made:

- RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level.

- gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

- Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

- RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP.

- RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE

- FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

XR services may comprise multiple distinct traffic flows. These traffic flows may be mapped to a single DRB or multiple DRBs based on the local RRM strategy. Currently, the gNB-CU can indicate the mapping between QoS flows and DRBs to the DU via F1 signalling. Whether QoS flows support rate control may be configured or indicated by the gNB-CU or the core network (CN). Therefore, the gNB-CU can indicate which QoS flows or DRBs support rate control to the DU via F1 signalling.

Proposal 1: gNB-CU can indicate which QoS flows or DRBs supports rate control to DU via F1 siganlling.

Technically, both the gNB-DU and gNB-CU can evaluate the congestion status of QoS flows. Supporting per-QoS flow rate control in the MAC layer of the gNB is an implementation issue. When congestion is detected or when latency or data rate is lower than the expected value, the gNB-CU can inform the gNB-DU about which QoS flow(s) are experiencing congestion. With this information, the gNB-DU can decide whether to send a MAC CE to the UE to adjust the data rate.

Proposal 2: to introduce a new congestion status indication for QoS flow from gNB-CU to gNB-DU

3	Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: gNB-CU can indicate which QoS flows or DRBs supports rate control to DU via F1 siganlling.

Proposal 2: to introduce a new congestion status indication for QoS flow from gNB-CU to gNB-DU

4	Reference

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3