3GPP TSG RAN WG3 Meeting #127	R3-250017

Athens, Greece, 17 - 21 February, 2025

3GPP SA WG2 Meeting #166	S2-2412665

Orlando, FL,USA, 18-22 November 2024

# Title:	Reply LS on UE Location Information for NB-IoT NTN

# Response to:	Reply LS on UE Location Information for NB-IoT NTN (R2-2403770 & R3-245819)

# Release:	Release 18

# Work Item:	IoT_NTN_enh-Core

Source:	SA2

To:	RAN2, RAN3

Cc:	CT1

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.401 CR 3834

1. Overall Description:

SA2 thanks RAN2 and RAN3 for the reply LS on UE Location Information for NB-IoT NTN. SA2 approved the attached CR which allows the MME, if requested from eNB, to send the coarse location information received from the UE to NB-IoT eNB.

SA2 nevertheless observed that the CR approved by RAN3 (TS 36.413 CR1957) did not include the “Coarse UE Location Requested” IE in UL NAS Transport, Resume Request and the “Coarse UE Location IE” in Resume Response. Even though Coarse Location Information can be requested once in the Initial UE message also in these cases (and provided to the eNB in a downlink S1-AP message), this prevents the Coarse Location Information of the UE to be requested by eNB and provided from MME to eNB at a later stage during a RRC connection when user plane bearers are established. The relevant procedures in the SA2 specification are “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation” (see clause 5.3.4B.4 of TS 23.401), “Connection Resume procedure” (see clause 5.3.5A) and “MT-EDT procedure for User Plane CIoT EPS Optimisation” (see clause 5.3.5B). SA2 would like to understand if RAN2 and RAN3 has performed any analysis on whether Coarse Location Information is needed by the eNB in any of the above procedures. In case RAN2 and RAN3 conclude that Coarse Location Information is needed in these cases, SA2 would like to highlight that AS security is activated during these procedures which would also allow the UE to provide the Coarse Location Information to the eNB via the RRC protocol.

2. Actions:

To RAN2, RAN3:

ACTION: 	SA2 asks RAN2 and RAN3 to take the above information into account and indicate whether Coarse Location Information is needed by the eNB in the procedures defined in clause 5.3.4B.4, 5.3.5A and 5.3.5B.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #AH		24-28 January 2025	Electronic meeting

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece