3GPP TSG-RAN WG3 Meeting #127                             	  R3-250113

Athens, GR, 17th – 21st Feb, 2025

Source:	CATT

Title:	(TP to BL CR for TS38.455) Support of LMF-based AI positioning

Agenda Item:	20.2

Document for:	Discussion and Decision

Introduction

In the last RAN3 meeting, we initially discussed the potential RAN3 impact to support Direct AI/ML Positioning, i.e. the Case 3b, NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning. According to the discussion [1], some agreements and open issues as below:

Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. For case 3b: NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

SA2 asked RAN3 some questions to resolve the ENs of their spec, details could be found in the LS [2], as below:

SA2 asks RAN3 to provide feedback for the purpose to resolve the following Editor´s Notes in Clause 5.18 (“Support for UE positioning based on a ML Model at the LMF “) and Clause 6.22.3 (“Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements”) in TS 23.273 v.19.1.0: Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS. Please provide a reference to the RAN TS that includes the procedure for data collection by LMF from NG-RAN, so that SA2 can update TS 23.273 and remove these Editor´s Note (or an estimation of the time plan in RAN3). SA2 asks RAN1 to provide current progress and future work plans on data types defined for case 3b. If there is any existing agreement in RAN, please provide a reference.

In this contribution, we will further discuss the potential RAN3 impact to support Direct AI/ML positioning, taking the RAN1 progress/agreements into account. Base on the discussion, we will provide corresponding observations and proposals, and the draft LS response to SA2.

Discussion

Data collection

For the Data collection, corresponding RAN1 agreements or working assumptions could be found below:

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

RAN1#118 Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

Agreement (RAN1#119) For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion Agreement (RAN1#119) For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

In the previous RAN3 meetings, it’s agreed that:

- NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

- The details of the measurements are still pending and subject to further discussion in RAN1.

Which means, the channel measurement, quality indicator of channel measurement, time stamp of channel measurement should be provide to LMF for training data collection for Case 3b. And RAN1 agreed that For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. The other information, if needed, is pending to RAN1.

Proposal 1: In Case 3b, the information for training data collection, e.g. channel measurement, quality indicator of channel measurement, time stamp of channel measurement could be provided from gNB to LMF. The other information, if any, is pending to RAN1.

According to RAN1 agreements, the label and related information could be generated by PRU, Non-PRU UE with estimated location, or LMF. How to generate the label and related data is pending to RAN1 and RAN2, no impact to RAN3.

Proposal 2: In Case 3b, how to generate the label and related data is pending to RAN1 and RAN2, no impact to RAN3.

Inference

For Model Input/Report for inference, the corresponding agreements in RAN1 could be found below:

RAN1#116: Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

RAN1#116 bis: Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Conclusion - It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data.

RAN1#117: Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

RAN1#118: Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

RAN1#118bis: Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: (Mandatory) Channel measurement; (Optional) Quality of the channel measurement; FFS: details of the quality (Mandatory) Time stamp of the channel measurement.

In the existing NRPPa, above information could be provided to LMF in the TRP measurement Result, as shown below:

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| … |  |  |  |  |  |  |

According to the RAN1 agreements, both Sample-based and Path-based measurements are supported.

- Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

- Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

Proposal 3: Both Sample-based and Path-based measurements are supported for LMF-based AI positioning.

For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. In the NRPPa, this could be included in the Measurement Request message.

Proposal 4: For Sample-based measurement, Parameters Nt, Nt', k should be signaled to gNB in Measurement Request.

We assume NRPPa Measurement procedure could be reused to support the purpose of training data collection, inference and monitoring for Case 3b. Any other information need to be reported from gNB to LMF is pending to RAN1.

Proposal 5: In Case 3b, NRPPa measurement procedure could be reused for training data collection, inference and monitoring for Case 3b. Any other information need to be reported from gNB to LMF is pending to RAN1.

Base on the discussion above, we can reply the LS to SA2:

For case 3b, RAN3 agreed that at least the following data will be provided from TRP/gNB to LMF:

- Channel measurement

Two kinds of channel measurements are supported, i.e. path-based measurement and sample-based measurement.

- Path-based measurement: this measurement refers to the measurement in the existing specification (up to Rel-18).

- Sample-based measurement: in addition to the path-based measurement, some enhancements are also supported, e.g. LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting.

Channel measurement is mandatory in a measurement report from TRP/gNB to LMF.

- Quality of the channel measurement

Quality indicator associated to the channel measurement will also be reported to LMF. Details are FFS.

Quality of the channel measurement is optional in a measurement report from TRP/gNB to LMF.

- Time stamp of the channel measurement

Time stamp of the channel measurement will also be reported to LMF. From RAN1 perspective, existing IE “Time Stamp” in TS 38.455 can be reused.

Time stamp is mandatory in a measurement report from TRP/gNB to LMF.

Corresponding draft LS reply is provided in the Annex, RAN3 is requested to discuss and agree the draft LS reply to SA2.

Proposal 6: Discuss and agree the draft LS reply to SA2.

Performance Monitoring

For performance monitoring, corresponding RAN1 agreements are shown below:

RAN1#116 Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

For Case 3b, RAN1 discussed but not decided whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. In our understanding, RAN3 is not able to decide what should be provided from gNB to LMF for the performance monitoring in LMF. However, we assume at least the Part A (Part A: channel measurement, quality indicator of channel measurement, time stamp of channel measurement) parameters could be provided to LMF.

Proposal 7: In Case 3b, it’s not RAN3 to decide what should be provided from gNB to LMF for the performance monitoring in LMF. We assume at least the Part A (channel measurement, quality indicator of channel measurement, time stamp of channel measurement) could be provided for this purpose.

Base on the discussion above, corresponding TP for NRPPa is provided in the Annex. RAN3 is requested to discuss and agree the TP.

Proposal 8: Discuss and agree the TP for NRPPa, as provided in the Annex.

Conclusion

In this contribution, we discussed the RAN3 impact to support Direct AI/ML positioning for AI assisted Positioning Accuracy enhancements. Based on the discussion above, the following proposals are provided:

Proposal 1: In Case 3b, the information for training data collection, e.g. channel measurement, quality indicator of channel measurement, time stamp of channel measurement could be provided from gNB to LMF. The other information, if any, is pending to RAN1.

Proposal 2: In Case 3b, how to generate the label and related data is pending to RAN1 and RAN2, no impact to RAN3.

Proposal 3: Both Sample-based and Path-based measurements are supported for LMF-based AI positioning.

Proposal 4: For Sample-based measurement, Parameters Nt, Nt', k should be signaled to gNB in Measurement Request.

Proposal 5: In Case 3b, NRPPa measurement procedure could be reused for training data collection, inference and monitoring for Case 3b. Any other information need to be reported from gNB to LMF is pending to RAN1.

Proposal 6: Discuss and agree the draft LS reply to SA2.

Proposal 7: In Case 3b, it’s not RAN3 to decide what should be provided from gNB to LMF for the performance monitoring in LMF. We assume at least the Part A (channel measurement, quality indicator of channel measurement, time stamp of channel measurement) could be provided for this purpose.

Proposal 8: Discuss and agree the TP for NRPPa, as provided in the Annex.

Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R3-250074/S2-2501341 LS on LMF-based AIML Positioning for Case 3b

- RAN1#116/#116bis/#117/#118/#118bis/#119 Chair’s Notes

- Annex A. TP to BL CR for TS38.455

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider this as the time window(s) within which to perform the positioning measurements, as specified in TS 38.305 [18].

If the Sample-based Measurement Parameters IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node, shall, if supported, take it into account when configuring sample-based measurement.

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Sample-based Measurement Parameters | O |  |  |  | - |  |
| >>>Nt |  |  | ENUMERATED (32, 64, 128, ...) | Window size for measurement collection. |  |  |
| >>>Nt’ |  |  | FFS | Number of reported measurement samples. |  |  |
| >>>k |  |  | FFS | Interval between measurement samples. |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- Annex B. Draft LS reply to SA2

Title:	[Draft] Reply LS on LMF-based AIML Positioning for Case 3b

Response to:	R3-250074(S2-2501341)

Release:	Rel-19

Work Item:	NR_AIML_air, FS_AIML_CN

Source:	TSG RAN WG3

To:	TSG RAN SA2

Cc:	TSG RAN WG1

Contact Person:

Name:	Jiancheng Sun

E-mail Address:	sunjiancheng@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA2 for the LS. RAN3 has discussed the questions from SA2:

SA2 asks RAN3 to provide feedback for the purpose to resolve the following Editor´s Notes in Clause 5.18 (“Support for UE positioning based on a ML Model at the LMF “) and Clause 6.22.3 (“Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements”) in TS 23.273 v.19.1.0: Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS. Please provide a reference to the RAN TS that includes the procedure for data collection by LMF from NG-RAN, so that SA2 can update TS 23.273 and remove these Editor´s Note (or an estimation of the time plan in RAN3). SA2 asks RAN1 to provide current progress and future work plans on data types defined for case 3b. If there is any existing agreement in RAN, please provide a reference.

RAN3 would like to provide the response as following:

For case 3b, at least the following data will be provided from TRP/gNB to LMF:

- Channel measurement

Two kinds of channel measurements are supported, i.e. path-based measurement and sample-based measurement.

- Path-based measurement: this measurement refers to the measurement in the existing specification (up to Rel-18).

- Sample-based measurement: in addition to the path-based measurement, some enhancements are also supported, e.g. LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting.

Channel measurement is mandatory in a measurement report from TRP/gNB to LMF.

- Quality of the channel measurement

Quality indicator associated to the channel measurement will also be reported to LMF. Details are FFS.

Quality of the channel measurement is optional in a measurement report from TRP/gNB to LMF.

- Time stamp of the channel measurement

Time stamp of the channel measurement will also be reported to LMF. From RAN3 perspective, existing IE “Time Stamp” in TS 38.455 can be reused.

Time stamp is mandatory in a measurement report from TRP/gNB to LMF.

And RAN3 assumes the legacy NRPPa Measurement Request/Response could also be used to collect the input data for LMF-based AI/ML Positioning calculation. RAN3 will work on the NRPPa signallings according to the RAN1 agreements.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into consideration.

3. Date of Next TSG-RAN3 Meetings:

3GPP TSG RAN WG3#127     17 - 21 February 2025				Athens, GR

3GPP TSG RAN WG3#127-bis	07 - 11 April 2025				Wuhan, CN